3GPP TSG-RAN4 Meeting #93	R4-1915795

# Reno, Nevada, USA, 18th - 22th Nov, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on V2X initiation/cease of SLSS requirement R16 (section 12.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - V2X initiation/cease of SLSS requirement section is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the section skeleton for V2X initiation/cease of SLSS requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<Start of Change 1>

12.3	Initiation/Cease of SLSS Transmissions

12.3.1	Introduction

The requirements in this subclause are applicable to the UE capable of V2X sidelink communication when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	EUTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source.

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source

The requirements apply when the NR Cell is used as synchronization reference source and when the UE is

-	out of coverage on the V2X NR sidelink carrier and in-coverage with a serving cell on a NR non-V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in [2] are met; networkControlledSyncTx is not configured; and [syncTxThreshIC] is included in SystemInformationBlockTypeXX. The UE shall be capable of measuring the RSRP of the cell used as synchronization reference source to evaluate to initiate/cease SLSS transmissions within Tevaluate,SLSS

where,

-	Tevaluate,SLSS is as specified in Table 13.3.1.1-1 when UE performs SSB based measurements without measurement gaps.

-	Tevaluate,SLSS is as specified in Table 13.3.1.1-2 when UE performs SSB based measurements with measurement gaps.

Table 12.3.1.1-1: Tevaluate,SLSS for measurements without gaps when NR cell as synchronization reference source (FR1)

| DRX cycle in NR cell | Tevaluate,SLSS |
| --- | --- |
| No DRX | max([400ms], ceil( 2 x 5 x Kp) x SMTC period)Note 1 |
| DRX cycle≤ 320ms | max([400ms], ceil(1.5 x 2 x 5 x Kp) x max(SMTC period, DRX cycle)) |
| DRX cycle>320ms | ceil( 7 x Kp ) x DRX cycle |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 12.3.1.1-2: Tevaluate,SLSS for measurements with gaps when NR cell as synchronization reference source (FR1)

| DRX cycle in NR cell | Tevaluate,SLSS |
| --- | --- |
| No DRX | max([400ms], [2] x 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max([400ms], ceil([2] x 1.5x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra |

If higher layer filtering is configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the NR cell as synchronization reference source:

-	SS-RSRP related side conditions given in clauses 10.1.2 for FR1, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 for FR1, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 for FR1, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source

The requirements apply when the EUTRAN Cell is used as synchronization reference source and when the UE is

-	out of coverage on the V2X NR sidelink carrier and in-coverage with a serving cell on a LTE non-V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in [2] are met; networkControlledSyncTx is not configured; and [syncTxThreshIC] is included in SystemInformationBlockTypeXX. The UE shall be capable of measuring the RSRP of the cell used as synchronization reference source to evaluate to initiate/cease SLSS transmissions within Tevaluate,SLSS

where,

-	Tevaluate,SLSS = 0.4 seconds when UE is not configured with DRX.

-	Tevaluate,SLSS = as specified in Table 13.3.1.1-1 when UE is configured with DRX.

Table 12.3.1.2-1: Tevaluate,SLSS when EUTRAN cell as synchronization reference source

| DRX cycle length in EUTRAN cell[s] | Tevaluate,SLSS [s] (number of DRX cycles) |
| --- | --- |
| ≤0.04 | 0.4 (Note 1) |
| 0.04<DRX-cycle≤2.56 | Note 2 (6) |
| Note1: Number of DRX cycles depends upon the DRX cycle in use Note2: Time depends upon the DRX cycles in use |  |

If higher layer filtering is configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the cell as synchronization reference source:

-	RSRP related side conditions given in TS 36.133 Sections 9.1.2.1 and 9.1.2.2 and RSRQ related side conditions given in TS 36.133 Clause 9.1.5.1 for a corresponding Band are fulfilled,

-	SCH_RP and SCH Ês/Iot according to TS 36.133 Annex B.2.1 for a corresponding Band are fulfilled.

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source

The requirements apply when GNSS is used as synchronization reference source and when the UE is

- out of coverage on the V2X sidelink carrier and in-coverage with a serving cell on a non-V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in [2] are met; networkControlledSyncTx is not configured; and [syncTxThreshIC] is included in SystemInformationBlockTypeXX.

The requirements in Section 12.3.1.1 shall apply if the serving cell is a NR cell.

The requirements in Section 12.3.1.2 shall apply if the serving cell is a EUTRAN cell.

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source

The requirements apply when SyncRef UE is used as synchronization reference source and when the UE is

- in any cell selection state, or

- out of coverage on the V2X sidelink carrier and is associated with a serving cell on a non-V2X sidelink carrier,

and when the conditions for SLSS transmissions specified in [2] are met and when SyncRef UE is used as synchronization reference source and if [syncTxThreshOoC] is included in the preconfigured V2X parameters.

The UE shall be capable of measuring the S-RSRP of the selected SyncRef UE used as synchronization reference source and evaluate it to initiate/cease SLSS transmissions within Tevaluate,SLSS = [4] S-SSB periods.

If higher layer filtering for S-RSRP measurements is pre-configured, an additional delay in evaluation to initiate/cease SLSS transmissions can be expected.

For the selected SyncRef UE [2] used to derive transmission timing for V2X sidelink communication:

-	S-RSRP related side conditions given in Section 13.4 for a corresponding Band are fulfilled,

-	V2X S-SSB_RP and S-SSB Ês/Iot according to Annex [B.6.4] for a corresponding Band are fulfilled.

<End of Change 1>