3GPP TSG-RAN WG4 Meeting #93              	R4-1915880

Reno, USA, 18th – 22th Nov, 2019

Agenda item:	8.13.4.1

Source:	Samsung

Title:	Simulation results for HST PUSCH in LTE Rel-16

Document for:	Discussion

# 1	Introduction

During HST PUSCH for LTE Rel-16 discussion in RAN4#92b meeting, the simulation assumption for PUSCH requirement is captured in the WF [1].

In this contribution, the ideal and impairment results are provided for requirements.

# 2	Simulation results

## 2.1	Simulation assumption for PUSCH

As captured in the WF [1] as following

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Open space |  | Tunnel for multi-antennas |  |
|  | Bi-directional (1st priority) | Uni-directional | Bi-directional (1st priority) | Uni-directional |
| Ds | 1000 m |  | 300 m |  |
| Dmin | 50m |  | 2m |  |
| V | 500km/h |  | 500km/h |  |
| fd | 1944Hz If 70% of maximum throughput is not achievable with 1944Hz Case 1: fall back to 1750Hz Case 2: accept % of maximum throughput lower than 70% with 1944Hz |  |  |  |

the maximum Doppler value is agreed with 1944Hz. Considering the Doppler tracking limitation of PUCCH format2 is 1750Hz, if the maximum throughput is not achievable with 1944Hz, one option is to fall back the Doppler value with 1750Hz.

Another option is to define the test metric for throughput lower than 70% with 1944Hz.  In LTE, two test metrics with 30% and 70% of TP for targeting SNR are defined. In our view, the motivation is to verify performance of the cell edge UE with HARQ combining operation under 30% TP, and the medium SNR range UE performance. Additional test metric introduced will increase the test effort. We prefer to reuse the existing test metric

In table 1, we provide the simulation assumptions for BS demodulation based on agreement.

Table 1: simulation assumptions for PUSCH demodulation performance evaluation

| Parameters | Unit | Values |
| --- | --- | --- |
| Bandwidth | MHz | 1.4/3/5/10/15/20 |
| MCS for PUSCH |  | QPSK 1/3 |
| Propagation condition and correlation matrix |  | AWGN |
| Antenna configuration |  | 1x2 |
| Reference receiver |  | MRC |
| Noise estimation |  | Practical |
| Time and frequency track |  | Practical |

## 2.2	Simulation Results

In this subsection, the ideal and impairment results for PUSCH under two scenarios are provided to finalize the requirement of HST.

Table 2: ideal results and impairment results for HST PUSCH requirements in open space environment

| SCS, BW | MCS | Antenna Configuration | Doppler Value | Test Metric | SNR (ideal results) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 1.4MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -3.0 | -1.0 |
|  |  |  |  | 70% | -1.16 | 0.84 |
| 15KHz, 1.4MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -3.30 | -1.30 |
|  |  |  |  | 70% | -1.35 | 0.65 |
| 15KHz, 3MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -5.39 | -3.39 |
|  |  |  |  | 70% | -2.51 | -0.51 |
| 15KHz, 3MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -5.43 | -3.43 |
|  |  |  |  | 70% | -2.55 | -0.55 |
| 15KHz, 5MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -5.93 | -3.93 |
|  |  |  |  | 70% | -3.26 | -1.26 |
| 15KHz, 5MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -6.19 | -4.19 |
|  |  |  |  | 70% | -3.37 | -1.37 |
| 15KHz, 10MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -5.83 | -3.83 |
|  |  |  |  | 70% | -2.71 | -0.71 |
| 15KHz, 10MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -5.83 | -3.83 |
|  |  |  |  | 70% | -9.53 | -7.53 |
| 15KHz, 15MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -6.43 | -4.43 |
|  |  |  |  | 70% | -3.32 | -1.32 |
| 15KHz, 15MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -6.58 | -4.58 |
|  |  |  |  | 70% | -3.36 | -1.36 |
| 15KHz, 20MHz | QPSK 1/3 | 1x2 | 1944Hz | 30% | -6.05 | -4.05 |
|  |  |  |  | 70% | -2.78 | -0.78 |
| 15KHz, 20MHz | QPSK 1/3 | 1x2 | 1750Hz | 30% | -6.07 | -4.07 |
|  |  |  |  | 70% | -2.81 | -0.81 |

Table 3: ideal results and impairment results for HST PUSCH requirements in tunnel environment

| SCS, BW | MCS | Antenna Configuration | Doppler Value | Test Metric | SNR (ideal results) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 1.4MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -1.93 | 0.07 |
|  |  |  |  | 70% | 1.56 | 3.56 |
| 15KHz, 1.4MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -2.40 | -0.40 |
|  |  |  |  | 70% | 1.22 | 3.22 |
| 15KHz, 3MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -3.01 | -1.01 |
|  |  |  |  | 70% | 0.51 | 2.51 |
| 15KHz, 3MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -4.20 | -2.20 |
|  |  |  |  | 70% | 0.33 | 2.33 |
| 15KHz, 5MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -3.97 | -1.97 |
|  |  |  |  | 70% | -1.59 | 0.41 |
| 15KHz, 5MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -5.10 | -3.10 |
|  |  |  |  | 70% | -1.75 | 0.25 |
| 15KHz, 10MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -4.43 | -2.43 |
|  |  |  |  | 70% | 0.26 | 2.26 |
| 15KHz, 10MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -4.47 | -2.47 |
|  |  |  |  | 70% | 0.19 | 2.19 |
| 15KHz, 15MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -4.43 | -2.43 |
|  |  |  |  | 70% | 0.26 | 2.26 |
| 15KHz, 15MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -5.01 | -3.01 |
|  |  |  |  | 70% | -0.49 | 1.51 |
| 15KHz, 20MHz | QPSK 1/3 | 1x1 | 1944Hz | 30% | -4.50 | -2.50 |
|  |  |  |  | 70% | 0.28 | 2.28 |
| 15KHz, 20MHz | QPSK 1/3 | 1x1 | 1750Hz | 30% | -4.70 | -2.70 |
|  |  |  |  | 70% | 0.03 | 2.03 |

# 3	Conclusion

In this contribution, we presented the ideal and impairment results for finalizing the requirements of HST PUSCH.

# References

R4-1907747, Way forward on LTE HST BS demodulation performance requirement, NTT DOCOMO