3GPP TSG-RAN WG4 #94-e																			R4-2000078

Online, 24 Feb. – 6 Mar., 2020

Source: 			Qualcomm Incorporated

Title: 				Beam management CSI-RS design for BC requirement

Agenda Item:	8.14.1.2

Work Item:		NR_RF_FR2_req_enh

Release:			16

Document for:	Approval

# Introduction

The primary goal for enhancement of beam correspondence capability (BC) for Rel.16 is to ensure that the FR2 UE can perform beam refinement based on reference signal configured by the network. In the agreed WF [1], CSI-RS configuration details and candidates for side condition are summarized. From the discussion in the last RAN4 #93 meeting, the following bullets were perceived as the most controversial issues.

- Whether CSI-RS can be configured with ‘qcl-TypeD = none’ in FR2, and whether UE behaviour is specified in spec.

- How to ensure UE performs eBC based on configured CSI-RS(s) by the network

In this contribution, we provide technical arguments from many different angles for the above issues and propose details of CSI-RS configurations and side condition.

# Discussion

## CSI-RS configurations

Most alternatives in the CSI-RS configuration table of [1] boil down to the following issues.

- Whether CSI-RS can be configured with ‘qcl-TypeD = none’ in FR2, and whether UE behaviour is specified in spec.

- Whether P1 CSI-RS is necessary for the test

For the first aspect, we provide more arguments supporting why configuring CSI-RS with ‘qcl-TypeD = none’ in FR2 is not fully supported by the current spec on top of what we already presented in [2].

Firstly, it should be noted that CR [3] for “Default QCL assumption for periodic CSI-RS” was discussed in RAN1 #89 but rejected with the following decision.

Decision: No further discussion in Rel-15. The draft CR is not pursued.

In other words, even in RAN1 where CSI-RS configurations and pertinent UE behavior should be defined, there was no clear assumption on default QCL and subsequent UE behavior when qcl-TypeD of periodic CSI-RS is absent. In order to resolve issues that may arise when such an unexpected configuration is provided by network, [3] proposed to add default QCL assumption so that a subsequent UE behavior can be clearly defined. This, however, was not accepted by RAN1. Therefore, all test cases requiring periodic CSI-RS should avoid configuring qcl-typeD = ‘none’.

Observation 1: There is neither a default QCL assumption nor a subsequent UE behaviour in RAN1 when qcl-TypeD of periodic CSI-RS is absent in FR2.

Secondly, we also add technical implementation issues which can be caused by P1 CSI-RS configuration with qcl-typeD = ‘none’.

- An initial UE Rx beam for P1 CSI-RS would not be optimized because UE cannot use the Rx beam obtained from SSB, which makes UE consume more time and energy to train its Rx beam based on P1 CSI-RS.

- If there is no explicit QCL relation between SSB and P1 CSI-RS, UE would have to conduct two independent beam managements which may end up with conflict with each other.

- When P1 CSI-RS based beam tracking fails, UE cannot fallback to SSB-based Rx beam without going through RLF.

Observation 2: There will be technical implementation issues when P1 CSI-RS is configured with qcl-typeD = ‘none’.

Lastly, we would mention that there is no such a test case where a source of qcl-TypeD of periodic CSI-RS is not configured even if it aims to verify UE performances based on configured periodic CSI-RS. For instance, RLM requirements based on CSI-RS configured for RLM RS (defined from A.7.5.1.5 to A.7.5.1.8 in TS38.133) configure qcl-TypeD for RLM RS. Instead, they allocate different SNRs for SSB and CSI-RS for RLM RS to ensure UE uses the configured CSI-RS for RLM RS during tests.

Observation 3: There is no such a test case where a source of qcl-TypeD of periodic CSI-RS is not configured even though it aims to verify UE performances based on configured periodic CSI-RS.

For the second aspect “Whether P1 CSI-RS is necessary for the test”, as it was addressed in Fig. 1 [2], if CSI-RS with repetition ‘off’ is configured, an effective CSI-RS based BC test time window will be shrunk due to P1 and/or P2 CSI-RS measurements, reports, and so on. It becomes more serious when CSI-RSs are periodically transmitted. Besides, as CSI-RS based eBC requirements will be verified eventually based on P3 CSI-RS, P1 CSI-RS should not be unnecessarily configured.

In summary, the table from [1] can be filled in as below in Table 1. Proposals are highlighted in yellow.

Table 1. CSI-RS configurations for CSI-RS based Beam Correspondence Test

| Parameter | Value | Justification |
| --- | --- | --- |
| P1 CSI-RS periodicity | Alt.1: P1 CSI-RS is configured with [TBD] ms periodicity, the QCL (qcl-TypeD) relation is configured as ‘SSB’ Alt.2: P1 CSI-RS is not configured; instead aperiodic P2 CSI-RS can be considered if necessary. If P2 CSI-RS is supported, its qcl-TypeD is ‘SSB’ Alt.3: P1 CSI-RS is configured with [TBD] ms periodicity, the QCL (qcl-TypeD) relation is configured as ‘none’ | P1 CSI-RS is not necessary for the test |
| P3 CSI-RS repetitions per resource set | Alt. 1: maxNumberRxBeam in UE capability IE of MIMO-ParametersPerBand Alt. 2: 8 | Respect UE capability declaration. Besides, UE is not required to meet L1-RSRP accuracy if it is smaller than maxNumberRxBeam, and it should not exceed maxNumberAperiodicCSI-RS-Resource as per TS38.133. |
| P3 CSI-RS configuration repetition | On |  |
| P3 CSI-RS trigger | Alt.1: once P1 CSI-RS is finished Alt.2: once every SSB cycle (20 ms) if P1 CSI-RS is not configured | P1 CSI-RS is not necessary for the test |
| Tracking CSI-RS periodicity | reuse Rel-15 60 kHz SCS: 40 slots for CSI-RS resources 1 and 2 120 kHz SCS: 80 slots for CSI-RS resources 1 and 2 |  |
| P3 CSI-RS QCL info | Alt.1: Type D to P1 CSI-RS Alt.2: If P2 CSI-RS is transmitted; - Type A to TRS - Type D to P2 CSI-RS Otherwise; - Type C to SSB - Type D to SSB | P1 CSI-RS is not necessary for the test |
| P1 CSI-RS QCL info | Alt.1: P1 CSI-RS is transmitted and the QCL relation is configured as ‘SSB’ Alt.2: P1 CSI-RS is not transmitted Alt.3: P1 CSI-RS is transmitted and the QCL relation is configured as ‘none’ | P1 CSI-RS is not necessary for the test |

Proposal 1: Parameters for CSI-RS based Beam Correspondence test shall be per Table 1

## Side conditions

To ensure UE uses CSI-RS, not SSB-only, for beam refinement, we narrowed down a list of candidates to two methods as below [1].

- Method 1: DUT is configured with an active BWP containing no SSB

- Method-3: SSB and CSI-RS are present, but SSB’s PSD is back-off by XdB from CSI-RS

As mentioned in Observation 3, verifying UE performance by differentiating SNR between SSB and CSI-RS is already implemented in other test requirements where CSI-RS based UE behavior is of interest. Besides, as depicted in Fig.1 [2], UEs that use SSB for beam refinement instead of configured CSI-RS for BM will experience severe performance degradation in terms of DL-based BC because of lower SSB SNR than CSI-RS one, which leads to Rel.16 eBC failure if UE BC tolerance requirement is defined based on CSI-RS SNR. Therefore, Rel.16 eBC with CSI-RS can be verified with differentiating SNR between SSB and CSI-RS. One way to have SNR offset between SSB and CSI-RS is to update the parameter powerControlOffsetSS of CSI-RS resources from 0dB, defined for Rel.15 BC, to 3dB or 6dB.

Proposal 2: RAN4 defines CSI-RS based eBC requirement by Method-3 below.

- Method-3: SSB and CSI-RS are present, but SSB’s PSD is back-off by XdB from CSI-RS

- X is either 3 or 6

- CSI-RS SNR is [6]dB

- EIRP requirement in terms of ∆EIRPBC CDF will be defined in such a way that UE relying on SSB-only for beam refinement cannot meet the requirement but UE using CSI-RS can satisfy the requirement

# Conclusion

Based on RAN1 Rel-15 maintenance discussion and conclusion, it can be said RAN4 cannot expect UE behaviour when a UE has to manage its beams by using CSI-RS for BM without QCL relationship.

Observation 1: There is neither a default QCL assumption nor a subsequent UE behaviour in RAN1 when qcl-TypeD of periodic CSI-RS is absent in FR2.

From implementation perspective, we expect UE to encounter some difficulties when not clearly addressed configurations are provided.

Observation 2: There will be technical implementation issues when P1 CSI-RS is configured with qcl-typeD = ‘none’.

And we also found there is no such a RAN4 test where qcl-typeD for periodic CSI-RS for FR2 is omitted.

Observation 3: There is no such a test case where a source of qcl-TypeD of periodic CSI-RS is not configured even though it aims to verify UE performances based on configured periodic CSI-RS.

We propose detailed parameters of CSI-RS configuration and Method-3 to assess Rel.16 CSI-RS based Beam Correspondence performance under a practical deployment scenario while ensuring UE utilizes CSI-RS for BM.

Proposal 1: Parameters for CSI-RS based Beam Correspondence test shall be as captured in Table 1, and reproduced below in compact form

| Parameter | Proposed value |
| --- | --- |
| P1 CSI-RS periodicity | Alt.2: P1 CSI-RS is not configured; instead aperiodic P2 CSI-RS can be considered if necessary. If P2 CSI-RS is supported, its qcl-TypeD is ‘SSB’ |
| P3 CSI-RS repetitions per resource set | Alt. 1: maxNumberRxBeam in UE capability IE of MIMO-ParametersPerBand |
| P3 CSI-RS configuration repetition | On |
| P3 CSI-RS trigger | Alt.2: once every SSB cycle (20 ms) if P1 CSI-RS is not configured |
| Tracking CSI-RS periodicity | reuse Rel-15 60 kHz SCS: 40 slots for CSI-RS resources 1 and 2 120 kHz SCS: 80 slots for CSI-RS resources 1 and 2 |
| P3 CSI-RS QCL info | Alt.2: If P2 CSI-RS is transmitted; - Type A to TRS - Type D to P2 CSI-RS Otherwise; - Type C to SSB - Type D to SSB |
| P1 CSI-RS QCL info | Alt.2: P1 CSI-RS is not transmitted |

Proposal 2: RAN4 defines CSI-RS based eBC requirement by Method-3 below.

- Method-3: SSB and CSI-RS are present, but SSB’s PSD is back-off by XdB from CSI-RS

- X is either 3 or 6

- CSI-RS SNR is [6]dB

- EIRP requirement in terms of ∆EIRPBC CDF will be defined in such a way that UE relying on SSB-only for beam refinement cannot meet the requirement but UE using CSI-RS can satisfy the requirement

# Reference

- R4-1916172, “WF on side conditions of SSB based and CSI-RS based beam correspondence tests,” Apple Inc., RAN4#93, Reno, USA, Nov. 2019

- R4-1913205, “Beam management CSI-RS design for BC requirement,” Qualcomm Incorporated, RAN4#93, Reno, USA, Nov. 2019

- R1-1909416, “Default QCL assumption for periodic CSI-RS,” Nokia, NSB, RAN1#98, Prague, Czech Republic, Aug. 2019