3GPP TSG-RAN WG4 Meeting #94-e	R4-2000101

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Source:	Verizon, Nokia, Qualcomm, Samsung, Ericsson

Title:	TP for TR 37.716-11-11: DC_5_n66A

Agenda item:	9.3.2

Document for:	Approval

# Introduction

In RAN#86 meeting, the revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 1 band DL with 1 band UL [1]. This contribution is a text proposal for TR 37.716-11-11 to include DC_5A-5A_n66 and DC_5B_n66A.

# Reference

[1] 	RP-192742, Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band), CHTTL

# Text Proposal

<Start of Text Proposal>

## 6.1.x		DC_5_n66 and DC_5-5_n66

### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5_n66 DC_5-5_n66 | 5 CA_5-5 | n66 | No |

### 6.1.x.2	Configurations for DC

Table 6.1.x.1-2: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A_n66A DC_5A-5A_n66A DC_5B_n66A | DC_5A_n66A | 5A CA_5A-n5A CA_5B | n66A |

#### 6.1.x.3	Maximum output power for DC

The maximum output power for the uplink EN-DC configuration, DC_5A_n66A, is already specified in TS 38.101-3.

6.1.x.4	Spurious emission band UE co-existence

The study of spurious emission for this DC configuration has been already covered in 37.863-01-01 and specified in Table 6.5B.3.3.2-1 in 38.101-3. There is no additional requirement.

### 6.1.x.5	REFSENS requirements

The study of REFSENS requirements for this DC configuration has been already covered in 37.863-01-01. There is no additional requirement.

### 6.1.x.6	∆TIB and ∆RIB values

Both TIB,c and RIB,c studies for DC_5A_n66A have been already covered in 37.863-01-01 and specified in 38.101-3. Therefore DC_5-5_n66 can reuse these values as below tables list.

Table 6.1.x.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n66 DC_5-5_n66 | 5 | 0.3 |
|  | n66 | 0.3 |

Table 6.1.x.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5_n66 DC_5-5_n66 | 5 | 0 |
|  | n66 | 0 |

<Start of Text Proposal>