3GPP TSG-RAN WG4 Meeting #94-e                                                               R4-2000234

Electronic Meeting, 24th February – 6th March, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0201 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-1: to introduce BCS1 for CA_n77C and CA_n78C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CKH IOD UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1 |  |  |  |  |  |  | Date: |  | 2020-02-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | BCS0 configurations for CA_n77C and CA_n78C lack 10+100\20+100\30+100\40+100, etc. Because of the stronge market demand and uncertainty of spectrum auction in EU, we need to add BCS1 configurations for CA_n77C and CA_n78C. It’s inconveniet and unsuitable to place CA_n77C, CA_n78C and CA_79C into one grid. |  |  |  |  |  |  |  |  |
| Summary of change: |  | BCS1 configuration is added for CA_n77C and CA_n78C. CA_n77C, CA_n78C and CA_79C were seperated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BCS1 configuration for CA_n77C and CA_n78C can’t be captured into the spec and the stronge market demand can’t be met. CA_n77C, CA_n78C and CA_79C are still placed into one grid. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TS ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA for fallback group 1

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n1B | - | 10 | 10,15 |  |  |  | 40 | 0 |
|  |  | 15 | 15,20 |  |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |  |
| CA_n7B | CA_n7B | 10, 15, 20 | 10, 15, 20, 30, 35, 40 |  |  |  | 50 | 0 |
| CA_40B | - | 20 | 80 |  |  |  | 100 | 0 |
|  |  | 50 | 50 |  |  |  |  |  |
| CA_n41C | - | 40 | 80, 100 |  |  |  | 180 | 0 |
|  |  | 50, 60, 80 | 60, 80, 100 |  |  |  |  |  |
|  |  | 10, 15, 20, 40, 50, 60, 80, 90 | 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  | 190 | 1 |
| CA_n48B | CA_n48B | 51, 10 | 10, 15, 20 |  |  |  | 40 | 0 |
|  |  | 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |
|  | - | 10 | 50, 60, 80, 90 |  |  |  | 100 | 1 |
|  |  | 15, 20 | 40, 50, 60, 80 |  |  |  |  |  |
|  |  | 40 | 40, 50, 60 |  |  |  |  |  |
|  |  | 50 | 50 |  |  |  |  |  |
| CA_n48C | - | 10, 15 | 90, 100 |  |  |  | 150 | 0 |
|  |  | 20 | 80, 90, 100 |  |  |  |  |  |
|  |  | 40 | 60, 80, 90, 100 |  |  |  |  |  |
|  |  | 50 | 50, 60, 80, 90, 100 |  |  |  |  |  |
|  |  | 60 | 40, 50, 60, 80, 90 |  |  |  |  |  |
|  |  | 80 | 20, 40, 50, 60 |  |  |  |  |  |
|  |  | 90 | 10, 15, 20, 40, 50, 60 |  |  |  |  |  |
|  |  | 100 | 10, 15, 20, 40, 50 |  |  |  |  |  |
| CA_n66B | - | 5 2 | 20, 40 |  |  |  | 50 | 0 |
|  |  | 10 | 15, 20, 40 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5 2, 10, 15 |  |  |  |  |  |
|  |  | 40 | 5 2, 10 |  |  |  |  |  |
| CA_n71B |  | 5 | 20 |  |  |  | 25 | 0 |
|  |  | 10 | 15 |  |  |  |  |  |
|  |  | 15 | 10 |  |  |  |  |  |
|  |  | 20 | 5 |  |  |  |  |  |
| CA_n77C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 200 | 1 |
|  |  | 20 | 90, 100 |  |  |  |  |  |
|  |  | 30 | 80, 90, 100 |  |  |  |  |  |
|  |  | 40 | 70, 80, 90,100 |  |  |  |  |  |
|  |  | 50 | 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 60 | 50, 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 70 | 40, 50, 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 80 | 30, 40, 50, 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 90 | 20, 25, 30, 40, 50, 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70,80,90, 100 |  |  |  |  |  |
| CA_n78C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | 10 | 100 |  |  |  | 200 | 1 |
|  |  | 20 | 90, 100 |  |  |  |  |  |
|  |  | 30 | 80, 90, 100 |  |  |  |  |  |
|  |  | 40 | 70, 80, 90,100 |  |  |  |  |  |
|  |  | 50 | 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 60 | 50, 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 70 | 40, 50, 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 80 | 30, 40, 50, 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 90 | 20, 25, 30, 40, 50, 60, 70,80,90, 100 |  |  |  |  |  |
|  |  | 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70,80,90, 100 |  |  |  |  |  |
| CA_n78B | - | 20 | 50 |  |  |  | 70 | 0 |
| CA_n79C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
| NOTE 1: Unless otherwise stated, minimum requirements are applicable irrespective of the order of the component carriers. NOTE 2: 5 MHz is not applicable for 30/60 kHz SCS. |  |  |  |  |  |  |  |  |

Table 5.5A.1-2: Void

<End of Changes>