3GPP TSG-RAN WG4 Meeting #94-e                                                            R4-2000238

Online, 24th Feb-6th Mar, 2020

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_2-5_n2 and DC_2-5-5_n2

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_2-5_n2 and DC_2-5-5_n2 according to the request in [1].

2. Reference

- RP-192815, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_2-5_n2 and DC_2-5-5_n2

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n2 | CA_2-5 | n2 | No |
| DC_2-5-5_n2 | CA_2-5-5 | n2 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-5A_n2A | DC_5A_n2A | CA_2A-5A | n2A |
| DC_2A-5B_n2A | DC_5A_n2A | CA_2A-5B | n2A |
| DC_2A-5A-5A_n2A | DC_5A_n2A | CA_2A-5A-5A | n2A |

5.1.66.3	Coexistence studies

Based on co-existence studies of DC_5A_n2A in TR 37.716-11-11, there is no harmonic or IMD issues for this band combination,.

5.1.x.4	∆TIB and ∆RIB values

For DC_2-5_n2, the TIB,c and RIB,c values reused the values of DC_5A_n2A in TS 38.101-3.

Table 5.1.x.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n2 DC_2-5-5_n2 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n2 | 0.3 |

Table 5.1.x.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n2 DC_2-5-5_n2 | 2 | 0 |
|  | 5 | 0 |
|  | n2 | 0 |

### 5.1.x.5	REFSENS requirements

There is no additional requirement for this band combination.

<End of Text Proposal>