3GPP TSG-RAN WG4 Meeting #94-e                                                            R4-2000243

Online, 24th Feb-28th Mar, 2020

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_2-66_n48 and DC_2-66-66_n48

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_2-66_n48 and DC_2-66-66_n48 according to the request in [1].

2. Reference

- RP-192815, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

5.1.x	DC_2-66_n48 and DC_2-66-66_n48

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n48 | CA_2-66 | n48 | No |
| DC_2-66-66_n48 | CA_2-66-66 | n48 | No |

### 5.1.x.2	Configurations for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A | CA_2A-66A | n48A |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A | CA_2A-66A | CA_n48B |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A | CA_2A-66A-66A | n48A |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A | CA_2A-66A-66A | CA_n48B |

### 5.1.x.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, and the impact to own Rx of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band 2 + Band n48 may fall into own Rx of band 66.

- 5th order IMD generated by dual uplink of Band 66 + Band n48 may fall into own Rx of band 2.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_2-66_n48 and DC_2-66-66_n48, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n48 DC_2-66-66_n48 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n48 | 0.8 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n48 DC_2-66-66_n48 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n48 | 0.5 |

### 5.1.x.5	REFSENS requirements

Table 5.1.x.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 12.1 | FDD | IMD4 ¦3*fB2-fBn48¦ |
|  | n48 | 3560 | 5 | 25 | 3560 | N/A | TDD | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1880 | 5 | 25 | 1960 | 28.3 | FDD | IMD5 ¦2*fBn48-3*fB66¦ |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | TDD | N/A |

<End of Text Proposal>