3GPP TSG-RAN WG4 Meeting #94-e                                          R4-2000273

Electronic Meeting, 24th Feb– 6th Mar, 2020

Source:	Samsung

Title:	DSS in LTE/NR band 48/n48

Agenda item:			9.23.2

Document for:	Approval

# 1. Introduction

In RAN#86 meeting the WI on LTE/NR spectrum sharing in band 48/n48 frequency range was agreed “to add new requirements, if needed, for band n48 (only 30 kHz SCS for NR)” with following objectives:

-	Channel raster: Confirm that NR channel raster can be aligned with LTE center frequencies [RAN4];

-	UL shift: Investigate whether the 7.5kHz sub-carrier shift has any impact on the needed guard between LTE and 30kHz SCS NR. Specify the shift only if there is a clear benefit [RAN4];

-	Sync raster: Check mechanisms to avoid overlapping transmissions between NR SSB and LTE CRS. Apply changes to ensure non overlap of NR SSB and LTE CRS if determined that solutions with existing specifications are insufficient [RAN4].

This contribution summarizes our observations on how to enable dynamic spectrum sharing in case of LTE/NR band 48/n48 based on previous contribution in [1].

# 2. Discussion

For NR band n41, as shown in table below the channel raster is defined as SCS based which is different compared with common 100 KHz LTE channel raster. To ensure the LTE and NR dynamic spectrum sharing, the solution with selected raster points which match with LTE channel raster can be used as implementation way. That implies a sparse raster of 300 kHz which is the LCM of 100 kHz and 15 kHz would be applied to enable the DSS with LTE spectrum. This approach can be implemented by network configuration without explicit specification impact. However, it should be admitted that that may result in sparse channel raster, e.g. one-third of LTE channel raster # and one-tenth of NR channel raster occasions, valid for band 48/n48 case. But considering the step size is still in the similar order, it is not expected extraordinary expect in real network.

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |

Observation 1: for channel raster to enable DSS between LTE and NR 48/n48, the 300 kHz raster can be used as implementation approach without specification impact.

As shown in figure below 7.5 kHz UL shift is applied to ensure the UL orthogonal between LTE and NR in case of 15 kHz SCS of NR operation in the context of DSS. It should be noted that it is not precluded the application of UL 7.5 kHz shift of other case. Then for the case of NR operating in 30 kHz SCS, the 7.5 kHz UL shift still could not resolve the issue of UL non-orthogonal between NR and LTE. To resolve this problem, legacy agreement in R15 NR discussion is to apply gap between LTE and NR should as implementation manner.

Observation 2: one use case of UL 7.5 kHz shift is to resolve the non-orthogonal issue between LTE and NR in case of 15 kHz SCS of NR operation.

Observation 3: the legacy solution to avoid non-orthogonal interference for operation between LTE@15 kHz SCS and NR@30 kHz SCS in case of DSS is to adopt guard gap by implementation manner.

To avoid confliction between LTE CRS and NR SSB, SSB pattern B is introduced to resolve the issue and applied for NR band n5 and n66. However, in existing NR specification the SSB pattern defined for n48 is Case C. If we include SSB pattern B for n48 to enable DSS for this spectrum range following the approach of n5 and n66, it would result in additional burden for UE initial cell search due to more SSB entries with increase on power consumption and implementation complexity. Hence it is not preferred to include SSB pattern B for n48 DSS.

| NR operating band | SS Block SCS | SS Block pattern (note) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n48 | 30 kHz | Case C | 7884 – <1> – 7982 |

The other alternative way as proposed in [2] and adopted in discussion of n41/n90 is the MBSFN approach. And our previous contribution in [2] has already provided full background regarding this solution. Hence we submit the analysis on this aspect again for formal discussion under dedicated WI on band 48/n48 DSS.

One concern raised here for MBSFN approach is whether the LTE MBSFN configuration accommodate flexible configuration on SSB allocation for all potential TDD UL/DL configuration. For this purpose, we can compare the LTE configuration and SSB with further detail here.

TDD Uplink-downlink configuration is defined in TS36.211 as table below for LTE.

Table 4.2-2 in TS36.211: Uplink-downlink configurations

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

The MBSFN configuration related parameters are defined in TS36.331 for LTE.

#### –	MBMS-NotificationConfig in TS36.331

The IE MBMS-NotificationConfig specifies the MBMS notification related configuration parameters, that are applicable for all MBSFN areas.

MBMS-NotificationConfig information element

-- ASN1START

MBMS-NotificationConfig-r9 ::=				SEQUENCE {

notificationRepetitionCoeff-r9		ENUMERATED {n2, n4},

notificationOffset-r9				INTEGER (0..10),

notificationSF-Index-r9				INTEGER (1..6)

}

MBMS-NotificationConfig-v1430 ::=				SEQUENCE {

notificationSF-Index-v1430				INTEGER (7..10)

}

-- ASN1STOP

MBMS-NotificationConfig field descriptions

notificationOffset Indicates, together with the notificationRepetitionCoeff, the radio frames in which the MCCH information change notification is scheduled i.e. the MCCH information change notification is scheduled in radio frames for which: SFN mod notification repetition period = notificationOffset.

notificationRepetitionCoeff Actual change notification repetition period common for all MCCHs that are configured= shortest modification period/ notificationRepetitionCoeff. The 'shortest modificaton period' corresponds with the lowest value of mcch-ModificationPeriod of all MCCHs that are configured. Value n2 corresponds to coefficient 2, and so on.

notificationSF-Index Indicates the subframe used to transmit MCCH change notifications on PDCCH. FDD: Value 1, 2, 3, 4, 5 and 6 correspond with subframe #1, #2, #3 #6, #7, and #8 respectively. Value 7, 8, 9 and 10 correspond with subframe #0, #4, #5 and #9 respectively. If notificationSF-Index-v1430 is included, UE ignores notificationSF-Index-r9. TDD: Value 1, 2, 3, 4, and 5 correspond with subframe #3, #4, #7, #8, and #9 respectively.

According to current LTE MBSFN design, for most of the TDD  UL/DL configurations widely reused as real network, there are occasions that DL subframe can be configured as MBSFN. And for FDD case, all DL subframes in radio frame can be configured as MBSFN subframe.

Observation 4: MBSFN approach can be applied to resolved LTE CRS and NR SSB collision for TDD NR band refarmed from LTE to support DSS for all extensively used LTE TDD UL/DL configurations.

Observation 5: MBSFN approach can be applied to resolved LTE CRS and NR SSB collision for FDD NR band refarmed from LTE to support DSS without any restriction on DL subframe configuration.

# 3. Conclusion

In this contribution, the potential impact on RAN4 to enable DSS on LTE existing bands was analyzed with below observations:

Observation 1: for channel raster to enable DSS between LTE and NR 48/n48, the 300 kHz raster can be used as implementation approach without specification impact.

Observation 2: one use case of UL 7.5 kHz shift is to resolve the non-orthogonal issue between LTE and NR in case of 15 kHz SCS of NR operation.

Observation 3: the legacy solution to avoid non-orthogonal interference for operation between LTE@15 kHz SCS and NR@30 kHz SCS in case of DSS is to adopt guard gap by implementation manner.

Observation 4: MBSFN approach can be applied to resolved LTE CRS and NR SSB collision for TDD NR band refarmed from LTE to support DSS for all extensively used LTE TDD UL/DL configurations.

Observation 5: MBSFN approach can be applied to resolved LTE CRS and NR SSB collision for FDD NR band refarmed from LTE to support DSS without any restriction on DL subframe configuration.

# 4. Reference

[1] RP-193213, New WID: LTE/NR spectrum sharing in band 48/n48 frequency range

[2] R4-1913331, Further discussion on DSS in LTE refarming bands