3GPP TSG-RAN WG4 Meeting #94-e                                          R4-2000282

Electronic Meeting, 24th Feb– 6th Mar, 2020

Source:	Samsung

Title:	OTA in-band selectivity for IAB-MT

Agenda item:			8.5.4.2.2

Document for:	Approval

# 1. Introduction

This contribution provides considerations on OTA In-band selectivity requirement for FR2 IAB-MT

# 2. Discussion

In band selectivity requirement for receiver contains both ACS and In-band blocking requirement. For ACS requirement it is agreed in RAN4#92 bis meeting as below:

| Frequency range | ACS for IAB-MT |
| --- | --- |
| FR2 | ACS [23.5]dB same as BS for <37 GHz ACS [22.5]dB same as BS above >37 GHz |

It should be indicated that the simulation assumption for IAB-MT co-existence study is referred to WP5D response as

# Table 2: NR ACS values for the WP5D response.

| Frequency range | 30 GHz (24.24 – 33.4 GHz) | 45GHz (37 – 52.6 GHz) | 70 GHz (66 – 86 GHz) |
| --- | --- | --- | --- |
| BS ACS | 23.5 dB | 22.5 dB | 21.5 dB |
| UE ACS | 22.5dB | 21.5dB | 20.5dB |

But in NR Rel-15 following discussion it is rounded up to the integer value as below.

| Frequency range | 30 GHz (24.24 – 33.4 GHz) | 45GHz (37 – 52.6 GHz) |
| --- | --- | --- |
| BS ACS | 24 dB | 23 dB |
| UE ACS | 23dB | 22dB |

Consequently, the conclusion of IAB-MT ACS should be updated as 24dB for 28GHz and 23dB for 39GHz respectively.

For IAB-MT reception, which is victim of other gNB DL transmission, according to our simulation result achieved in co-existence study, IAB-MT reception will face increased co-channel interference than normal UE due to the antenna location and size. It’s expected that the IAB-MT should be operating with relative higher wanted signal level from the min REFSENS condition. Hence the delta value on top of REFSENS should be no less than the level of corresponding UE requirement, i.e. 14dB for wanted signal. The other aspect is since no maximum input level for IAB-MT reception, there is no need to have two cases for ACS requirement.

UE ACS and IBB share the same selectivity level but still ensure the good throughput performance. But BS IBB is assessed by statistic worst case condition with probability from 99% to 99.9%. And the conclusion is that the BS IBB selectivity is several dB higher compared to ACS to cope with the higher interference signal level in IBB range. If we take case of IAB-MT, as mentioned above, the main impact on IAB-MT reception would be co-channel interference. According to our observation of co-existence simulation result, it is expected that the keep the same selectivity of ACS for IAB-MT IBB could still ensure the system performance.

# 3. Summary

The proposals on the FR2 IAB-MT in this contribution are summarized as below:

| Frequency range | 30 GHz (24.24 – 33.4 GHz) |  | 45GHz (37 – 52.6 GHz) |
| --- | --- | --- | --- |
| IAB MT ACS | 24 dB |  | 23 dB |
| IAB-MT IBB selectivity | 24 dB |  | 23 dB |
| Wanted signal/dBm | REFSENS+[14dB] |  |  |
| Interference signal/dBm | REFSENS+[36.5dB] | REFSENS+[35.5dB] |  |

Please note that the in-band selectivity requirement should be further updated deepened on the approach for REFSENS definition of IAB-MT.

# 4. Reference

[1] R4-1916161, IAB Ad Hoc meeting minutes

[2] R4-200XXXX, IAB REFSENS

[3]R4-1908075, Simulation result for layout

[4]R4-1908076, Simulation result for layout 2