3GPP TSG-RAN WG4 Meeting #94-e		            R4-2000305

Online, 24th February - 6th March, 2020

Agenda item:	8.17.2.1.4

Source:	Samsung

Title:	Simulation results for UE demodulation requirements under fading channel with high Doppler value

Document for:	Discussion

# Introduction

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we provide simulation results for TDLC channel based on the assumptions agreed in RAN4#93 found in [1].

# Common test setup

Table1: Simulation assumption for multi-path fading channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 and 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | - 600Hz | - 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |

# Simulation results

## FDD Simulation results

Figure 3-1:     Simulation results for NR PDSCH FDD under multi-path fading TDLC channel

## TDD Simulation results

Figure 3-2:     Simulation results for NR PDSCH TDD under multi-path fading TDLC channel

Observation 1:  The SNR with 70% TP for MCS 17 is not achievable with Rank2 for both FDD and TDD with 2x2 antenna configuration

Observation 2:  The SNR with 70% TP for MCS 13 is high under Doppler value 1200Hz with Rank2 with 2x2 antenna configuration in TDD

Based on the above observation, we prefer to specify the requirement with MCS 4, MCS 13, MCS 17 for rank1, and MCS4 only for rank2.

Proposal 1:  Specify the requirement with MCS4, MCS13 and MCS 17 for rank1, MCS4 for rank2.

## Summary of @ 70% of max throughput

Table 2: Ideal Simulation results for NR PDSCH under multi-path fading channel

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | Rank | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 1 | 4 | -1.5 |
| 2 |  |  |  |  | 13 | 7.1 |
| 3 |  |  |  |  | 17 | 11.7 |
| 4 |  |  |  | 2 | 4 | 2.6 |
| 5 |  |  |  |  | 13 | 17.5 |
| 6 |  |  |  |  | 17 | N/A |
| 7 |  |  | 2Tx 4Rx ULA Low | 1 | 4 | -4.1 |
| 8 |  |  |  |  | 13 | 3.2 |
| 9 |  |  |  |  | 17 | 6.9 |
| 10 |  |  |  | 2 | 4 | -0.9 |
| 11 |  |  |  |  | 13 | 9.1 |
| 12 |  |  |  |  | 17 | 15.3 |
| 13 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 1 | 4 | -2.1 |
| 14 |  |  |  |  | 13 | 7.2 |
| 15 |  |  |  |  | 17 | 10.9 |
| 16 |  |  |  | 2 | 4 | 3.4 |
| 17 |  |  |  |  | 13 | 18.9 |
| 18 |  |  |  |  | 17 | N/A |
| 19 |  |  | 2Tx 4Rx ULA Low | 1 | 4 | -4.6 |
| 20 |  |  |  |  | 13 | 4.1 |
| 21 |  |  |  |  | 17 | 8.0 |
| 22 |  |  |  | 2 | 4 | -0.2 |
| 23 |  |  |  |  | 13 | 13.3 |
| 24 |  |  |  |  | 17 | 16.7 |

# Conclusion

In this contribution we provide simulation results for Multi-Path Fading channels – TDLC channel.

Observation 1:  The SNR with 70% TP for MCS 17 is not achievable under Doppler value with Rank2 for both FDD and TDD with 2x2 antenna configuration

Observation 2:  The SNR with 70% TP for MCS 13 is high under Doppler value 1200Hz with Rank2 with 2x2 antenna configuration in TDD

Proposal 1:  Specify the requirement with MCS4, MCS13 and MCS 17 for Rank1, MCS4 for Rank2.

# Reference

- R4-1915926 , Draft WF on UE demodulation for NR HST, RAN4#93, CMCC

- R4-1912808, WF on UE demodulation for NR HST, RAN4#92Bis, CMCC