3GPP TSG-RAN WG4 Meeting # 94-e	R4-2000358

# E-meeting, 24th February 2020 - 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0034 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-4: Editorial corrections (R15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Tables with TDD configurations and FRC contain several typos and formating misaligment Titles naming of sections A.1.2 and A.1.3 is not aligned |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated ‘pattern1’ and ‘pattern2’ paremeter fields in tables with TDD configurations - Modified title for section A.1.3 from “TDD UL-DL configuration for FR2” to “TDD UL-DL configurations for FR2” to align with title naming for FR1 - Corrected unit for “Number of Code Blocks” in E-UTRA FRC tables - Removed row with “Maximum number of HARQ transmissions” configuration from several TDD FRC tables - Added missing units in Table A.3.2.2.2-8 - Aligned formatting for several rows in tables A.3.2.2.2-8 and A.3.2.2.5-8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Typos and formating misaligment in tables with TDD configurations and FRC. Mislaignment in naming of sections A.1.2 and A.1.3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.2, A.1.3, A.3.2.1.4, A.3.2.2.2, A.3.2.2.5, A.3.2.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## A.1.2	TDD UL-DL configurations for FR1

TDD UL-DL configurations for performance requirements are provided in Tables A.1.2-1, A.1.2-2, and A.1.2-3.

Table A.1.2-1: TDD UL-DL configuration for SCS 15 kHz

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR1.15-1 |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| referenceSubcarrierSpacing |  | kHz | 15 |
| pattern1 | dl-UL-TransmissionPeriodicity | ms | 5 |
|  | nrofDownlinkSlots |  | 3 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 1 |
|  | nrofUplinkSymbols |  | 2 |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  |  | 4 if mod(i,5) = 0 3 if mod(i,5) = 1 2 if mod(i,5) = 2 6 if mod(i,5) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9}. |  |  |  |

Table A.1.2-2: TDD UL-DL configuration for SCS 30 kHz

| Parameter |  | Unit | UL-DL pattern |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1.30-1 | FR1.30-2 | FR1.30-3 | FR1.30-4 | FR1.30-5 | FR1.30-6 |
| TDD Slot Configuration pattern (Note 1) |  |  | 7DS2U | DDDSU | DDDSUDDSUU | DDDSUUDDDD | DSUU | DS1S2U |
| Special Slot Configuration (Note 2) |  |  | 6D+4G+4U | 10D+2G+2U | 10D+2G+2U | 6D+4G+4U | 12D+2G | S1: 10D+2G+2U S2: 12D+2G+0U |
| referenceSubcarrierSpacing |  | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
| pattern1 |  |  |  |  |  |  |  |  |
|  | dl-UL-TransmissionPeriodicity | ms | 5 | 2.5 | 2.5 | 3 | 2 | 1 |
|  | nrofDownlinkSlots |  | 7 | 3 | 3 | 3 | 1 | 1 |
|  | nrofDownlinkSymbols |  | 6 | 10 | 10 | 6 | 12 | 10 |
|  | nrofUplinkSlot |  | 2 | 1 | 1 | 2 | 2 | 0 |
|  | nrofUplinkSymbols |  | 4 | 2 | 2 | 4 | 0 | 2 |
| pattern2 |  |  |  |  |  |  |  |  |
|  | dl-UL-TransmissionPeriodicity | ms | N/A | N/A | 2.5 | 2 | N/A | 1 |
|  | nrofDownlinkSlots |  | N/A | N/A | 2 | 4 | N/A | 0 |
|  | nrofDownlinkSymbols |  | N/A | N/A | 10 | 0 | N/A | 12 |
|  | nrofUplinkSlot |  | N/A | N/A | 2 | 0 | N/A | 1 |
|  | nrofUplinkSymbols |  | N/A | N/A | 2 | 0 | N/A | 0 |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  |  | 8 if mod(i,10) = 0 7 if mod(i,10) = 1 6 if mod(i,10) = 2 5 if mod(i,10) = 3 5 if mod(i,10) = 4 4 if mod(i,10) = 5 3 if mod(i,10) = 6 2 if mod(i,10) = 7 | 4 if mod(i,5) = 0 3 if mod(i,5) = 1 2 if mod(i,5) = 2 6 if mod(i,5) = 3 | 4 if mod(i,10) = 0 3 if mod(i,10) = 1 2 if mod(i,10) = 2 5 if mod(i,10) = 3 3 if mod(i,10) = 5 3 if mod(i,10) = 6 2 if mod(i,10) = 7 | 5 if mod(i,10) = 0 4 if mod(i,10) = 1 3 if mod(i,10) = 2 2 if mod(i,10) = 3 8 if mod(i,10) = 6 7 if mod(i,10) = 7 6 if mod(i,10) = 8 5 if mod(i,10) = 9 | 3 if mod(i,4) = 0 2 if mod(i,4) = 1 | 3 if mod(i,4) = 0 2 if mod(i,4) = 1 3 if mod(i,4) = 2 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} |  |  |  |  |  |  |  |  |

Table A.1.2-2a: TDD UL-DL configuration for SCS 30 kHz for DCI-based dynamic UL/DL detection

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR1.30-1A |
| TDD Slot Configuration pattern (Note 1) |  |  | 7DS2U |
| Special Slot Configuration (Note 2) |  |  | 6D+4G+4U |
| referenceSubcarrierSpacing |  | kHz | N/A |
| pattern1 (Note 4) |  |  |  |
|  | dl-UL-TransmissionPeriodicity | ms | N/A |
|  | nrofDownlinkSlots |  | N/A |
|  | nrofDownlinkSymbols |  | N/A |
|  | nrofUplinkSlot |  | N/A |
|  | nrofUplinkSymbols |  | N/A |
| pattern2 (Note 4) |  |  |  |
|  | dl-UL-TransmissionPeriodicity | ms | N/A |
|  | nrofDownlinkSlots |  | N/A |
|  | nrofDownlinkSymbols |  | N/A |
|  | nrofUplinkSlot |  | N/A |
|  | nrofUplinkSymbols |  | N/A |
| PDCCH DCI Configuration | DCI Format |  | 1-1 for slot indices with mod(i,10) = 0,1,2,3,4,5,6,7 |
|  | Scheduled Grant |  | Symbol 2-13 for slot indices with mod(i,10) = 0,1,2,3,4,5,6 and Symbol 2-5 for slot indices with mod(i,10) = 7 |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) (PDSCH-to-HARQ-timing-indicator) |  |  | 8 if mod(i,10) = 0 7 if mod(i,10) = 1 6 if mod(i,10) = 2 5 if mod(i,10) = 3 5 if mod(i,10) = 4 4 if mod(i,10) = 5 3 if mod(i,10) = 6 2 if mod(i,10) = 7 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G and U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: Do not configure tdd-UL-DL-semi-statically using RRC configuration. |  |  |  |

## A.1.3	TDD UL-DL configurations for FR2

<SKIP UNCHANGED PART>

Table A.1.3-2a: TDD UL-DL configuration for SCS 120 kHz for DCI-based dynamic UL/DL detection

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR2.120-1A |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| referenceSubcarrierSpacing |  | kHz | N/A |
| pattern1 (Note 4) | dl-UL-TransmissionPeriodicity | ms | N/A |
|  | nrofDownlinkSlots |  | N/A |
|  | nrofDownlinkSymbols |  | N/A |
|  | nrofUplinkSlot |  | N/A |
|  | nrofUplinkSymbols |  | N/A |
| pattern2 (Note 4) |  |  |  |
|  | dl-UL-TransmissionPeriodicity | ms | N/A |
|  | nrofDownlinkSlots |  | N/A |
|  | nrofDownlinkSymbols |  | N/A |
|  | nrofUplinkSlot |  | N/A |
|  | nrofUplinkSymbols |  | N/A |
| PDCCH DCI Configuration | DCI Format |  | 1-1 for slot indices with mod(i,5) = 0,1,2,3 |
|  | Scheduled Grant |  | Symbol 1-13 for slot indices with mod(i,5) = 0,1,2 and Symbol 1-9 for slot indices with mod(i,5) = 3 |
| The number of slots between PDSCH and corresponding HARQ-ACK information(Note 3) |  |  | 4 if mod(i,5) = 0 3 if mod(i,5) = 1 2 if mod(i,5) = 2 6 if mod(i,5) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G and U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,79} Note 4: Do not tdd-UL-DL-ConfigurationCommon semi-statically using RRC configuration. |  |  |  |

<SKIP UNCHANGED PART>

#### A.3.2.1.4	Reference measurement channels for E-UTRA

Table A.3.2.1.4-1: PDSCH Reference Channel for sustained data-rate test (64QAM, 2 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.4-1.1 FDD | R.PDSCH.4-1.2 FDD | R.PDSCH.4-1.3 FDD | R.PDSCH.4-1.4 FDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | Note 6 | Note 7 | Note 8 | Note 9 |
| Allocated subframes per Radio Frame |  | 9 | 10 | 10 | 10 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9, |  | 0.85 | 0.85 | 0.85 | 0.88 |
| For Sub-Frame 5 |  | N/A | 0.89 | 0.91 | 0.87 |
| For Sub-Frame 0 |  | 0.83 | 0.90 | 0.88 | 0.90 |
| Information Bit Payload (Note 3) |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 18336 | 36696 | 55056 | 75376 |
| For Sub-Frame 5 | Bits | N/A | 35160 | 52752 | 71112 |
| For Sub-Frame 0 | Bits | 15840 | 36696 | 55056 | 75376 |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | CBs | 3 | 6 | 9 | 13 |
| For Sub-Frame 5 | CBs | N/A | 6 | 9 | 12 |
| For Sub-Frame 0 | CBs | 3 | 6 | 9 | 13 |
| Binary Channel Bits (Note 3) |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 21600 | 43200 | 64800 | 86400 |
| For Sub-Frame 5 | Bits | N/A | 39744 | 60480 | 82080 |
| For Sub-Frame 0 | Bits | 19152 | 40752 | 62352 | 83952 |
| Number of layers |  | 2 | 2 | 2 | 2 |
| Max. Throughput averaged over 1 frame (Note 3) | Mbps | 16.253 | 36.542 | 54.826 | 74.950 |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: Given per component carrier per codeword. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 6: Resource blocks nPRB = 0..24 in sub-frames 0,1,2,3,4,6,7,8,9. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,1,2,3,4,6,7,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,1,2,3,4,6,7,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,1,2,3,4,6,7,8,9. |  |  |  |  |  |

Table A.3.2.1.4-2: PDSCH Reference Channel for sustained data-rate test (64QAM, 4 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.4-2.1 FDD | R.PDSCH.4-2.2 FDD | R.PDSCH.4-2.3 FDD | R.PDSCH.4-2.4 FDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | Note 6 | Note 7 | Note 8 | Note 9 |
| Allocated subframes per Radio Frame |  | 9 | 10 | 10 | 10 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9, |  | 0.78 | 0.78 | 0.77 | 0.79 |
| For Sub-Frame 5 |  | N/A | 0.80 | 0.79 | 0.81 |
| For Sub-Frame 0 |  | 0.85 | 0.83 | 0.8 | 0.81 |
| Information Bit Payload (Note 3) |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 31704 | 63776 | 93800 | 128496 |
| For Sub-Frame 5 | Bits | N/A | 59256 | 90816 | 124464 |
| For Sub-Frame 0 | Bits | 30576 | 63776 | 93800 | 128496 |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | CBs | 6 | 11 | 16 | 21 |
| For Sub-Frame 5 | CBs | N/A | 10 | 15 | 21 |
| For Sub-Frame 0 | CBs | 5 | 11 | 16 | 21 |
| Binary Channel Bits (Note 3) |  |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 40800 | 81600 | 122400 | 163200 |
| For Sub-Frame 5 | Bits | N/A | 74976 | 114144 | 154944 |
| For Sub-Frame 0 | Bits | 36192 | 76992 | 117792 | 158592 |
| Number of layers |  | 4 | 4 | 4 | 4 |
| Max. Throughput averaged over 1 frame (Note 3) | Mbps | 28.421 | 63.324 | 93.502 | 128.093 |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: Given per component carrier per codeword. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 6: Resource blocks nPRB = 0..24 in sub-frames 0,1,2,3,4,6,7,8,9. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,1,2,3,4,6,7,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,1,2,3,4,6,7,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,1,2,3,4,6,7,8,9. |  |  |  |  |  |

Table A.3.2.1.4-3: PDSCH Reference Channel for sustained data-rate test (256QAM, 2 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.4-3.1 FDD | R.PDSCH.4-3.2 FDD | R.PDSCH.4-3.3 FDD | R.PDSCH.4-3.4 FDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | Note 6 | Note 7 | Note 8 | Note 9 |
| Allocated subframes per Radio Frame |  | 10 | 10 | 10 | 10 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 |  | 0.85 | 0.85 | 0.88 | 0.85 |
| For Sub-Frames 1,2,6,7 |  | 0.77 | 0.74 | 0.74 | 0.74 |
| For Sub-Frame 5 |  | 0.79 | 0.77 | 0.77 | 0.75 |
| For Sub-Frame 0 |  | 0.84 | 0.78 | 0.77 | 0.76 |
| Information Bit Payload (Note 3) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 24496 | 48936 | 75376 | 97896 |
| For Sub-Frames 1,2,6,7 | Bits | 21384 | 42368 | 63776 | 84760 |
| For Sub-Frame 5 | Bits | 19848 | 40576 | 61664 | 81176 |
| For Sub-Frame 0 | Bits | 21384 | 42368 | 63776 | 84760 |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | CBs | 4 | 8 | 13 | 16 |
| For Sub-Frames 1,2,6,7 | CBs | 4 | 7 | 11 | 14 |
| For Sub-Frame 5 | CBs | 4 | 7 | 11 | 14 |
| For Sub-Frame 0 | CBs | 4 | 7 | 11 | 14 |
| Binary Channel Bits (Note 3) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 28800 | 57600 | 86400 | 115200 |
| For Sub-Frames 1,2,6,7 | Bits | 28800 | 57600 | 86400 | 115200 |
| For Sub-Frame 5 | Bits | 25344 | 52992 | 80640 | 109440 |
| For Sub-Frame 0 | Bits | 25536 | 54336 | 83136 | 111936 |
| Number of layers |  | 2 | 2 | 2 | 2 |
| Max. Throughput averaged over 1 frame (Note 3) | Mbps | 22.475 | 44.816 | 68.205 | 89.656 |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: Given per component carrier per codeword. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 6: Resource blocks nPRB = 2..24 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..24 in sub-frames 0,1,2,3,4,6,7,8,9. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,1,2,3,4,6,7,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,1,2,3,4,6,7,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,1,2,3,4,6,7,8,9. |  |  |  |  |  |

Table A.3.2.1.4-4: PDSCH Reference Channel for sustained data-rate test (256QAM, 4 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.4-4.1 FDD | R.PDSCH.4-4.2 FDD | R.PDSCH.4-4.3 FDD | R.PDSCH.4-4.4 FDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | Note 6 | Note 7 | Note 8 | Note 9 |
| Allocated subframes per Radio Frame |  | 10 | 10 | 10 | 10 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 |  | 0.85 | 0.78 | 0.79 | 0.78 |
| For Sub-Frames 1,2,6,7 |  | 0.77 | 0.78 | 0.79 | 0.78 |
| For Sub-Frame 5 |  | 0.79 | 0.82 | 0.82 | 0.786 |
| For Sub-Frame 0 |  | 0.84 | 0.83 | 0.82 | 0.80 |
| Information Bit Payload (Note 3) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 42368 | 84760 | 128496 | 169544 |
| For Sub-Frames 1,2,6,7 | Bits | 42368 | 84760 | 128496 | 169544 |
| For Sub-Frame 5 | Bits | 39232 | 81176 | 124464 | 161760 |
| For Sub-Frame 0 | Bits | 39232 | 84760 | 128496 | 169544 |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | CBs | 7 | 14 | 21 | 28 |
| For Sub-Frames 1,2,6,7 | CBs | 7 | 14 | 21 | 28 |
| For Sub-Frame 5 | CBs | 7 | 14 | 21 | 27 |
| For Sub-Frame 0 | CBs | 7 | 14 | 21 | 28 |
| Binary Channel Bits (Note 3) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 54400 | 108800 | 163200 | 217600 |
| For Sub-Frames 1,2,6,7 | Bits | 54400 | 108800 | 163200 | 217600 |
| For Sub-Frame 5 | Bits | 47744 | 99968 | 152192 | 206592 |
| For Sub-Frame 0 | Bits | 48256 | 102656 | 157056 | 211456 |
| Number of layers |  | 4 | 4 | 4 | 4 |
| Max. Throughput averaged over 1 frame (Note 3) | Mbps | 41.741 | 84.4016 | 128.093 | 168.766 |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: Given per component carrier per codeword. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 6: Resource blocks nPRB = 2..24 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..24 in sub-frames 0,1,2,3,4,6,7,8,9. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,1,2,3,4,6,7,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,1,2,3,4,6,7,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,1,2,3,4,6,7,8,9. |  |  |  |  |  |

Table A.3.2.1.4-5: PDSCH Reference Channel for sustained data-rate test (1024QAM, 2 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.4-5.1 FDD | R.PDSCH.4-5.2 FDD | R.PDSCH.4-5.3 FDD | R.PDSCH.4-5.4 FDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | Note 6 | Note 7 | Note 8 | Note 9 |
| Allocated subframes per Radio Frame |  | 10 | 10 | 10 | 10 |
| Modulation |  | 1024QAM | 1024QAM | 1024QAM | 1024QAM |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 |  | 0.76 | 0.73 | 0.75 | 0.76 |
| For Sub-Frames 1,2,6,7 |  | 0.76 | 0.73 | 0.75 | 0.76 |
| For Sub-Frame 5 |  | 0.80 | 0.77 | 0.78 | 0.77 |
| For Sub-Frame 0 |  | 0.86 | 0.78 | 0.78 | 0.79 |
| Information Bit Payload (Note 3) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 27376 | 52752 | 81176 | 110136 |
| For Sub-Frames 1,2,6,7 | Bits | 27376 | 52752 | 81176 | 110136 |
| For Sub-Frame 5 | Bits | 25456 | 51024 | 78704 | 105528 |
| For Sub-Frame 0 | Bits | 27376 | 52752 | 81176 | 110136 |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | CBs | 5 | 9 | 14 | 18 |
| For Sub-Frames 1,2,6,7 | CBs | 5 | 9 | 14 | 18 |
| For Sub-Frame 5 | CBs | 5 | 9 | 13 | 18 |
| For Sub-Frame 0 | CBs | 5 | 9 | 14 | 18 |
| Binary Channel Bits (Note 3) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 36000 | 72000 | 108000 | 144000 |
| For Sub-Frames 1,2,6,7 | Bits | 36000 | 72000 | 108000 | 144000 |
| For Sub-Frame 5 | Bits | 31680 | 66240 | 100800 | 136800 |
| For Sub-Frame 0 | Bits | 31920 | 67920 | 103920 | 139920 |
| Number of layers |  | 2 | 2 | 2 | 2 |
| Max. Throughput averaged over 1 frame (Note 3) | Mbps | 27.18 | 52.58 | 80.93 | 109.68 |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: Given per component carrier per codeword. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 6: Resource blocks nPRB = 2..24 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..24 in sub-frames 0,1,2,3,4,6,7,8,9. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,1,2,3,4,6,7,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,1,2,3,4,6,7,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,1,2,3,4,6,7,8,9. |  |  |  |  |  |

Table A.3.2.1.4-6: PDSCH Reference Channel for sustained data-rate test (1024QAM, 4 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.4-6.1 FDD | R.PDSCH.4-6.2 FDD | R.PDSCH.4-6.3 FDD | R.PDSCH.4-6.4 FDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | Note 6 | Note 7 | Note 8 | Note 9 |
| Allocated subframes per Radio Frame |  | 10 | 10 | 10 | 10 |
| Modulation |  | 1024QAM | 1024QAM | 1024QAM | 1024QAM |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 |  | 0.78 | 0.81 | 0.79 | 0.81 |
| For Sub-Frames 1,2,6,7 |  | 0.78 | 0.81 | 0.79 | 0.81 |
| For Sub-Frame 5 |  | 0.82 | 0.81 | 0.83 | 0.82 |
| For Sub-Frame 0 |  | 0.87 | 0.86 | 0.82 | 0.83 |
| Information Bit Payload (Note 3) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 52752 | 110136 | 161760 | 220296 |
| For Sub-Frames 1,2,6,7 | Bits | 52752 | 110136 | 161760 | 220296 |
| For Sub-Frame 5 | Bits | 48936 | 101840 | 157432 | 211936 |
| For Sub-Frame 0 | Bits | 52752 | 110136 | 161760 | 220296 |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | CBs | 9 | 18 | 27 | 36 |
| For Sub-Frames 1,2,6,7 | CBs | 9 | 18 | 27 | 36 |
| For Sub-Frame 5 | CBs | 8 | 17 | 26 | 35 |
| For Sub-Frame 0 | CBs | 9 | 18 | 27 | 36 |
| Binary Channel Bits (Note 3) |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 68000 | 136000 | 204000 | 272000 |
| For Sub-Frames 1,2,6,7 | Bits | 68000 | 136000 | 204000 | 272000 |
| For Sub-Frame 5 | Bits | 59680 | 124960 | 190240 | 258240 |
| For Sub-Frame 0 | Bits | 60320 | 128320 | 196320 | 264320 |
| Number of layers |  | 4 | 4 | 4 | 4 |
| Max. Throughput averaged over 1 frame (Note 3) | Mbps | 52.37 | 109.31 | 161.33 | 219.46 |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: Given per component carrier per codeword. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 6: Resource blocks nPRB = 2..24 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..24 in sub-frames 0,1,2,3,4,6,7,8,9. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,1,2,3,4,6,7,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,1,2,3,4,6,7,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,1,2,3,4,6,7,8,9. |  |  |  |  |  |

<SKIP UNCHANGED PART>

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.2.2-4: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-4.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 256QAM |  |  |  |  |
| MCS index |  | 24 |  |  |  |  |
| Modulation |  | 256QAM |  |  |  |  |
| Target Coding Rate |  | 0.82 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 29192 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 92200 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 11 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 106848 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 35616 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 111936 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 130.308 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

Table A.3.2.2.2-6: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-3

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-6.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} |  | 8 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 27 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 5376 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} | Bits | 8456 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i = 20, 21 | Bits | 26712 |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 17808 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,6} for i from {1,…,19,22,…,39} | Bits | 27984 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 10.184 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

Table A.3.2.2.2-8: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR1.30-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-8.1 TDD | R.PDSCH.2-8.2 TDD |  |  |  |
| Channel bandwidth | MHz | 40 | 40 |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks | PRBs | 106 | 106 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 23 | 23 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 2 |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 24576 | 49176 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24576 | 49176 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | CBs | N/A | N/A |  |  |  |
| For Slot i = 20 | CBs | 3 | 6 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | CBs | 3 | 6 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 48336 | 96672 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 50880 | 101760 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 28.2624 | 56.5524 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

<SKIP UNCHANGED PART>

Table A.3.2.2.5-8: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-2 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-8.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 59 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 14344 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 14344 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i = 80 | CBs | 2 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,8) =1 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80 | Bits | 28776 |  |  |  |  |
| For Slot i, if mod(i, 8) = {0,4,5} for i from {1,…,79,82,…,159} | Bits | 30360 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 42.3148 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

#### A.3.2.2.6	Reference measurement channels for E-UTRA

Table A.3.2.2.6-1: PDSCH Reference Channel for sustained data-rate test (64QAM, 2 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.6-1.1 TDD | R.PDSCH.6-1.2 TDD | R.PDSCH.6-1.3 TDD |  |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |
| Allocated resource blocks |  | Note 7 | Note 8 | Note 9 |  |
| Uplink-Downlink Configuration (Note 3) |  | 2 | 2 | 2 |  |
| Number of HARQ Processes per component carrier |  | 10 | 10 | 10 |  |
| Allocated subframes per Radio Frame (D+S) |  | 6 | 6 | 6 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 |  | N/A | N/A | N/A |  |
| For Sub-Frames 3,4,8,9 |  | 0.85 | 0.85 | 0.88 |  |
| For Sub-Frame 5 |  | 0.88 | 0.87 | 0.87 |  |
| For Sub-Frame 0 |  | 0.90 | 0.88 | 0.90 |  |
| Information Bit Payload (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4,8,9 | Bits | 36696 | 55056 | 75376 |  |
| For Sub-Frame 5 | Bits | 35160 | 52752 | 71112 |  |
| For Sub-Frame 0 | Bits | 36696 | 55056 | 75376 |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | CBs | N/A | N/A | N/A |  |
| For Sub-Frames 3,4,8,9 | CBs | 6 | 9 | 13 |  |
| For Sub-Frame 5 | CBs | 6 | 9 | 12 |  |
| For Sub-Frame 0 | CBs | 6 | 9 | 13 |  |
| Binary Channel Bits (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4,8,9 | Bits | 43200 | 64800 | 86400 |  |
| For Sub-Frame 5 | Bits | 40176 | 60912 | 82512 |  |
| For Sub-Frame 0 | Bits | 41184 | 62784 | 84384 |  |
| Number of layers |  | 2 | 2 | 2 |  |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 21.864 | 32.803 | 44.799 |  |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: As per Table 4.2-2 in TS 36.211 [15]. Note 4: Given per component carrier per codeword. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 6: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,3,4,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,3,4,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,3,4,8,9. |  |  |  |  |  |

Table A.3.2.2.6-2: PDSCH Reference Channel for sustained data-rate test (64QAM, 4 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.6-2.1 TDD | R.PDSCH.6-2.2 TDD | R.PDSCH.6-2.3 TDD |  |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |
| Allocated resource blocks |  | Note 7 | Note 8 | Note 9 |  |
| Uplink-Downlink Configuration (Note 3) |  | 2 | 2 | 2 |  |
| Number of HARQ Processes per component carrier |  | 10 | 10 | 10 |  |
| Allocated subframes per Radio Frame (D+S) |  | 6 | 6 | 6 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 |  | N/A | N/A | N/A |  |
| For Sub-Frames 3,4,8,9 |  | 0.78 | 0.77 | 0.79 |  |
| For Sub-Frame 5 |  | 0.79 | 0.79 | 0.80 |  |
| For Sub-Frame 0 |  | 0.82 | 0.79 | 0.81 |  |
| Information Bit Payload (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4,8,9 | Bits | 63776 | 93800 | 128496 |  |
| For Sub-Frame 5 | Bits | 59256 | 90816 | 124464 |  |
| For Sub-Frame 0 | Bits | 63776 | 93800 | 128496 |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | CBs | N/A | N/A | N/A |  |
| For Sub-Frames 3,4,8,9 | CBs | 11 | 16 | 21 |  |
| For Sub-Frame 5 | CBs | 10 | 15 | 21 |  |
| For Sub-Frame 0 | CBs | 11 | 16 | 21 |  |
| Binary Channel Bits (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4,8,9 | Bits | 81600 | 122400 | 163200 |  |
| For Sub-Frame 5 | Bits | 75840 | 115008 | 155808 |  |
| For Sub-Frame 0 | Bits | 77856 | 118656 | 159456 |  |
| Number of layers |  | 4 | 4 | 4 |  |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 37.813 | 55.981 | 76.694 |  |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: As per Table 4.2-2 in TS 36.211 [15]. Note 4: Given per component carrier per codeword. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 6: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,3,4,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,3,4,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,3,4,8,9. |  |  |  |  |  |

Table A.3.2.2.6-3: PDSCH Reference Channel for sustained data-rate test (256QAM, 2 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.6-3.1 TDD | R.PDSCH.6-3.2 TDD | R.PDSCH.6-3.3 TDD |  |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |
| Allocated resource blocks |  | Note 7 | Note 8 | Note 9 |  |
| Uplink-Downlink Configuration (Note 3) |  | 2 | 2 | 2 |  |
| Number of HARQ Processes per component carrier |  | 10 | 10 | 10 |  |
| Allocated subframes per Radio Frame (D+S) |  | 6 | 6 | 6 |  |
| Modulation |  | 256QAM | 256QAM | 256QAM |  |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 |  | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 |  | 0.74 | 0.79 | 0.74 |  |
| For Sub-Frames 8,9 |  | 0.85 | 0.88 | 0.85 |  |
| For Sub-Frame 5 |  | 0.76 | 0.76 | 0.74 |  |
| For Sub-Frame 0 |  | 0.78 | 0.77 | 0.76 |  |
| Information Bit Payload (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | Bits | 42368 | 63776 | 84760 |  |
| For Sub-Frames 8,9 | Bits | 48936 | 75376 | 97896 |  |
| For Sub-Frame 5 | Bits | 40576 | 61664 | 81176 |  |
| For Sub-Frame 0 | Bits | 42368 | 63776 | 84760 |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | CBs | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | CBs | 7 | 11 | 14 |  |
| For Sub-Frames 8,9 | CBs | 8 | 13 | 16 |  |
| For Sub-Frame 5 | CBs | 7 | 11 | 14 |  |
| For Sub-Frame 0 | CBs | 7 | 11 | 14 |  |
| Binary Channel Bits (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | Bits | 57600 | 86400 | 115200 |  |
| For Sub-Frames 8,9 | Bits | 57600 | 86400 | 115200 |  |
| For Sub-Frame 5 | Bits | 53568 | 81216 | 110016 |  |
| For Sub-Frame 0 | Bits | 54912 | 83712 | 112512 |  |
| Number of layers |  | 2 | 2 | 2 |  |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 26.555 | 40.374 | 53.125 |  |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: As per Table 4.2-2 in TS 36.211 [15]. Note 4: Given per component carrier per codeword. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 6: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,3,4,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,3,4,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,3,4,8,9. |  |  |  |  |  |

Table A.3.2.2.6-4: PDSCH Reference Channel for sustained data-rate test (256QAM, 4 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.6-4.1 TDD | R.PDSCH.6-4.2 TDD | R.PDSCH.6-4.3 TDD |  |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |
| Allocated resource blocks |  | Note 7 | Note 8 | Note 9 |  |
| Uplink-Downlink Configuration (Note 3) |  | 2 | 2 | 2 |  |
| Number of HARQ Processes per component carrier |  | 10 | 10 | 10 |  |
| Allocated subframes per Radio Frame (D+S) |  | 6 | 6 | 6 |  |
| Modulation |  | 256QAM | 256QAM | 256QAM |  |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 |  | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 |  | 0.78 | 0.79 | 0.78 |  |
| For Sub-Frames 8,9 |  | 0.78 | 0.79 | 0.78 |  |
| For Sub-Frame 5 |  | 0.81 | 0.82 | 0.78 |  |
| For Sub-Frame 0 |  | 0.82 | 0.82 | 0.80 |  |
| Information Bit Payload (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | Bits | 84760 | 128496 | 169544 |  |
| For Sub-Frames 8,9 | Bits | 84760 | 128496 | 169544 |  |
| For Sub-Frame 5 | Bits | 81176 | 124464 | 161760 |  |
| For Sub-Frame 0 | Bits | 84760 | 128496 | 169544 |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | CBs | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | CBs | 14 | 21 | 28 |  |
| For Sub-Frames 8,9 | CBs | 14 | 21 | 28 |  |
| For Sub-Frame 5 | CBs | 14 | 21 | 27 |  |
| For Sub-Frame 0 | CBs | 14 | 21 | 28 |  |
| Binary Channel Bits (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | Bits | 108800 | 163200 | 217600 |  |
| For Sub-Frames 8,9 | Bits | 108800 | 163200 | 217600 |  |
| For Sub-Frame 5 | Bits | 101120 | 153344 | 207744 |  |
| For Sub-Frame 0 | Bits | 103808 | 158208 | 212608 |  |
| Number of layers |  | 4 | 4 | 4 |  |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 50.498 | 76.694 | 100.948 |  |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: As per Table 4.2-2 in TS 36.211 [15]. Note 4: Given per component carrier per codeword. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 6: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,3,4,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,3,4,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,3,4,8,9. |  |  |  |  |  |

Table A.3.2.2.6-5: PDSCH Reference Channel for sustained data-rate test (1024QAM, 2 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.6-5.1 TDD | R.PDSCH.6-5.2 TDD | R.PDSCH.6-5.3 TDD |  |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |
| Allocated resource blocks |  | Note 7 | Note 8 | Note 9 |  |
| Uplink-Downlink Configuration (Note 3) |  | 2 | 2 | 2 |  |
| Number of HARQ Processes per component carrier |  | 10 | 10 | 10 |  |
| Allocated subframes per Radio Frame (D+S) |  | 6 | 6 | 6 |  |
| Modulation |  | 1024QAM | 1024QAM | 1024QAM |  |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 |  | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 |  | 0.76 | 0.75 | 0.76 |  |
| For Sub-Frames 8,9 |  | 0.76 | 0.75 | 0.76 |  |
| For Sub-Frame 5 |  | 0.76 | 0.78 | 0.77 |  |
| For Sub-Frame 0 |  | 0.80 | 0.78 | 0.78 |  |
| Information Bit Payload (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | Bits | 55056 | 81176 | 110136 |  |
| For Sub-Frames 8,9 | Bits | 55056 | 81176 | 110136 |  |
| For Sub-Frame 5 | Bits | 51024 | 78704 | 105528 |  |
| For Sub-Frame 0 | Bits | 55056 | 81176 | 110136 |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | CBs | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | CBs | 9 | 14 | 18 |  |
| For Sub-Frames 8,9 | CBs | 9 | 14 | 18 |  |
| For Sub-Frame 5 | CBs | 9 | 13 | 18 |  |
| For Sub-Frame 0 | CBs | 9 | 14 | 18 |  |
| Binary Channel Bits (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | Bits | 72000 | 108000 | 144000 |  |
| For Sub-Frames 8,9 | Bits | 72000 | 108000 | 144000 |  |
| For Sub-Frame 5 | Bits | 66960 | 101520 | 137520 |  |
| For Sub-Frame 0 | Bits | 68640 | 104640 | 140640 |  |
| Number of layers |  | 2 | 2 | 2 |  |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 32.630 | 48.458 | 65.621 |  |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: As per Table 4.2-2 in TS 36.211 [15]. Note 4: Given per component carrier per codeword. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 6: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,3,4,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,3,4,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,3,4,8,9. |  |  |  |  |  |

Table A.3.2.2.6-6: PDSCH Reference Channel for sustained data-rate test (1024QAM, 4 MIMO layers)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.6-6.1 TDD | R.PDSCH.6-6.2 TDD | R.PDSCH.6-6.3 TDD |  |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |
| Allocated resource blocks |  | Note 7 | Note 8 | Note 9 |  |
| Uplink-Downlink Configuration (Note 3) |  | 2 | 2 | 2 |  |
| Number of HARQ Processes per component carrier |  | 10 | 10 | 10 |  |
| Allocated subframes per Radio Frame (D+S) |  | 6 | 6 | 6 |  |
| Modulation |  | 1024QAM | 1024QAM | 1024QAM |  |
| Coding Rate |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 |  | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 |  | 0.81 | 0.79 | 0.81 |  |
| For Sub-Frames 8,9 |  | 0.81 | 0.79 | 0.81 |  |
| For Sub-Frame 5 |  | 0.81 | 0.82 | 0.82 |  |
| For Sub-Frame 0 |  | 0.85 | 0.82 | 0.83 |  |
| Information Bit Payload (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | Bits | 110136 | 161760 | 220296 |  |
| For Sub-Frames 8,9 | Bits | 110136 | 161760 | 220296 |  |
| For Sub-Frame 5 | Bits | 101840 | 157432 | 211936 |  |
| For Sub-Frame 0 | Bits | 110136 | 161760 | 220296 |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | CBs | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | CBs | 18 | 27 | 36 |  |
| For Sub-Frames 8,9 | CBs | 18 | 27 | 36 |  |
| For Sub-Frame 5 | CBs | 17 | 26 | 35 |  |
| For Sub-Frame 0 | CBs | 18 | 27 | 36 |  |
| Binary Channel Bits (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,2,6,7 | Bits | N/A | N/A | N/A |  |
| For Sub-Frames 3,4 | Bits | 136000 | 204000 | 272000 |  |
| For Sub-Frames 8,9 | Bits | 136000 | 204000 | 272000 |  |
| For Sub-Frame 5 | Bits | 126400 | 191680 | 259680 |  |
| For Sub-Frame 0 | Bits | 129760 | 197760 | 265760 |  |
| Number of layers |  | 2 | 2 | 2 |  |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 65.252 | 96.623 | 131.342 |  |
| Note 1: 1 symbol allocated to PDCCH for all tests. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [15]. Note 3: As per Table 4.2-2 in TS 36.211 [15]. Note 4: Given per component carrier per codeword. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 6: Resource blocks nPRB = 0..2 are allocated for SIB transmissions in sub-frame 5 for all bandwidths. Note 7: Resource blocks nPRB = 3..49 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..49 in sub-frames 0,3,4,8,9. Note 8: Resource blocks nPRB = 4..74 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..74 in sub-frames 0,3,4,8,9. Note 9: Resource blocks nPRB = 4..99 are allocated for the user data in sub-frame 5, and resource blocks nPRB = 0..99 in sub-frames 0,3,4,8,9. |  |  |  |  |  |

END OF CHANGE