3GPP TSG-RAN WG4 Meeting #94                                                                         R4-2000386

# E-meeting, 24th February – 6th March, 2020

Agenda Item:	8.16.1.1

Source:	Intel Corporation

Title:	Discussion about CSI-RS L3 measurement bandwidth

Document for:	Discussion

# Introduction

In last meeting, there is WF [1] about CSI-RS L3 measurement, there are still some open issues:

- CSI-RS configuration for measurement requirement - CSI-RS based intra-frequency and inter-frequency measurement definition - MO configuration

In this contribution, we will provide our views regarding the configuration for measurement requirements.

# CSI-RS configuration for measurement requirement

In last meeting, CSI-RS configuration is still open:

- CSI-RS configurations (bandwidth of CSI-RS and density of CSI-RS resource) used to define requirements - Define requirements at least for 1 set of configurations - Option 1: 48PRBs and D = 1 - Option 2: 48PRBs and D = 3 - FFS whether to define requirements for additional configurations - 48PRBs and D = 1 - 48PRBs and D = 3 - 96PRBs and D = 1 - 96PRBs and D = 3

# Simulation results of measurement accuracy for FR1

In this stage, we provide our RSRP delta for FR1 for EPA and ETU channel for 48RB and 96 RB based on 3/5/10 samples respectively.

Table 1: RSRP delta for EPA5 channel (dB)

|  | 3 sample | 5 sample | 10 samples |
| --- | --- | --- | --- |
| 48RB, D=1 | 3.82 | 3.32 | 2.57 |
| 48RB, D=3 | 2.96 | 2.17 | 1.58 |
| 96RB, D=1 | 3.32 | 2.62 | 2.26 |
| 96RB, D=3 | 2.10 | 1.84 | 1.35 |

Table 2: RSRP delta for ETU30 channel(dB)

|  | 3 sample | 5 sample | 10 samples |
| --- | --- | --- | --- |
| 48RB, D=1 | 3.94 | 3.11 | 2.6 |
| 48RB, D=3 | 2.87 | 2.02 | 1.6 |
| 96RB, D=1 | 3.44 | 2.65 | 2.23 |
| 96RB, D=3 | 2.15 | 1.76 | 1.37 |

To define the CSI-RS based measurement performance accuracy test, the CSI-RS configuration may need to be chosen that CSI-RS based measurement performance will not be worse than that of SSB based measurement. Since for SSB, there are 127 REs per symbol. If 48PRB is assumed for CSI-RS configuration, D=3 shall be needed to achieve similar performance.  If 48PRB with D=1 is chosen, longer evaluation time may be expected. It is observed that when 10 samples are used in case of 48RB with D=1, the performance is still 0.5dB worse than the case where 48PRB with D=3 and 5 samples are used. Comparing the performance of 48RB with D=3 and 96RB with D=3, less than 0.5dB gain can be obtained. Therefore, there is no need to define other configurations.

Proposal 1: Define requirement for one configuration with 48PRBs and D = 3.

# Conclusion

In this contribution, we provide our simulation results for L1-RSRP measurement accuracy, the following conclusion can be drawn:

Proposal 1: Define requirement for one configuration with 48PRBs and D = 3.

# References

- R4-1913749, “WF on CSI-RS L3 RRM requirements”, CATT