3GPP TSG-RAN WG4 Meeting #94-e	R4-2000394

# Online, Feb 24th – Mar 6th, 2020

Agenda item:	8.14.1.2

Source: 	Intel Corporation

Title: 	SSB based Beam Correspondence

Document for:	Approval

# Introduction

In Rel-15, Beam correspondence requirements were based on SSB + CSI-RS, where CSI-RS is configured for beam management purpose with repetition on within a slot. While in Rel-16, a new WID introduced SSB based beam correspondence and CSI-RS based on beam correspondence to accommodate the fact that network may not configure CSI-RS (with repetition on) and/or desired spatial relationship between SSB and CSI-RS if both are present. While CSI-RS based beam correspondence received less concern except some details in parameter settings, SSB based beam correspondence is still under debating in term of feasibility and optionality. In this contribution, we present our view on SSB based beam correspondence.

# Discussion

SSB based beam correspondence

- By RAN4 #94, RAN4 could specify BC based on SSB only unless RAN4 find critical technical problems, considering:

  - Side condition feasibility

  - Common understanding of UE behavior

  - Performance difference with SSB + CSI-RS based BC

- After stabilizing the configuration and requirement structure and confirming the feasibility, the optionality of this requirement can be discussed

| Parameter | Value |
| --- | --- |
| SSB periodicity | 20 ms [1] |
| Use P1 CSI-RS configuration? | No |
| Use P3 CSI-RS configuration? | Alt. 1: Yes Alt. 2: No |
| Use tracking CSI-RS? | Yes |
| Tracking CSI-RS QCL info | qcl-TypeD=SSB |
| Tracking CSI-RS min SNR | TBD |
| PDCCH/PDSCH DM-RS QCL info | qcl-TypeD=TRS |
| SSB min SNR level | Alt. 1: 6 dB [1] [10] Alt. 2: 13 dB [14] |
| (note) | RAN4 didn’t assume more than [1] SSB indices should be transmitted SSB use configuration for Rel-15 which is specified in 38.508 per agreed in RAN4 #92bis meeting |

SSB based beam correspondence and CSI-RS based beam correspondence have been discussed for a few meetings for Rel-16 beam correspondence. Companies have shown different views on SSB based beam correspondence. A WF captured the latest status on SSB based on beam correspondence [1] as shown below.

The main discussion points are

- Side condition feasibility

- Common understanding of UE behavior

- Performance difference with SSB + CSI-RS based BC

To evaluate the side condition feasibility and common understanding of UE behavior, physical layer procedure (UE behaviors) defined in 38.214 must be reviewed.

5.1.6.1.1	CSI-RS for tracking

A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

In 38.214, the CSI-RS reference signals can be only configured after UE entering connected mode since CSI-RS is a UE specific reference signal. Although it is mentioned for tracking CSI-RS, it applies to all types of CSI-RS.

It can be understood that the tracking CSI-RS in side condition listed in above table can only be configured when UE is in RRC connected mode.

Observation 1: CSI-RS is a UE specific reference signal and can only be configured when UE is in RRC connected mode.

It is our understanding that the side condition is feasible once UE is in RRC connected mode where UE can fine tune ‘narrow’ beams based on SSB and tracking CSI-RS.

Observation 2: When UE is in RRC connected mode, side condition of SSB + tracking CSI-RS can be used for UE to fine tune ‘narrow beam’ for beam correspondence.

With only SSB signals available at the initial access stage, a UE has to rely on only SSB signals for beam refinement. Since some UEs may only use ‘fat’ beam for initial access, the question is if the UEs can manage to use ‘fat’ beams to successfully finish the initial access and enter RRC connected mode. Of course, this question is of more interests when the UE is at cell edge.

When comparing beam forming gain between ‘narrow’ and ‘fat’ beams, we limit gain difference within 3dB.

During initial access, a UE performs the following steps to establish the network connection:

- Search SSS and PSS to establish the synchronization with network (DL)

- Decode PBCH to acquire MIB (DL)

- Based on MIB info, perform SIB1 decoding (DL, PDCCH and PDSCH corresponding CORESET0)

- RACH procedure (UL/DL)

For UEs at cell edge, during the initial access, comparing with ‘narrow’ beams used for control and data transmisisons in RRC connected mode, we thinik ‘fat’ beams don’t necessarily degrade cell coverage.

First, a UE can detect PSS and SSS with ‘fat’ beams in low SNR since miss detection rate becomes low with multi-shot detection on repeated SSB bursts.

5.2 System information

5.2.1 Introduction

System Information (SI) is divided into the MIB and a number of SIBs where:

- the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB;

- the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

With 20ms SSB periodicity in the side condition, MIB and SIB1 can be decoded with soft-combining of multiple repetitions. The repetitions of MIB and SIB1 are 4 times and 8 times within 80ms MIB periodicity and 160ms SIB1 periodicity respectively. See system information description captured here from 38.331

Acquiring MIB and SIB1 does not need a UE to have ‘narrow’ beams at cell edge since MIB and SIB1 transmission are designed to be more robust by repetitions and combining gain is at least 3dB or more.

After acquiring system information, a UE starts RACH procedure to request RRC connection with network. Msg 1 (UL), the RACH preamble waveform, depending on number of OFDM symbols network configured, can be very robust with repetition gain. Msg 2 (DL), Random Access Response (RAR), can also be scheduled with even lower coding rate by parameter ‘TB scaling’ in DCI format 1_0 scrambled by RA-RNTI to achieve more robust reception at UE than other PDSCHs with same MCS at least by 3dB. Msg 3 (UL) supports HARQ retransmissions scheduled by DCI format 0_0 scrambled by a TC-RNTI if first Msg 3 transmission scheduled by RAR fails. Msg 4 (DL) also supports HARQ retransmissions. Both HARQ retransmissions in msg3 and msg4 can compensate ‘fat’ beam gain loss of 3dB.

Due to robust design in initial access, a UE is able to use ‘fat’ beam to acquire synchronization, system information and also to establish RRC connection with network based on our analysis. In a UE’s operation horizontal, the impact of a few retransmissions, if any, during initial access can be negligible.

Observation 3: A UE is able to finish initial access and establish RRC connection with network without sacrificing the cell coverage with ‘fat’ beams

But if a UE is in RRC connected mode, low gain of UE’s ‘fat’ beam can degrade throughput for both UL and DL. A UE has to form ‘narrow’ beam in RRC connected mode.

With tracking CSI-RS configured in side condition, a UE not supporting ‘narrow’ beam refinements with reference signal of SSB only can have opportunities to do ‘narrow’ beam refinements. For such UEs, configuring tracking CSI-RS is helpful as listed in the parameter table since the UEs can still do ‘narrow’ beam refinements based on tracking CSI-RS. But we think P3 CSI-RS is not needed. For tracking CSI-RS, the configuration needs to be set according to 38.214 with highlighted option.

5.1.6.1.1	CSI-RS for tracking

…

For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same. For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. For frequency range 2 the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

…

A UE does not expect to be configured with a NZP-CSI-RS-ResourceSet configured both with trs-Info and repetition.

Tracking CSI-RS cannot be configured with parameter repetition=on. But it can be configured with multiple NZP CSI-RS sets with each set configured with 4 CSI-RS periodic resources in two consecutive slots (which are the same across two consecutive slots).

The exact configuration for tracking CSI-RS is reflected in the updated parameter table below.

Proposal 1: Updated parameter table for SSB based beam correspondence should be as table 1

Table 1 Parameter configuration for SSB based beam correspondence

| Parameter | Value |
| --- | --- |
| SSB periodicity | 20 ms [1] |
| Use P1 CSI-RS configuration? | No |
| Use P3 CSI-RS configuration? | Alt. 1: Yes Alt. 2: No |
| Use tracking CSI-RS? | Yes |
| Tracking CSI-RS QCL info | qcl-TypeD=SSB |
| Tracking CSI-RS min SNR | TBD 6dB |
| Tracking CSI-RS resource sets | 2 NZP CSI-RS resource sets, set0 and set1; each resource set has 4 periodic resources over two consecutive slots |
| Tracking CSI-RS resource periodicity | 20 ms |
| Tracking CSI-RS resource time domain location and slot offsets | Resource mapping: Set0: l∈{0,4}, set1: l∈{0,4}; Slot offset: set0 = 2µ10; set1 = 2µ10 +2; |
| Tracking CSI-RS resource frequency domain configuration | ρ = 3; 48RBs for BW=100/200/400MHz, 32RBs for BW=50MHz |
| PDCCH/PDSCH DM-RS QCL info | qcl-TypeD=TRS |
| SSB min SNR level | Alt. 1: 6 dB [1] [10] Alt. 2: 13 dB [14] |
| (note) | RAN4 didn’t assume more than [1] SSB indices should be transmitted SSB use configuration for Rel-15 which is specified in 38.508 per agreed in RAN4 #92bis meeting |

The relationship of configured SSB and tracking CSI-RS is shown in the following figure.

Figure 1 SSB and tracking CSI-RS time domain arrangement

Although CSI-RS is UE specific signal but can be shared by the multiple UEs.

Another alternative way is UE purely relies on SSB signals without any CSI-RS to do ‘narrow’ beam refinement. But we believe SSB periodicity needs to be properly selected in order to prevent network performance degradation in practical mobility scenarios. This can be concluded from the following discussion.

Assume a UE has identified the best ‘narrow’ beam at the moment, now it has to track the best Rx beam by scanning the surrounding beams, normally 4-8 candidate beams will be scanned around the current Rx beam depending on UE codebook design. Let’s conservatively assume 8 candidate beams will be scanned. Within an SSB block, there are 4 symbols theoretically that can be used for beam measurements. Let’s assume only 3 symbols can be used. Then a UE has to perform Rx beam scanning in next 3 consecutive SSB blocks. Within the duration of 3 SSB blocks, it is assumed that the best Rx beam changes slowly at most to one of its adjacent beams during the scanning. Otherwise, the new best Rx beam can’t be guaranteed to be identified and beam failure can happen quite often.

SSB periodicity can be any value in the set of [5, 10, 20, 40, 80, 160] ms configured by gNB defined in RAN1 spec. With SSB periodicity = 160ms, a time duration of 3 consecutive SSB blocks is around 0.5 seconds. Within 0.5 seconds, a UE may experience fast incoming AoA changes and/or receive power fluctuations.

The assumptions about the UE identifying the best Rx beam within a beam scanning cycle are 1) the incoming DL AoA (relative to UE) should change slowly, at most from the previous best Rx beam to one of its adjacent beams; 2) the power level of DL RS signals should keep constant during scanning period so that the received RSRP can be used to identify best Rx beam by comparing RSRP values. These assumptions barely hold true when SSB periodicity goes large in a mobility environment. For example, an armband holding a UE is attached to a user’s upper arm when the user is running for workout, the arm’s swing back and forth can change incoming AoA (relative to UE) quite rapidly, the trace drawn by the best Rx beams’ movement can have multiple hops across Rx beams; another example is a moving vehicle with speed of 40km/hr ( speed limit in most US cities) in the urban street can experience large fluctuation in RSRP measurements within 0.5 seconds ( travel distance  = 5.6m ) since channel coherent time due to vehicle’ moving is much smaller, like blockage coming and/or going, surrounding reflection changing, etc. The speed limit can be even higher when highway is across the city.

By considering the real deployments, large SSB periodicity is not expected to be configured in the network. Otherwise, beam tracking may be broken quite often. To prevent this from happening, a UE may use ‘fat’ beams to reduce above detrimental impacts from mobility since beam tracking is more robust with ‘fat’ beam codebook even beam gain is reduced. When duration of 3 SSB bursts is reduced, ‘narrow’ beam refinements become possible in practical sense. We see 20ms of SSB periodicity is a reasonable threshold.

Proposal 2: When SSB periodicity <=20ms, a UE should meet Rel15 beam correspondence requirements without CSI-RS assistance under the condition 1) SNR = 6dB, 2) For each test grid point, at least 3 SSB bursts should be provided for beam refinements.

In above analysis of tracking CSI-RS, two sets of NZP tracking CSI-RS resources are configured for a UE to do ‘narrow’ beam refinements with each set having four CSI-RS symbols, a total of 8 CSI-RS symbols are confined within 4 consecutive slots for UE to finish a complete Rx beam scanning. At most, 4 slots take 1 ms (when SCS=60KHz, the smallest SCS in FR2). Within 1 ms, the channel can be considered as constant and assumptions are kept. This is also true for CSI-RS based beam correspondence where P3 CSI-RS with repetition = on (the Rx beam scanning is within one slot only, 0.25ms for SCS=60KHz).

# Conclusion

Since CSI-RS is a UE specific reference signal, during initial access, only SSB can be used for beam refinement.

Observation 1: CSI-RS is a UE specific reference signal and can only be configured when UE is in RRC connected mode.

Observation 2: When UE is in RRC connected mode, side condition of SSB + tracking CSI-RS can be used for UE to fine tune ‘narrow beam’ for beam correspondence.

Even with SSB only, during initial access, a UE can establish the RRC connection by using ‘fat’ beams

Observation 3: A UE is able to finish initial access and establish RRC connection with network without sacrificing the cell coverage with ‘fat’ beams

With tracking CSI-RS configured,

Proposal 1: Updated parameter table for SSB based beam correspondence should be as table 1

Without CSI-RS configured, we also think it is possible to maintain Rel-15 beam correspondence with explicit restriction on SSB periodicity

Proposal 2: When SSB periodicity <=20ms, a UE should meet Rel15 beam correspondence requirements without CSI-RS assistance under the condition 1) SNR = 6dB, 2) For each test grid point, at least 3 SSB bursts should be provided for beam refinements.

# References

[1] R4-1916172, “WF on side conditions of SSB based and CSI-RS based beam correspondence tests”, Apple, RAN4#93bis