3GPP TSG-RAN WG4 Meeting #94-e	R4-2000406

Electronic meeting, 24th February – 6th March 2020

Source:	Ericsson

Title:	Simulation results for Rel-16 NR PUSCH UL timing adjustment

Agenda item:	8.17.2.2.3

Document for:	Discussion

# Introduction

In this contribution, we presented our simulation results for NR HST PUSCH UL timing adjustment.

# Simulation assumptions and simulation results

In this section, our presented our simulation assumptions based on [1] in Table 1 for UL timing adjustment. The corresponding ideal simulation results and results with impairments are given in Table 2 and Table 3 respectively.

| Parameters | Values |
| --- | --- |
|  | FR1 – Scenario Y |
| Channel model | Stationary UE: AWGN Moving UE: AWGN |
| Number of Tx and Rx | 1Tx 2Rx |
| Number of DMRS symbols | (1+1+1) DM-RS symbols with l0 = 2 for PUSCH mapping type A, and l0 = 0 for PUSCH mapping type B |
| (A, Δω) | SCS = 15 kHz: (10 µs, 0.13 s-1) SCS = 30 kHz: (5 µs, 0.26 s-1) |
| System bandwidth | SCS = 15 kHz: 10MHz SCS = 30 kHz: 40 MHz |
| Frequency domain resource | SCS = 15 kHz: 25 PRBs for each UE SCS = 30 kHz: 50 PRBs for each UE |
| MCS index | 16 |
| SRS (optional) | SCS = 15 kHz: single region with length of 40 PRBs SCS = 30 kHz: single region with length of 80 PRBs |
| Testing metric | SNR @70% of maximum throughput |

Table 1. Simulation assumptions for UL timing adjustment at 350 km/h, scenario Y [1].

Table 5. Ideal simulation results for NR UL timing adjustment at MCS = 16.

| Scenario | SCS / BW | PUSCH mapping type | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Scenario Y | 15 kHz / 10 MHz | A | 6.0 |
|  |  | B | 6.0 |
|  | 30 kHz / 40 MHz | A | 6.0 |
|  |  | B | 6.0 |

Table 6. Results with impairments for NR UL timing adjustment at MCS = 16.

| Scenario | SCS / BW | PUSCH mapping type | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Scenario Y | 15 kHz / 10 MHz | A | 8.3 |
|  |  | B | 8.2 |
|  | 30 kHz / 40 MHz | A | 8.3 |
|  |  | B | 8.1 |

Observation 1: Under scenario Y, comparable performance can be achieved with both PUSCH mapping type A and mapping type B.

Observation 2: It is unclear why PUSCH mapping type B was introduced for UL timing adjustment under HST condition while PUSCH mapping type B has not be considered for HST PUSCH demodulation performance. With PUSCH mapping type B, (1+1+1) DM-RS symbols and 14 OFDM symbols may be suitable for UL timing adjustment where UE is travelling at 500 km/h. However, it should be noted that with the same PUSCH mapping type B configuration, it would not be able to achieve satisfactory demodulation performance with a basic receiver, as the maximum unambiguous sampling frequency = 1400 Hz and 2800 Hz for SCS = 15 kHz and SCS = 30 kHz, respectively.

Proposal 1: Remove PUSCH mapping type B assumptions and requirements for UL timing adjustment to align them to PUSCH HST demodulation requirements.

# Conclusion

In this contribution, we summarized our simulation results for NR UL timing adjustment for PUSCH under scenario Y channel condition. Below, we also summarized our observations:

Observation 1: Under scenario Y, comparable performance can be achieved with both PUSCH mapping type A and mapping type B.

Observation 2: It is unclear why PUSCH mapping type B was introduced for UL timing adjustment under HST condition while PUSCH mapping type B has not be considered for HST PUSCH demodulation performance. With PUSCH mapping type B, (1+1+1) DM-RS symbols and 14 OFDM symbols may be suitable for UL timing adjustment where UE is travelling at 500 km/h. However, it should be noted that with the same PUSCH mapping type B configuration, it would not be able to achieve satisfactory demodulation performance with a basic receiver, as the maximum unambiguous sampling frequency = 1400 Hz and 2800 Hz for SCS = 15 kHz and SCS = 30 kHz, respectively.

Proposal 1: Remove PUSCH mapping type B assumptions and requirements for UL timing adjustment to align them to PUSCH HST demodulation requirements.

# References

R4-1915870, Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon.