3GPP TSG-RAN WG4 Meeting #94-e                 	 R4-2000460

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.15.1.5

Source:	MediaTek Inc.

Title:	Inter-frequency measurement requirement without gap

Document for:		Discussion

# Introduction

In last meeting, a WF [1] was agreed to capture the conclusions of the discussion as well as the open issues to be further addressed in this meeting. In short the open issues are

- Issue #1: UE behaviour when configured MG is partially overlapped with inter-frequency SMTC and inter-frequency SSB is completely contained in the active BWP of the UE - Issue #2: Synchronization assumption for scheduling restriction when the inter-frequency layer is within the same TDD band as UE’s serving cell(s). - Issue #3: UE capability

In this paper, we provide our views on above issues and we also bring one new issues for scheduling restriction.

- Issue #2A: SCS grid alignment for scheduling restriction

# Discussion

Issue #1: UE behaviour for partially overlapped case

UE can perform inter-frequency measurement without MG if the SSB is completely contained in the DL active BWP. Therefore, the scenario is very clear from the frequency domain. To introduce the requirement in to R15 framework, we also need to consider the time domain.

In Figure 1, we show how different scenarios are mapped to different requirements. In Figure 1(b), if the SMTC occasions of this target MO are fully-overlapped by measurement gap, then UE still needs to conduct the measurement within gap. Then the requirements remain the same as the R15 baseline. If the SMTC occasions of this target MO are fully non-overlapped or partially overlapped by gap, then the measurements are conducted outside gap. We also plotted the scenarios for legacy UEs for comparison in Figure 1(a).

Figure 1. How to capture the requirements in different scenarios for different UEs

One key point is on how we handle the case when SMTC occasions are partially overlapped by MG. Theoretically, UE can perform measurement for this MO both outside gap and within gap. However, allowing this UE behavior in the requirements could make the spec very complicated, because it is out of the current framework of CSSF. To simplify the spec work, we prefer to make it always measured outside gap. So that the requirement structure for inter-frequency measurement becomes similar to that of intra-frequency measurement. This can help reduce the effort in spec and reduce UE’s implementation complexity at the same time. Note that UE actually still has the flexibility to conduct measurement within gap, if UE is able to fulfil the requirements of all MOs.

Proposal 1: For inter-frequency measurement without gap, if the SMTC occasions of an MO are partially overlapped by MG, UE is only required to conduct measurement for this MO outside measurement gap. Therefore, the factor Kp in intra-frequency measurement without gap should be re-used in this scenario.

Issue #2: Scheduling and measurement restriction

The measurement behavior of this inter-frequency without gap is somehow like intra frequency measurement without gap in which the scheduling restriction needs to be addressed. The scheduling restriction was introduced due to 3 issues.

- Collision with UL transmission and DL measurement on TDD carrier

- Mix-numerology between data and SSB symbols

- The need of Rx beam sweeping in FR2

To our understanding, the baseline UE behavior is to allow scheduling restriction for all OFDM symbols within the SMTC of this inter-frequency MO without gap, because there was no any synchronization relation introduced between an inter-frequency layer and UE’s serving cell.

One discussion point raised from last meetings is for the case when the inter-frequency layer is on the same TDD band as UE’s serving cell(s). From the view point of network vendors or operators it seems straightforward that carriers within the same TDD band are well synchronized. Therefore, the OFDM symbols with scheduling restriction can be reduced from the entire SMTC duration to only those SSB symbols and those OFDM symbols adjacent to SSB. However, from UE perspective, this synchronization relation is not so clear. For an example, UE may not know whether the serving cell and the target inter-frequency belong to the same infra vendor or different infra vendors. It is highly concerned from a UE vendor’s perspective if UE always assume synchronization among carriers on the same band. As a results, it would be beneficial to provide an explicit network signaling to tell UE if the synchronization between an inter-frequency MO and one UE’s serving cell can be assumed. To our understanding, providing this signaling should not be a difficult task to network.

Once the signaling is provided, UE can assume the synchronization between the target inter-frequency layer and one of UE’s serving cell. Then, the requirement for scheduling restriction can follow what we have for the intra-frequency measurement without gap.

Proposal 2: An explicit signaling is introduced to tell UE whether the synchronization between an inter-frequency MO and one UE’s serving cell can be assumed. Send an LS to RAN2 to request the corresponding signaling.

Proposal 3: For the scheduling restriction requirements for inter-frequency measurement without gap, if the synchronization signaling is provided, the scheduling restriction is applied only to those SSB symbols and OFDM symbols adjacent to SSB. Otherwise, the scheduling restriction is applied to the entire SMTC duration.

Issue #2A: SCS grid alignment

The SCS applicable for SSB are 15KHz, 30KHz, 120KHz and 240KHz, while the granularity of ARFCN that are used in MO to indicate the frequency location of SSB can be 5KHz.

In TS38.331 MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected … In TS38.101-1

That means the spec allows the indication of an inter-frequency MO with SSB not on the same SCS grid as UE’s serving cell, even if they are with the same SCS. Figure 2 provides one example with 15KHz SCS for both UE’s serving cell and the target SSB but on different grids. In this case, additional effort is required by UE to perform measurement and data reception at the same time.

Figure 2. Unaligned grid between target SSB and UE’s serving cell

Introducing a UE capability to indicate this additional capability to network could be one solution. However, we believe this scenario can be easily avoided by network deployment. So adding new UE capability does not make much sense. An easiest way to address this issue is to add a condition in the scheduling restriction requirements, e.g., when the target SSB has a different SCS grid as that of UE’s serving cell, UE is allowed to have scheduling restriction in the entire SMTC duration.

Proposal 4: When the target SSB has a different SCS grid as that of UE’s serving cell, UE is allowed to have scheduling restriction in the entire SMTC duration.

Issue #3: UE capability

Since this is a new UE behavior which was not introduced in Rel-15 and this feature is not an essential functionality for the NR system to work, it is very straightforward to introduce a UE capability for this feature.

Proposal 5: UE capability is needed for the feature of inter-frequency measurement without gap.

If UE capability is introduced, it is very important to make it clear in the spec that which set of requirements are applicable to legacy UEs and which set of requirements are applicable to new UEs. In our view, Section 9.1.5.1 of TS38.133 needs to be updated like the following.

#### 9.1.5.1 Monitoring of multiple layers outside gaps The carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types: - Intra-frequency measurement with no measurement gap in clause 9.2.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap. - Intra-frequency measurement with no measurement gap in clause 9.2.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap. - Inter-frequency measurement with no measurement gap in clause TBD, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports inter-frequency measurement without MG. - Inter-frequency measurement with no measurement gap in clause TBD, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports inter-frequency measurement without MG. UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps

Section 9.1.5.2 also needs to be updated correspondingly, e.g.,

#### 9.1.5.2 Monitoring of multiple layers within gaps The carrier-specific scaling factor CSSFwithin_gap,i for measurement object i derived in this chapter is applied to following measurement types: - Intra-frequency measurement with no measurement gap in clause 9.2.5, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap. - Intra-frequency measurement with measurement gap in clause 9.2.6. - Inter-frequency measurement in clause 9.3, if UE does not support inter-frequency measurement without MG - Inter-frequency measurement with no measurement gap in clause TBD, when all of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports inter-frequency measurement without MG - Inter-RAT measurement in clause 9.4 UE is expected to conduct the measurement of this measurement object i only within the measurement gaps.

Regarding the requirements it is a more complicated issue is how to add these new requirements into current spec. Note that we not only need to introduce new requirements, but also need to update the existing requirements, according to Figure 1. At the same time, we need to be careful about how we handle UEs supporting this feature and UEs not supporting this feature. For an example, different requirements should be applied for different UEs when the SMTC occasions of an MO are partially overlapped by MG.

In our view, the most clear way is to separate different UE capabilities by sections, e.g., reusing existing requirements (Section 9.3.4 and 9.3.5) for UEs not supporting this feature, while creating new sections for UEs supporting this feature, e.g.,

9.3 NR inter-frequency measurements 9.3.2 Requirements applicability 9.3.3 Number of cells and number of SSB 9.3.4 Inter frequency cell identification for UE which does not support FEATURE_NAME 9.3.5 Inter frequency measurements for UE which does not support FEATURE_NAME 9.3.6 NR Inter frequency measurements reporting requirements 9.3.7 Void Inter frequency cell identification for UE which supports FEATURE_NAME 9.3.7.1 Requirements with measurement gap (refer to 9.3.4) 9.3.7.2 Requirements without measurement gap (New requirement) 9.3.8 Inter frequency cell identification for UE which supports FEATURE_NAME 9.3.8.1 Requirements with measurement gap (refer to 9.3.5) 9.3.8.2 Requirements without measurement gap (New requirement) 9.3.9 Scheduling availability for UE which supports FEATURE_NAME (New requirement)

In this way, we can avoid the confusion between UEs who supports and who does not support this feature. We also believe that with this structure it is easier for future maintenance when we see some error in Rel-15 and have to revise both Rel-15 and Rel-16 specs.

Proposal 6: New sections for delay requirements are introduced for UE who supports inter-frequency measurement without gap. Within each new section, add sub sections for requirements with gap and without gap.

One issue raised which was not resolved in last meeting is about UEs without CA capability. For such UEs, they are not required to have 2 searchers according to R15 baseline. One possible way to address this issue is to allow some extension in the measurement delay requirements of PCell. So that UE can spend some time to perform inter-frequency measurement without gap. As an example shown in Table 1, the measurement delay of PCC is extended twice.

Table 1 CSSFoutside_gap,i scaling factor for UE without CA or DC capability

| Scenario | CSSFoutside_gap,i for PCC | CSSFoutside_gap,i for inter-frequency measurement without MG |
| --- | --- | --- |
| Single CC | 2 | 2 x Y |

Proposal 7: For UE without CA capability, the number of search assumed in the requirement is 1. The measurement delay of PCC is doubled to allow UE to conduct inter-frequency measurement without gap.

# Summary

In this paper, we discuss the requirement of inter-frequency measurement without gap. We have the following proposals.

Proposal 1: For inter-frequency measurement without gap, if the SMTC occasions of an MO are partially overlapped by MG, UE is only required to conduct measurement for this MO outside measurement gap. Therefore, the factor Kp in intra-frequency measurement without gap should be re-used in this scenario.

Proposal 2: An explicit signaling is introduced to tell UE whether the synchronization between an inter-frequency MO and one UE’s serving cell can be assumed. Send an LS to RAN2 to request the corresponding signaling.

Proposal 3: For the scheduling restriction requirements for inter-frequency measurement without gap, if the synchronization signaling is provided, the scheduling restriction is applied only to those SSB symbols and OFDM symbols adjacent to SSB. Otherwise, the scheduling restriction is applied to the entire SMTC duration.

Proposal 4: When the target SSB has a different SCS grid as that of UE’s serving cell, UE is allowed to have scheduling restriction in the entire SMTC duration.

Proposal 5: UE capability is needed for the feature of inter-frequency measurement without gap.

Proposal 6: New sections for delay requirements are introduced for UE who supports inter-frequency measurement without gap. Within each new section, add sub sections for requirements with gap and without gap.

Proposal 7: For UE without CA capability, the number of search assumed in the requirement is 1. The measurement delay of PCC is doubled to allow UE to conduct inter-frequency measurement without gap.

# Reference

[1] R4-1915853, “WF on inter-frequency without MG”, CMCC