3GPP TSG-RAN WG4 Meeting #94-e                 	 R4-2000462

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.16.1.1

Source:	MediaTek Inc.

Title:	Simulation results for CSI-RS measurement BW

Document for:		Discussion

# Introduction

This paper is a re-submission of [3] with minor modifications (modifications are marked in blue). In last meeting, a WF [1] was agreed with some progress on the measurement bandwidth of CSI-RS.

CSI-RS configurations (bandwidth of CSI-RS and density of CSI-RS resource) used to define requirements - Define requirements at least for 1 set of configurations - Option 1: 48PRBs and D = 1 - Option 2: 48PRBs and D = 3 - FFS whether to define requirements for additional configurations - 48PRBs and D = 1 - 48PRBs and D = 3 - 96PRBs and D = 1 - 96PRBs and D = 3

In this paper, we express our view on this issue.

# Discussion

Regarding the simulation assumption, we follow the agreed documents [2]. The results are provided in Table 1 based on AWGN at SINR = -6dB with 15KHz SCS. In the discussion of SSB based requirements, the accuracy contributed by baseband is roughly 2dB. Therefore, we also highlighted those configurations with baseband accuracy < 2dB in Table 1.

Table 1. RSRP accuracy for AWGN at SINR = -6dB with 15KHz SCS

| RB # | Density | Sample # | 5% | 50% | 95% | 90% abs |
| --- | --- | --- | --- | --- | --- | --- |
| 24 | 3 | 1 | -2.45 | 2.01 | 4.36 | 4.11 |
|  |  | 3 | -0.99 | 1.51 | 3.08 | 2.82 |
|  |  | 5 | -0.51 | 1.32 | 2.64 | 2.38 |
|  |  | 10 | -0.2 | 1.11 | 2.16 | 1.93 |
| 48 | 1 | 1 | -2.51 | 2.43 | 4.96 | 4.69 |
|  |  | 3 | -0.82 | 1.85 | 3.67 | 3.33 |
|  |  | 5 | -0.35 | 1.63 | 3.16 | 2.85 |
|  |  | 10 | 0.03 | 1.41 | 2.59 | 2.34 |
|  | 3 | 1 | -1.92 | 1.42 | 3.45 | 3.22 |
|  |  | 3 | -0.81 | 1 | 2.37 | 2.12 |
|  |  | 5 | -0.55 | 0.85 | 2 | 1.78 |
|  |  | 10 | -0.3 | 0.72 | 1.57 | 1.38 |
| 96 | 1 | 1 | -2.21 | 1.75 | 3.97 | 3.72 |
|  |  | 3 | -0.89 | 1.25 | 2.81 | 2.53 |
|  |  | 5 | -0.53 | 1.07 | 2.38 | 2.12 |
|  |  | 10 | -0.24 | 0.91 | 1.88 | 1.67 |
|  | 3 | 1 | -1.41 | 1.01 | 2.66 | 2.43 |
|  |  | 3 | -0.69 | 0.67 | 1.77 | 1.56 |
|  |  | 5 | -0.5 | 0.55 | 1.47 | 1.28 |
|  |  | 10 | -0.3 | 0.45 | 1.12 | 0.97 |
| 192 | 1 | 1 | -1.7 | 1.24 | 3.11 | 2.89 |
|  |  | 3 | -0.73 | 0.85 | 2.11 | 1.88 |
|  |  | 5 | -0.51 | 0.71 | 1.78 | 1.55 |
|  |  | 10 | -0.3 | 0.58 | 1.36 | 1.19 |
|  | 3 | 1 | -1.03 | 0.73 | 2.05 | 1.83 |
|  |  | 3 | -0.5 | 0.46 | 1.31 | 1.15 |
|  |  | 5 | -0.38 | 0.38 | 1.07 | 0.92 |
|  |  | 10 | -0.23 | 0.3 | 0.78 | 0.68 |

The decision on the measurement BW has the big impact to both the measurement accuracy, measurement delay and UE complexity. To simplify the spec, we suggest that requirements is defined based on only one single measurement bandwidth. Another reason is that BWP can be dynamically changed in NR. Therefore, UE’s active BWP BW could be changed from time to time. BW-specific requirement adds additional complexity to UE which needs to change its scheduling based on the BW of active BWP. This is some unnecessary complexity. This is different to what we have in LTE, because LTE channel BW never changes.

Proposal 1: Requirement is defined based on single measurement bandwidth (N=1).

Regarding the size of the measurement BW, a common concern from UE vendor is that the larger BW demands higher UE complexity as well as higher power consumption. Although from the simulation results we can see that larger measurement BW brings more accurate measurement results, but it also introduces larger RF path loss calibration error which cannot be seen from the simulation result.

Observation 1: Larger measurement BW does not always lead to better measurement accuracy requirement because the RF path loss calibration error also increases.

For RLM, BFD, CBD and L1-RSRP, the requirements were specified based on 48 PRBs with density 3. We think it could provide UE a chance to re-use the measurement engines designed for L1 measurement for CSI-RS based L3 measurement. This is good to this feature because UE is able to support it through only software upgrade without hardware impact. After checking Table 1, the accuracy of 48 PRBs with density 3 is also within 2dB. Therefore, we think the requirements of CSI-RS based L3 measurement can be specified based on 48 PRBs and density 3.

Proposal 2: For CSI-RS based L3 measurement, the requirements are specified based on 48 PRBs and density 3.

# Summary

In this paper, we provide the simulation results of RSRP accuracy for CSI-RS based L3 measurement. We have the following observations and proposals:

Observation 1: Larger measurement BW does not always lead to better measurement accuracy requirement because the RF path loss calibration error also increases.

Proposal 1: Requirement is defined based on single measurement bandwidth (N=1).

Proposal 2: For CSI-RS based L3 measurement, the requirements are specified based on 48 PRBs and density 3.

# Reference

[1] R4-1913749, “WF on CSI-RS based RRM requirements”, CATT

[2] R4-1910949, “Link-level simulation assumptions for CSI-RS based measurements”, CATT

[3] R4-1913588, “Simulation results for CSI-RS measurement BW”, MediaTek