3GPP TSG-RAN4  #94-e								R4-2000505

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Source:	Qualcomm Incorporated

Title:	Study feasible SNR ranges for NR FR2 MIMO OTA in 3D MPAC

Agenda item: 	10.2.3.2

Document for:	Approval

# 1	Introduction

In RAN4#93 meeting, the WF on FR2 MIMO OTA was agreed in [1]. To finalize the FR2 MIMO OTA static testing, the following actions were made:

  - Channel model and Quality of Quiet Zone validation procedures:

    - TPs to finalize FR2 channel model validation procedure and quiet zone validation procedure

  - 3D MPAC calibration and test procedures:

    - TPs to finalize calibration and test procedures

  - MU assessment

    - Define key elements and descriptions

  - Study feasible SNR ranges for 3D MPAC

    - Provide FR2 SNR analysis based on whole 3D-MPAC system

  - UE Direction of Travel for FR2 channel models

    - Make decision on DoT for FR2, based on the same approach for FR1 in [R4-1915060]

  - CE vendors align on channel model tap resolution

  - Initial phase definition of channel model shall be studied

In this paper, we provide the analysis on the feasible SNR ranges for static MIMO OTA in 3D-MPAC system.

# 2	Discussion

To study the feasibility of adding 64QAM RMC, the initial feasible SNR was analyzed in [2]. While based on the agreements of 0.75m minimum range length and [8] probes in chamber, further analysis on feasible SNR ranges is needed.

In FR2 demodulation testability study, the maximum testable SNR was derived in the attached calculator in TR38810. To achieve 1dB difference between Reference point SNR and Baseband SNR, artificial noise is sent with wanted signal from the test equipment.

The levels of artificial noise (Noc levels) are based on UE REFSENS for the operating band and UE Power class. Taking a baseline of UE Power Class 3 in band n260, the Noc can be calculated by the following equation:

Noc = RESFENSPC3, n260, 50MHz -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal

where:

-	REFSENSPC3, n260, 50MHz is the REFSENS value in dBm specified for Power Class 3 UE in band n260 for 50MHz channel bandwidth in TS 38.101-2 Table 7.3.2.3-1, [dBm/Hz]

-	SCSREFSENS is a subcarrier spacing associated with NRB for 50MHz in TS 38.101-2 Table 5.3.2-1, chosen as 120kHz.

-	PRBREFSENS is NRB associated with subcarrier spacing 120kHz for 50MHz in TS 38.101-2 Table 5.3.2-1 and is 32.

-	12 is the number of subcarriers in a PRB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB.

With that, the SNR upper bound for Direct far field (DFF) with 100MHz channel bandwidth multi-band UE is 17.4dB.

While in FR2 MIMO OTA testing, the noise-limited test condition was agreed to be used which means only wanted signal will be transmitted from test equipment. With the 0.75m measurement distance and further consider the probe number is [8], we update the attached calculator with the assumption of 1 probe and the following SNR upper bound values for both 1 and 8 probes are provided in Table 1.

Table 1: SNR upper bound value for MIMO OTA with measurement distance of 0.75m

|  | Channel Bandwidth | Maximum SNR with 1 probe | Maximum SNR with 8 probe3 |
| --- | --- | --- | --- |
| Multi-band UE (Note) | 100 MHz | [24.4dB] | [33.4dB] |
| Note 1: For ∑MBp from TS 38.101-2 Table 6.2.1.3-4 allow up to 2 dB. Note 2: The SNR is calculated based on the UE noise floor derived by REFSENS. Note 3: Compared with 1 probe, 8 probes can reach up to 9dB gain. |  |  |  |

Observation 1: The SNR upper bound for MIMO OTA with 8 probes is 33.4dB for 100MHz channel bandwidth.

We should note that the maximum SNR of 33.4dB is calculated with the assumption that DUT just meets the REFSENS. Therefore, there should be some margin between actual feasible SNR and the upper bound SNR shown in Table 1.

Proposal: Capture SNR upper bound values in Table 1 and attached calculator in TR38.827.

# 3	Conclusion

In this paper, we provide the analysis on feasible SNR ranges for NR FR2 MIMO OTA in 3D MPAC. The following observations and proposal are given:

Observation 1: The SNR upper bound for MIMO OTA with 8 probes is 33.4dB for 100MHz channel bandwidth.

We should note that the maximum SNR of 33.4dB is calculated with the assumption that DUT just meets the REFSENS. Therefore, there should be some margin for the upper bound SNR of 33.4dB.

Proposal: Capture SNR upper bound values in Table 2 and attached calculator in TR38.827.

# 4	Reference

- R4-1916174, “WF on FR2 MIMO OTA”, CAICT, Spirent, 3GPP RAN4 #93 meeting, Nov. 2019.

- R4-1915091, “Adding 64QAM RMC for NR FR2 MIMO OTA”, Qualcomm, 3GPP RAN4 #93 meeting, Nov. 2019.

- TR38.810v16.5.0