3GPP TSG-RAN4 Meeting #94-e	R4-2000526

# Online, 24th Feb 2020 - 6th Mar 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Discussion on RAN5 LS on Multiband relaxation for FR2

Agenda Item: 			6.5.6

Document for:	Approval

# 1	Introduction

RAN4 has received RAN5 LS on Multiband relaxation for FR2 [1] which was triggered by RAN4 LS [2] on proposal how to use the multi-band relaxation (MBR) for FR2 RF testing, suggesting that the UE vendor declares per band relaxation factors which should comply with the max allowed accumulated value over supported bands defined in TS 38.101-2. RAN5 has identified two problems when implementing the RAN4 guidance [2].

Problem description 1 - Some bands may not be tested: The issue is that if not all the supported bands by the UE are tested, a situation that will be common as more bands are being defined in 3GPP, the UE could safely assign all the allowed multiband relaxation on the tested bands (setting MB=0 on untested bands) thereby allowing a non-compliant UE to pass certification

Problem description 2 - Impact on testability analysis if MBR increase: If maximum MBR becomes larger, RAN5 will need to re-evaluate testability issues per test case. Hence RAN5 cannot and does not intend to keep re-opening testability topics considered complete.

# 2	Discussion

It seems based on RAN5 LS that something needs to be done for multiband relaxation for FR2 to address the two problems RAN5 raised. In this paper we discuss some alternatives.

- Alternative 1: Remove the MBR from REL16 38.101-2 specification. In our view MBR was introduced to assist introduction of early UE and is not anymore needed in REL-16. Concerning problem 1: RAN4 could indicate that case of allowing a non-compliant UE to pass certification is not very likely but even if in some case this could happen it can be considered acceptable for REL15 early UEs. Concerning problem 2: If MBR is removed from REL-16 specification then it obviously will not change in future. Additional benefit is that there is no need to discuss MBR every time new band is introduced and as we know one new band leads typically many new MBRs.

- Alternative 2: Replace MBR with CA style dTib/dRib relaxation which is fixed value per band. Widely used since REL-10 hence does not have RAN5 issues. Introduction of new band needs similar RAN4 attention as MBR because dTib/dRib relaxation is needed for new band combinations. Current MBR needs to be converted into dTib/dRib such way that per band combination ∑ dTib=∑MBP and ∑ dRib=∑MBS.

- Alternative 3: Remove MBR and introduce lower tolerance values for peak EIRP and EIRP spherical coverage requirement. This approach is not technically as sound as Alt 2 because it would give same tolerance to all bands but depending on the band combination the claimed need is not the same. However this solution would likely solve RAN5 issues. Introduction of new band does not need additional work assuming tolerance is same for all bands but needed work is limited in any case to one band.

Our preference is Alt 1 thus

Proposal: Multi-band relaxations are removed from REL-16 38.101-2 and are not applicable for REL-16 and beyond UEs.

# 3	Conclusion

In this contribution we have discussed the RAN5 LS on Multiband relaxation for FR2 and how RAN4 can solve RAN5 problems. As we are also in a view that MBR was intended to support introduction of early UE we propose.

Proposal: Multi-band relaxations are removed from REL-16 38.101-2 and are not applicable for REL-16 and beyond UEs. Inform RAN5.

# 4	References

[1] R5-199424, LS on Multiband relaxation for FR2, Reno

[2] R4-1902181