3GPP TSG-RAN4#94-e	R4-2000537

24th – 06th Mar 2020

Source: 	Nokia, US Cellular

Title:	TP for TR 37.716-21-21: DC_66A_n12A-n258A

Agenda item:			9.7.3

Document for:	Approval

# 1	Introduction

This is a TP into TR 37.716-21-21 to add DC_66A_n12A-n258A

********************************** Start of TP ***********************************************

## 6.x	DC_66_n12-n258

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n12-n258 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n12A-n258A | DC_66A_n258A DC_66A_n12A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 435 |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n258 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

Co-existence analysis is summarized in Table 5.3-1.

### 6.x.4	∆TIB and ∆RIB values

For DC_66_n12-n258 the TIB,c and RIB,c values are derived from DC_12_n66 combination.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n12-n258 | 66 | 0.5 |
|  | n12 | 0.8 |
|  | n258 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n12-n258 | 66 | 0 |
|  | n12 | 0 |
|  | n258 | 0 |

### 6.x.5	MSD

There is no need to define MSD.

********************************** End of TP ***********************************************