3GPP TSG-RAN4 Meeting #94-e		R4-2000586

Online, 24th Feb – 6th Mar, 2020

Title:	Discussion on CSI-RS based measurement requirements

Source:	CATT

Agenda Item:	8.16.1.4

Document for:	Discussion

- Introduction

In RAN #84 meeting, a new WID on RRM requirements for CSI-RS based L3 measurement was approved [1]. CSI-RS based mobility plays an important role in mobility and network deployment/optimization. And the objectives of this WI are described as follows:

- Assumptions on Synchronization - Single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements. - Measurement bandwidth of CSI-RS for the minimum requirements - CSI-RS based intra-frequency and inter-frequency definition - Intra-frequency and inter-frequency measurement requirements - Evaluate and specify CSI-RS based L3 measurement accuracy - Introduce new UE measurement capability, including number of frequency layer and number of cells etc., for CSI-RS based measurement

In this contribution, we provide some discussion on CSI-RS based measurement requirements, and give our proposals.

- Discussion

CSI-RS resources can be configured by network for mobility purpose, and UE shall be able to perform CSI-RSRP/CSI-RSRQ/CSI-SINR measurements on configured CSI-RS resource(s). According to TS38.214 [2], CSI-RS resources for mobility can be configured with or without associated SSB.

If associated SSB is not configured for CSI-RS resources, UE will assume the timing of neighbour cell is the same as the serving cell and there is no need to configure the associated SSB for the UE to obtain the timing of the neighbour cell in order to detect the target CSI-RS resource. Thus, with such configuration, UE in not required to perform cell identification prior to the CSI-RS based measurement. Hence, the CSI-RS identification delay requirement can be expressed as: TCSI-RS_identification_without_associatedSSB = TCSI-RS_measurement_period.

Proposal 1: If associated SSB is not configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_without_associatedSSB = TCSI-RS_measurement_period.

If associated SSB is configured for CSI-RS resources, UE is assumed that the timing of the neighbour cell is not synchronized to the serving cell, and UE needs first detect the associated SSB to get the timing of the neighbour cell before measuring the target CSI-RS resource. In addition, according to 38.331, as the network will configure the associated SSB index to UE, UE is not required to read SSB index. However, UE needs to perform cell identification prior to the CSI-RS based measurement if the target cell is the first time to measure and the cell is detectable. Hence, the CSI-RS identification time can be expressed as: TCSI-RS_identification_with_associatedSSB = TPSS/SSS_sync + TCSI-RS_measurement_period.

CSI-RS-Resource-Mobility ::=        SEQUENCE {

csi-RS-Index                        CSI-RS-Index,

slotConfig                          CHOICE {

ms4                                 INTEGER (0..31),

ms5                                 INTEGER (0..39),

ms10                                INTEGER (0..79),

ms20                                INTEGER (0..159),

ms40                                INTEGER (0..319)

},

associatedSSB                       SEQUENCE {

ssb-Index                           SSB-Index,

isQuasiColocated                    BOOLEAN

}

Proposal 2: If associated SSB is configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_with_associatedSSB = TPSS/SSS_sync + TCSI-RS_measurement_period.

In case that CSI-RS resource is configured with associated SSB, network will configured the QCL information between the configured CSI-RS resources and the associated SSB.

When the configured CSI-RS resource is QCLed to the associated SSB, UE would use the same Rx beam selected for SSB measurement to perform CSI-RS based measurement in FR2. And the CSI-RS based measurement period does not need to consider the scaling factor due to Rx beam sweeping. Otherwise, the scaling factor due to Rx beam sweeping shall be considered.

Proposal 3: If the CSI-RS resource is configured with associated SSB and QCLed to the associated SSB, the CSI-RS based measurement requirement does not need to consider the scaling factor due to Rx beam sweeping.

For SSB-based measurement, the scheduling restriction was introduced for the following cases:

- Mix-numerology between data and SSB symbols

- RX beam sweeping in FR2

- Collision between UL transmission and DL measurement for TDD carrier

For case 1, since the SCS of CSI-RS for RRM measurement can be configured independently of the SCS of PDSCH/PDCCH, thus, if UE does not support to receive simultaneous data or SSB of serving cell and CSI-RS signal of neighbour cell, as illustrated in figure 1, UE will prioritize to perform CSI-RS based measurement on neighbour cell and the requirements of scheduling restriction is applied to the mixed numerology between data/SSB of serving cell and CSI-RS of neighbour cell.

Figure 1: Mixed numerology case

For case 2, if UE needs a different Rx beam other than the Rx beam used for serving cell reception to perform CSI-RS measurement on neighbour cell, and the requirement of scheduling restriction specified for SSB measurement in FR2 will be reused for CSI-RS based measurement in FR2.

For case 3, in case there is the collision between UL transmission and neighbour cell measurement, according to my understanding, the same principle defined for SSB based measurement shall be reused, in other word, UE will prioritize to perform CSI-RS based neighbour cell measurement other than UL transmission.

According to the analysis for case 1-3, the scheduling restriction for CSI-RS based measurement shall be introduced for the following cases:

- Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell

- RX beam sweeping in FR2

- Collision between UL transmission and DL measurement for TDD carrier

Proposal 4: the scheduling restriction for CSI-RS based measurement shall be introduced for the following cases:

- Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell

- RX beam sweeping in FR2

- Collision between UL transmission and DL measurement for TDD carrier

For SSB based measurement, 2 searchers are assumed for UE to perform SSB based intra-frequency measurement, where one dedicated searcher is used for NR PCell (SA) or NR PSCell (NSA) measurements and an additional searcher shared among SCells. And the scaling factor CSSF was introduced for the measurement conducted outside measurement gap and within measurement gap, respectively.

The CSI-RS based measurements should be shared with the SSB based measurement on SCells, and the CSSF factor should be updated by considering the CSI-RS based measurement within gap and outside gap.

CSI-RS resource can be configured within, partial overlapping or outside the active BWP, and the CSI-RS of neighbour cell can be configured with associated SSB or non-associated SSB. Figure 12 and 3 illustrate the CSI-RS based measurement within and outside the active BWP, respectively. According to our understanding, as long as the target CSI-RS resource configured for neighbour cell is within the serving cell’s active BWP, the measurement for target CSI-RS can be considered as measurement within gap. Otherwise, the measurement for target CSI-RS can be considered as measurement outside gap.

Proposal 5: the CSSFwithin_gap specified in 38.133 should be updated by considering the CSI-RS based measurement within the active BWP, and the CSSFoutside_gap specified in 38.133 should be updated by considering the CSI-RS based measurement outside the active BWP.

Figure 2: CSI-RS measured outside the active BWP

Figure 3: CSI-RS measured within the active BWP

According to TS38.214, it has been agreed the following UE measurement behaviour restriction for CSI-RS based measurement.

If the UE is configured with DRX, the UE is not required to perform measurement of CSI-RS resources other than during the active time for measurements based on CSI-RS-Resource-Mobility. If the UE is configured with DRX and DRX cycle in use is larger than 80 ms, the UE may not expect CSI-RS resources are available other than during the active time for measurements based on CSI-RS-Resource-Mobility. Otherwise, the UE may assume CSI-RS are available for measurements based on CSI-RS-Resource-Mobility.

In DRX mode, UE is not required to perform measurement of CSI-RS resources outside of the active time of DRX cycle. And if the DRX cycle is larger than 80ms, UE may not expect CSI-RS resources are available other than during the active time for measurement. We think there is the similar case when UE is configured with measurement gap to perform CSI-RS based measurement on neighbour cell, since the maximum length of measurement gap is 6ms, however the CSI-RS resources can be randomly configured during the period, thus, not all the CSI-RS resources can be within the measurement gap.

Hence, we think the network should not randomly configure the parameter “slotConfigs” for CSI-RS based measurement for neighbour cells, the CSI-RS resource configuration needs to carefully match the DRX configuration or measurement gap configuration. For requirement side, we think the following options can be considered to define the corresponding requirements:

- Option 1: UE is allowed not to measure the CSI-RS resources that are not within DRX on-duration or measurement gap;

- Option 2: No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap;

- Option 3: the network needs to make sure the configured CSI-RS resources falls within the configured measurement gaps

Proposal 6: For CSI-RS based measurement, the following options can be considered to define the corresponding requirements:

- Option 1: UE is allowed not to measure the CSI-RS resources that are not within DRX on-duration or measurement gap;

- Option 2: No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap;

- Option 3: the network needs to make sure the configured CSI-RS resources falls within the configured measurement gaps

- Conclusion

In this contribution, we discussed the CSI-RS based UE measurement capabilities, and give our proposal as follows.

Proposal 1: If associated SSB is not configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_without_associatedSSB = TCSI-RS_measurement_period.

Proposal 2: If associated SSB is configured for CSI-RS resources, the CSI-RS identification time can be expressed as: TCSI-RS_identification_with_associatedSSB = TPSS/SSS_sync + TCSI-RS_measurement_period.

Proposal 3: If the CSI-RS resource is configured with associated SSB and QCLed to the associated SSB, the CSI-RS based measurement requirement does not need to consider the scaling factor due to Rx beam sweeping.

Proposal 4: the scheduling restriction for CSI-RS based measurement shall be introduced for the following cases:

- Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell

- RX beam sweeping in FR2

- Collision between UL transmission and DL measurement for TDD carrier

Proposal 5: the CSSFwithin_gap specified in 38.133 should be updated by considering the CSI-RS based measurement within the active BWP, and the CSSFoutside_gap specified in 38.133 should be updated by considering the CSI-RS based measurement outside the active BWP.

Proposal 6: For CSI-RS based measurement, the following options can be considered to define the corresponding requirements:

- Option 1: UE is allowed not to measure the CSI-RS resources that are not within DRX on-duration or measurement gap;

- Option 2: No UE performance requirement is defined for the CSI-RS resources that are not within DRX on-duration or measurement gap;

- Option 3: the network needs to make sure the configured CSI-RS resources falls within the configured measurement gaps

- Reference

[1]	RP-191580	New WID on RRM requirements for CSI-RS based L3 measurement, CATT

[2]	TS38.214

[3]	TS38.133