3GPP TSG-RAN WG4 Meeting #94-e	R4-2000603

Online, 24th Feb. – 6th Mar. 2020

Title: 	Discussion on Rx-Tx timing difference measurement

Source: 	CATT

Agenda item:	8.8.2.1.4

Document for:	Discussion

- Introduction

In last RAN4#93 meeting, a way forward is approved [1]. The conclusion for UE Rx-Tx time difference measurement has been gotten in the way forward:

=====

UE Rx-Tx time difference measurement period

- When UE Rx-Tx time difference measurement is configured to be measured along with PRS-RSRP using the same assistance data:

  - then whether the measurement periods of UE Rx-Tx time difference measurement and PRS-RSRP are the same or not, is FFS.

- Otherwise:

  - In non-DRX the UE Rx-Tx time difference measurement period is FFS.

  - When DRX is used then whether or not the UE Rx-Tx time difference measurement period depends on DRX cycle may depend on the positioning method; details are FFS.

- RAN4 needs to check RAN1/2 agreements if UE Rx-Tx time difference measurement can be configured to be measured along with RSTD using the same assistance data

Side conditions for UE Rx-Tx time difference

- Serving cell:

  - Side conditions (PRS Es/Iot) for UE Rx-Tx time difference in FR1 are FFS.

  - Side conditions (PRS Es/Iot) for UE Rx-Tx time difference in FR2 are FFS.

- Neighbour cell:

  - Side conditions (PRS Es/Iot) for UE Rx-Tx time difference and for RSTD measurement in FR1 are the same.

  - Side conditions (PRS Es/Iot) for UE Rx-Tx time difference and for RSTD measurement in FR2 are the same.

UE Rx-Tx time difference measurement under cell change

- If the cell change occurs on the serving cell where the SRS is configured then after the serving cell change:

  - the UE shall restart the UE Rx-Tx time difference measurement;

- otherwise the UE shall continue the ongoing UE Rx-Tx time difference measurement.

=====

In this contribution, we further discuss the UE Rx – Tx timing difference measurement based on the way forward.

- Discussion

For open issues of UE Rx-Tx time difference measurement period requirements, we have following considerations.

UE Rx-Tx time difference measurement period

The requirements of UE Rx-Tx time difference measurement period for E-UTRA can be refer. When UE Rx-Tx time difference measurement is configured to be measured along with PRS-RSRP using the same assistance data, the measurement period of UE Rx-Tx time difference measurement is prefer defined same as PRS-RSRP. Otherwise UE will measure UE Rx-Tx time difference with other reference signal such as SSB signal and/or CSI-RS. In this case, the measurement period of UE Rx-Tx time difference can be defined same as measurement period of SSB-RSRP or CSI-RSRP measurement in non-DRX and DRX mode.

Proposal 1: The measurement period of UE Rx-Tx time difference is defined same as measurement period of PRS-RSRP measurement when UE Rx-Tx time difference measurement is configured to be measured along with PRS-RSRP using the same assistance data, otherwise, same as measurement period of SSB-RSRP or CSI-RSRP measurement in non-DRX and DRX mode.

Side conditions for UE Rx-Tx time difference

For serving cell, the side conditions (PRS Es/Iot) for UE Rx-Tx time difference are defined same as side conditions for SSB-RSRP measurement, such as -6dB. No necessary defined lower side conditions due to cell reselection and handover.

Proposal 2: Side conditions (PRS Es/Iot) for UE Rx-Tx time difference for serving cell are defined same as side conditions for SSB-RSRP measurement in FR1 or FR2.

- Conclusion

The document discusses the open issues for UE Rx-Tx time difference measurement, following proposals are presented:

Proposal 1: The measurement period of UE Rx-Tx time difference is defined same as measurement period of PRS-RSRP measurement when UE Rx-Tx time difference measurement is configured to be measured along with PRS-RSRP using the same assistance data, otherwise, same as measurement period of SSB-RSRP or CSI-RSRP measurement in non-DRX and DRX mode.

Proposal 2: Side conditions (PRS Es/Iot) for UE Rx-Tx time difference for serving cell are defined same as side conditions for SSB-RSRP measurement in FR1 or FR2.

- Reference

[1]	R4-1915854, Way forward on NR Positioning RRM, Ericsson

[2]	3GPP TS 38.215 V16.0.1, NR: Physical layer measurements