3GPP TSG-RAN WG4 Meeting #94-e                                                             R4-2000610

# Online, 24th February – 6th March, 2020

Source: 	CATT

Title: 	Initial simulation results for NRHST PUSCH demodulation requirement

Agenda item:	8.17.2.2.1

Document for:	Agreement

- Introduction

In the recent RAN4#93 meeting, a WF [1] on NR HST BS demodulation requirements was approved. This paper will provide our evaluation results for NR HST PUSCH for simulation alignment based on agreed simulation asumptions in WF.

- Evaluation results

The agreed simuation asumptions in WF[1] are summarized and shown in Table A-1 in Annex.

The evaluation results for 350km/h and 500km/h are shown in Table 2-1 and Table 2-2 respectively.

Table 2-1 Evaluation results for 350km/h

| Channel model | Propagation condition | Carrier frequency (GHz) | SCS | CBW | Maximum Doppler shift | Tx/Rx | PUSCH mapping type | DMRS | l0 | MCS | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| AWGN | Tunnel | 2.1 | 15KHz | 10MHz | 1340Hz | 1T1R[FFS] | Type A | 1+1+1 | 2[FFS] | 2 | -3.47 | -1.47 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | 8.65 | 10.65 |
|  |  |  |  |  |  | 1T2R | Type A | 1+1+1 | 2[FFS] | 2 | -6.2 | -4.2 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | 5.68 | 7.68 |
|  |  | 3.6 | 30KHz | 40MHz | 2334Hz | 1T1R[FFS] | Type A | 1+1+1 | 2[FFS] | 2 | -3.37 | -1.37 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | 8.73 | 10.73 |
|  |  |  |  |  |  | 1T2R | Type A | 1+1+1 | 2[FFS] | 2 | -6.11 | -4.11 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | 5.79 | 7.79 |
|  | Open space | 2.1 | 15KHz | 10MHz | 1340Hz | 1T2R | Type A | 1+1+1 | 2[FFS] | 2 | -6.21 | -4.21 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | 5.67 | 7.67 |
|  |  |  |  |  |  | 1T8R | Type A | 1+1+1 | 2[FFS] | 2 | -11.2 | -9.2 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | -0.24 | 1.76 |
|  |  | 3.6 | 30KHz | 40MHz | 2334Hz | 1T2R | Type A | 1+1+1 | 2[FFS] | 2 | -6.12 | -4.12 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | 5.76 | 7.76 |
|  |  |  |  |  |  | 1T8R | Type A | 1+1+1 | 2[FFS] | 2 | -11.05 | -9.05 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | -0.11 | 1.89 |

Table 2-2 Evaluation results for 500km/h

| Channel model | Propagation condition | Carrier frequency (GHz) | SCS | CBW | Maximum Doppler shift | Tx/Rx | PUSCH mapping type | DMRS | l0 | MCS | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| AWGN | Tunnel | 1.8 | 15KHz | 10MHz | 1740Hz | 1T1R[FFS] | Type A | 1+1+1 | 2[FFS] | 2 | -3.46 | -1.46 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16[FFS] | 8.67 | 10.67 |
|  |  |  |  |  |  | 1T2R | Type A | 1+1+1 | 2[FFS] | 2 | -6.2 | -4.2 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16[FFS] | 5.7 | 7.7 |
|  |  | 3.6 | 30KHz | 40MHz | 3334Hz | 1T1R[FFS] | Type A | 1+1+1 | 2[FFS] | 2 | -3.37 | -1.37 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16[FFS] | 8.77 | 10.77 |
|  |  |  |  |  |  | 1T2R | Type A | 1+1+1 | 2[FFS] | 2 | -6.11 | -4.11 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16[FFS] | 5.83 | 7.83 |
|  | Open space | 1.8 | 15KHz | 10MHz | 1740Hz | 1T2R | Type A | 1+1+1 | 2[FFS] | 2 | -6.21 | -4.21 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | 5.68 | 7.68 |
|  |  |  |  |  |  | 1T8R | Type A | 1+1+1 | 2[FFS] | 2 | -11.2 | -9.2 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | [TBD] | [TBD] |
|  |  | 3.6 | 30KHz | 40MHz | 3334Hz | 1T2R | Type A | 1+1+1 | 2[FFS] | 2 | -6.12 | -4.12 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | 5.79 | 7.79 |
|  |  |  |  |  |  | 1T8R | Type A | 1+1+1 | 2[FFS] | 2 | -11.04 | -9.04 |
|  |  |  |  |  |  |  | Type A | 1+1+1 | 2[FFS] | 16 | -0.1 | 1.9 |

- Conclusion

This paper provides our evaluation results for NR HST PUSCH for simulation alignment based on agreed simulation asumptions in WF.

- References

- R4-1915886，Way forward on NR HST PUSCH demodulation requirements，Huawei, HiSilicon，RAN4#93

- R4-1910983,Discussion on HST PUSCH demodulation requirement, CATT, RAN4#92bis

- R4-1913725, Discussion on HST PUSCH demodulation requirement, CATT,RAN4#93

- Annex

Table A-1 Simulation assumptions for NR HST PUSCHwith CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | v=350km/h | V=500km/h |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | Tunnel: 2, FFS on 1 Open space: 2, 8 | Tunnel: 2, FFS on 1 Open space: 2, 8 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| symbols length | 14 | 14 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type A |
| MCS index | Tunnel: MCS 2 and MCS 16 Open space: MCS 2 and MCS 16 | Tunnel: MCS 2, FFS on MCS 16 Open space: MCS 2 and MCS 16 |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| RV | {0,2,3,1} | {0,2,3,1} |
| DMRS type | type 1 | Type1 |
| Number of DMRS symbols | 1+1+1 | 1+1+1 |
| Front loaded DMRS start symbol | l0 = 2 (for simulation alignment) (Note 1) | l0 = 2 (for simulation alignment) (Note 1) |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz | 15kHz: 10MHz; 30kHz: 40MHz |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth |
| Carrier frequency (GHz) | For 15kHz SCS: 2.1GHz For 30kHz SCS: 3.6GHz | For 15kHz SCS: 1.8GHz For 30kHz SCS: 3.6GHz |
| fd | For 15kHz SCS: 1340Hz For 30kHz SCS: 2334Hz | For 15kHz SCS: 1740Hz For 30kHz SCS: 3334Hz |
| Propagation condition | Tunnel: Ds=300m, Dmin=2m; Open space: Ds=700m, Dmin=150m | Tunnel: Ds=300m, Dmin=2m; Open space: Ds=700m, Dmin=150m |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |
| Note 1: Company is encouraged to evaluate the performance difference between l0 = 2 and l0 = 3 |  |  |