3GPP TSG-RAN WG4 Meeting #94-e	R4-2000734

Online, 24 February – 6 March, 2020

Agenda item:	8.8.2.1.5

Source: 	Qualcomm Incorporated

Title: 	On positioning requirements for E-CID

Document for:	Discussion

# Introduction

In RAN1, the following table was provided as a guideline to other working groups for different NR positioning methods and measurements:

| DL/UL Reference Signals | UE Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 DL PRS | DL RSTD | DL-TDOA |
| Rel.16 DL PRS | DL PRS RSRP | DL-TDOA, DL-AoD, Multi-RTT |
| Rel.16 DL PRS / Rel.16 SRS for positioning | UE Rx-Tx time difference | Multi-RTT |
| Rel. 15 SSB / CSI-RS for RRM | SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM), SS-RSRPB (for RRM) | E-CID |

In RAN1#99 meeting, further agreement was made as:

Agreement:

- Exclude SS-RSRPB from the list of UE RRM measurement for E-CID

- Include this agreement in an LS to other RAN 2/4 WGs

In this paper, we present our views on measurement requirements to support E-CID.

# Discussion

E-CID functions on existing R15 RRM measurements based on either SSB or CSI-RS. As such, E-CID positioning should be possible in the location server even for R15 UE’s. Consequently, same core requirements, reporting mapping, and accuracy requirements specified in R15 should be applicable to E-CID measurement.

Proposal 1. R15 core measurement requirements, measurement report mapping, and measurement accuracy requirements for SS-RSRP, SS-RSRQ, CSI-RS-RSRP and CSI-RS-RSRQ are applicable to E-CID positioning technique without any modification.

By the same logic, all existing measurement gap configurations in R15 that are applicable to RRM measurement should also be applicable for E-CID measurements. If any new measurement gap pattern is defined in R16 for RRM enhancements or positioning, it should not be applicable to E-CID measurements.

Proposal 2. All existing measurement gap configurations in R15 applicable to RRM measurements to be applicable for E-CID measurements.

# Conclusions

Proposal 1. R15 core measurement requirements, measurement report mapping, and measurement accuracy requirements for SS-RSRP, SS-RSRQ, CSI-RS-RSRP and CSI-RS-RSRQ are applicable to E-CID positioning technique without any modification.

Proposal 2. All existing measurement gap configurations in R15 applicable to RRM measurements to be applicable for E-CID measurements.

# References

[1] R4-1915854