3GPP TSG-RAN WG4 Meeting #94e	R4-2000767

24th February – 6th March, 2020

Source: 	Qualcomm Incorporated

Title: 	Switching times for MIMO layer only change

Agenda item:	8.7.2

Document for:	Approval

# Introduction

In the last few meetings, power savings for MIMO only change has been discussed extensively. In this paper, we propose via options based on RAN4 previous meeting discussion and RAN1 study.

# Discussion

Currently BWP switching time allows the following switching time:

Table 8.6.2-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

There are two types of UEs already defined. Also using the data usage pattern in RAN1 TR for power saving (TR 38.840 V16.0 [1], section 8.2 simulation assumptions, p71) it is quite evident that switching between the MIMO layers is of the order of hundreds of msec.

We believe that the feature is to allow UE to save power and considering the fact that the switching frequency is of the order of hundreds of msec its important to let the UE turn off as many components as possible. The trade-off between faster switch and turning off more component with the objective of power saving is analysed below.

First, we want to re-iterate the fact that UE is able to turn off more components to save more power if we allow longer BWP switch delay, i.e., using type 2 requirement, compared to type 1. If we take this into consideration, our analysis shows that UE using type 2 BWP switch delay can save additional 8% power when compared to UE using type 1 BWP switch delay. Here is the analysis:

System parameters

| 4Rx power | 1 |
| --- | --- |
| 2Rx fast switch (type1) power | 0.8 |
| 2Rx nominal switch (type2) power | 0.7 |
| inter arrival time (ms) | 200 |
| packet transmission time (ms) | 20 |
| DRx off time (ms) | 80 |

The first three rows are ratio between power consumption of 4Rx BWP, 2Rx BWP using type 1 switch delay, 2Rx BWP using type 2 switch delay.

UE with 2Rx BWP using type 2 switch delay can reduce the power consumption when operating in 2Rx BWP, since more components can be turned off. On the other hand, UE with 2Rx BWP using type 1 switch delay can achieve faster switch but at the expense of less components being turned off, therefore, power consumption is larger.

We consider the traffic model agreed by RAN1[1], inter arrival time of 200ms. Additional assumptions are packet transmission time and DRx off time are from [2].

The following figure illustrate this traffic model with BWP usage:

We plug the 4Rx BWP and 2Rx BWP numbers into the traffic model above, to derive the total power consumption for type 1 and type 2 UE. This analysis shows that using type 2 BWP switch delay saves more power (8% of baseline power consumption) than type 1 BWP switch delay. Baseline scheme refers to UE without MIMO layer adaptation, i.e., in both packet transmission time and idle time, UE is in 4Rx. The following table shows the ratio of power consumption comparison between baseline, type 1 and type 2 BWP switch delay.

|  | Power consumption ratio |
| --- | --- |
| baseline | 1 |
| Type 1 | 0.83 |
| Type 2 | 0.749286 |

Type 2 can save 8% more, compared to type 1 BWP switch delay. This is because in the idle period, type 2 switch delay UE enjoys lower power consumption in the entire period, while type 1 UE only gets 2 ms more time on 2Rx BWP, compared to type 2 UE, but spends more power on the whole idle period in 2Rx BWP. Therefore type 2 UE indeed saves more power than type 1 UE. Even if we assume UE with type 1 and type 2 delay consumes the same power, the additional power saving by faster switching is almost negligible, 25.5% power saving for type 1 compared to 25.1% power saving for type 2.

Hence, we believe that the existing switching delay options, both type 1 and type 2, should be kept for MIMO layer only change to enable better power saving performance. In previous meeting, companies proposed to introduce addition capability signalling flag for MIMO layer only change to accommodate different UE implementations. If UE prefer to save more power, longer switch delay can be signalled, otherwise if UE prefer faster switch, shorter switch delay can be signalled. We agree with this proposal since it provides implementation flexibility to UE and ensure power saving gain as we analysed above can be achieved.

However, infrastructure vendors expressed concern for implementation complexity if additional capability signalling is introduced. In fact, implementation complexity increased also by introducing different switch delay requirements for MIMO layer adaptation only case and general BWP switch, which has been agreed already.

All the studies and design in RAN1 did not assume anything different from the currently defined BWP switch delay.  The result does not depend on a shortened BWP switch delay. In previous meeting, BWP switch delay with both MIMO layer adaption and BWP size and location change has been agreed already. Currently, BWP switch delay discussion solely focus on MIMO layer adaption only case. In TR 38.840 section 5.1 [1], study on both frequency and MIMO layer adaptation achieved significant power saving. Therefore, we believe that the most common use case is to change both MIMO layer and BWP size and location to save power.

Let us analyze the power saving gain different between MIMO layer adaptation only and general BWP change. Using the common power saving simulation results in TR 38.840, 20% by frequency adaptation and 20% by number of antenna change, then following the analysis shown above, we found that much larger power saving gain (gap is larger than the previous case!) is achieved by general BWP (MIMO layer + BWP) switch.

|  | Power consumption ratio |
| --- | --- |
| baseline | 1 |
| Type 1(MIMO layer only) | 0.694 |
| Type 2(MIMO layer + BWP) | 0.832857 |

Given that MIMO layer adaptation is a corner case in which power saving gain is significantly lower than general BWP switch with both MIMO layer and BWP adaptation, keeping the same BWP switch delay requirement for both cases, general BWP and MIMO layer adaptation cases, is a reasonable option by taking implementation complexity into consideration

Proposal 1:We propose two options for BWP switch delay requirement for MIMO layer adaptation only cases:
(1) From UE implementation flexibility perspective, additional capability signalling flag for MIMO layer only change is introduced to report BWP switch delay time, with options the same as general BWP switch delay, namely, type 1 and type 2 delay.
(2) From gNB implementation complexity perspective, no additional capability signalling is introduced, MIMO layer adaptation only case follows general BWP switch delay requirement.

# Conclusion

Proposal 1:We propose two options for BWP switch delay requirement for MIMO layer adaptation only cases:
(1) From UE implementation flexibility perspective, additional capability signalling flag for MIMO layer only change is introduced to report BWP switch delay time, with options the same as general BWP switch delay, namely, type 1 and type 2 delay.
(2) From gNB implementation complexity perspective, no additional capability signalling is introduced, MIMO layer adaptation only case follows general BWP switch delay requirement.

# References

[1] TR.38.840

[2] R4-1914116 MediaTek, “Discussion on UE dynamic adaptation to the maximum number of MIMO layers”.