3GPP TSG-RAN WG4 Meeting #94-e	R4-2000808

Electronic Meeting, 24th Feb - 6th Mar 2020

Source:	ZTE Corporation

Title:	Simulation results for UL timing adjustment

Agenda Item:	8.17.2.2.3

Document for:	Approval

# Simulation setup

In RAN4#93, Scenario Y was chosen to simulate for UL timing adjustment [1], and the following setup is shown in details for 15k SCS and 30k SCS respectively:

Table-1 Simulation setup for 15k SCS

|  | UE0 (Moving) | UE1(Stationary) |
| --- | --- | --- |
| Channel | AWGN | AWGN |
| TA | A=10us,w=0.13 | 0 |
| CBW | 10M | 10M |
| DMRS | [2,7,11] | [2,7,11] |
| RB | S:0, L:25 | S:25, L:25 |
| Frame format | 3D1S1U | 3D1S1U |
| SCS | 15k | 15k |
| Antenna configuration | 1T2R | 1T2R |
| Waveform | CP-OFDM | CP-OFDM |
| MCS | 16 | 16 |
| HARQ | Open | Open |

Table-2 Simulation setup for 30k SCS

|  | UE0 (Moving) | UE1 (Stationary) |
| --- | --- | --- |
| Channel | AWGN | AWGN |
| TA | A=5us,w=0.26 | 0 |
| CBW | 40M | 40M |
| DMRS | [2,7,11] | [2,7,11] |
| RB | S:0, L:50 | S:50, L:50 |
| Frame format | 7D1S2U | 7D1S2U |
| SCS | 30k | 30k |
| Antenna configuration | 1T2R | 1T2R |
| Waveform | CP-OFDM | CP-OFDM |
| MCS | 16 | 16 |
| HARQ | [0,2,3,1] | [0,2,3,1] |

# Simulation results

The simulation results for required SNRs of UE0 and UE1 at 70% throughput are shown in below table:

|  | UE0 (Moving) | UE1 (Stationary) |
| --- | --- | --- |
| SCS 15kHz | 6.75 | 6.52 |
| SCS 30kHz | 6.74 | 6.52 |

# Reference

R4-1915886 Way forward on NR HST PUSCH demodulation requirements, Huawei