3GPP TSG-RAN WG4 Meeting #94-e	R4-2000824

Online, 24 Feb. – 6 Mar., 2020

Source: 	Huawei, HiSilicon, China Telecom, China Unicom, BT plc

Title:	FR1 IAB frequency band

Agenda Item:	8.5.1

Document for:	Approval

1	Introduction

IAB frequency bands have been discussed in previous meetings. In this contribution we provide our view on additional FR1 band for IAB deployments.

2	Discussion

At RAN4#90bis meeting, it was agreed that IAB node should support all the FR2 NR bands [1].

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |

At RAN4#91 meeting, it was agreed to introduce FR1 NR bands n41 and n79 in the IAB node specification [2]. Other FR1 frequency bands are not precluded according to operator input.

| NR operating band | Uplink (UL) and Downlink (DL) operating band FUL,low – FUL,high FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- |
| n41 | 2496 MHz – 2690 MHz | TDD |
| n79 | 4800 MHz – 5000 MHz | TDD |

As the primary FR1 bands for NR, Band n77 and n78 support wide channel bandwidths such as 100 MHz. IAB solution in band n77 and n78 can provide the benefit at the deployment scenarios of capability enhancement and cell coverage extension. In order to achieve more flexible and relative low cost network deployment, IAB node can be considered as a candidate solution. Since FR1 bands n41 and n79 were already agreed to be introduce in IAB specification, it is expected the additional work effort will not increase much to also introduce n77 and n78.

Based on above reasons,

Proposal: IAB node should support band n77 and n78

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

3	Conclusion

Proposal: IAB node should support band n77 and n78

4	References

[1] R4-1904252, IAB frequency band, Ericsson,

[2] R4-1905532, Discussion on NR IAB deployment scenarios, CMCC