3GPP TSG-RAN WG4 Meeting #94-e                                    R4-2000900

Online, 24 Feb. – 06 Mar., 2020

Agenda item:	8.5.4.1.1

Source: 	CMCC

Title: 	Discussion on IAB Conducted Requirements

Document for:	Discussion

Introduction

At the RAN4 #93 meeting, delegates have discussed RF requirements of the IAB nodes and related issues such as definition of frequency errors. However, in regard to detailed requirements, several related issues remain. In this contribution, an overview of IAB RF requirements for FR1 is presented. Related issues are also discussed.

Discussion

Following similar approach adopted in Reno meeting for FR2 in [1], an overview of the RF requirements of IAB node operating in FR1 is presented as follows. At present, hybrid requirements are defined for IAB node in FR1, and it is commonly understood that directional requirements of IAB node for both IAB DU and IAB MT will be defined as OTA requirements and the rest requirements will be considered as conducted requirements. Similar to IAB node in FR2, for IAB DU, most requirements can be imported from NR BS specifications with proper referencing/self-contained manner. For IAB MT, since it is considered to perform backhaul function similar to UE but may resembles the requirements of NR BS in some cases, some details in regard to specific requirements need to be further discussed.

- Output power: Radiated Tx power is measured as EIRP for each declared beam by the vendors to verify the directional Tx performance of the transmitter. In IAB node at FR1, radiated Tx power should apply to both IAB DU and IAB MT due to the fact that IAB node is considered a part of the operators network. Similarly, EIS is also a directional OTA requirement based upon one or more OSDD. Such requirement should also apply to IAB MT and DU.

For NR BS, different transmit power requirements are specified under normal and extreme conditions. For IAB node, considering the deployment scenario of the IAB node will mostly be at a certain height outside the building, different minimum requirements should also be specified for IAB MT as well as IAB DU under normal and extreme conditions.

- Referencing/self-contained: in WF [2], two options are proposed for properly handling IAB TS. Option 1 is referencing and Option 2 is to produce a self-contained TS. At present, it is stated that both options can be kept. From our perspective, a considerable amount of IAB node definitions are based on BS/UE specifications. In addition, from the implementation point of view, it is assumed that engineers shall have basic knowledge of BS/UE in advance. Therefore, Option 1 referencing seems to be the more prospective choice.

- Frequency error: For IAB DU, traditional method similar to a BS can be used to measure against the absolute reference frequency. For IAB MT, since IAB DU and IAB MT are implemented in the same IAB node, measurement against the absolute reference frequency from IAB DU side is technically feasible for IAB MT. In many cases, this will increase the accuracy and avoid accumulative error in the frequency error in multi-hop case. However, this is also limited by the IAB MT procedure of receiving reference signal from its donor and the frequency error is measured against the receive signal instead. Based on the discussion from previous meeting, a hierarchical requirement set with both options will be beneficial considering the special case of IAB node.

| Table - IAB node RF requirements in FR1 |  |  |  |
| --- | --- | --- | --- |
| Tx Requirements |  |  |  |
|  |  | IAB MT | IAB DU |
| Output power | Radiated Tx power | OTA EIRP based on BS specs | OTA EIRP based on BS specs |
|  | output power | Conducted based on BS specs with modifications | Conducted based on BS specs |
| output power dynamics | ON/OFF power/transient | Conducted based on UE specs | Conducted based on UE specs |
|  | power control | FFS |  |
| Frequency error |  | Based on UE specs | Based on BS specs |
| Unwanted emissions | ACLR | FFS | Based on BS specs |
|  | Spurious Emission | Based on BS specs | Based on BS specs |
| Tx intermodulation |  | Based on UE specs | Based on BS specs |
| Rx Requirements |  |  |  |
| OTA sensitivity |  | OTA EIS based on BS specs | OTA EIS based on BS specs |
| Adjacent channel selectivity |  | FFS depending on PC | Based on BS specs |
| Blocking characteristics |  | FFS depending on PC | Based on BS specs |
| Receiver spurious emissions |  | Based on BS specs | Based on BS specs |
| Rx intermodulation |  | Not defined for UE in FR1 | Based on BS specs |

Conclusions

In this contribution, an overview of conducted requirements of FR1 is presented.

Reference

- R4-1913412, Overview of IAB RF requirements in FR2, noted.

- R4-1916000, WF on IAB TS spec structure and terminology.