3GPP TSG-RAN WG4 Meeting #94e	R4-2000902

Online, 24 Febuary – 6 March, 2020

Agenda item:	8.4.3.1

Source:	CMCC

Title:	NR V2X licensed frequency bands for SL operation

Document for:	Approval

# 1	Introduction

To allow the V2X SL operation in licensed band, in last meeting RAN4 defined n38 for NR-V2X SL operation[1] and related operating scenarios as below:

-	n38 SL operation in whole n38 band without NR Uu

-	n38 SL operation V2X UE operated with n38 NR Uu UE with synchronous operation(TDD-TDD).

-	n38 SL operation does not allow the standalone operation which will be operated with NR Uu in other licensed band for NR V2X UE e.g. V2X_n20A-n38A in rel-16

In this contribution, we suggest to specify NR SL operation in licensed Band n34, n39, n40, n41 and n79.

# 2	Discussion

Last meeting defined the first licensed frequency band n38 for NR-V2X SL operation. This contribution proposes some new candidate licensed bands for NR V2X SL operation. According to the co-existence simulation analysis, the operating limits of PC5-based NR V2X service are acceptable when operating in thess new licensed frequecny bands with the conditions that the entire band is allocated for SL operation or the SL operation is in sync with the non-V2X operation in the same band.

Proposal 1: Specify NR SL operation in licensed Bands n34, n39, n40, n41 and n79 with the conditions that the entire band is allocated for SL operation in a particular region or the SL operation is in sync with the non-V2X operation in the same band.

NR V2X operating bands in FR1

| NR V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| # n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD | PC5 |
| # n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD | PC5 |
| # n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD | PC5 |
| # n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD | PC5 |
| # n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD | PC5 |

NR V2X Communication channel bandwidth

|  |  | NR V2X band / V2X channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Operating Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| # n34 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | # 30 |  | # Yes | # Yes |  |  |  |  |  |  |
|  | # 60 |  | # Yes | # Yes |  |  |  |  |  |  |
| # n39 | # 15 | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes |  |  |
|  | # 30 |  | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes |  |  |
|  | # 60 |  | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes |  |  |
| # n40 | # 15 | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes |  |
|  | # 30 |  | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes |
|  | # 60 |  | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes | # Yes |
| # n41 | # 15 |  | # Yes | # Yes | # Yes |  | # Yes | # Yes | # Yes |  |
|  | # 30 |  | # Yes | # Yes | # Yes |  | # Yes | # Yes | # Yes | # Yes |
|  | # 60 |  | # Yes | # Yes | # Yes |  | # Yes | # Yes | # Yes | # Yes |
| # n79 | # 15 |  |  |  |  |  |  | # Yes | # Yes |  |
|  | # 30 |  |  |  |  |  |  | # Yes | # Yes | # Yes |
|  | # 60 |  |  |  |  |  |  | # Yes | # Yes | # Yes |

The NR V2X operating scenarios for licensed frequency band has been analysed in last meeting, using n38 as example. These similar operating scenarios could also apply to bands n34, n39, n40, n41 and n79 as below:

•	SL operation in the whole band without NR Uu

•	Synchronous operation(TDD-TDD) between SL V2X UE and NR Uu UE in the same band

•	SL operation does not allow the standalone operation, which means SL in licensed band will be operated with NR Uu in other licensed band for NR V2X UE e.g. V2X_n34A-n79A in rel-16

# 3	Conclusions

The following NR-V2X licensed frequency bands are requested to be defined for SL operation in Rel-16.

Proposal 1: Specify NR SL operation in licensed Bands n34, n39, n40, n41 and n79 with the conditions that the entire band is allocated for SL operation in a particular region or the SL operation is in sync with the non-V2X operation in the same band.

# 4	References

[1] R4-1916144 “TP on 5G V2X UE RF requirements for SL operation at licensed whole band”, RAN4#93, LG Electronics