3GPP TSG-RAN WG4 Meeting #94                                                 R4-2000929

# E-meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.1.4.9

Source:	MediaTek Inc.

Title:	Discussion on RLM requirement for NR-U

Document for:		Discussion

# Introduction

In this paper, it provides the view on RLM requirements for NR-U.

# Discussion

In the last meeting, how to extend the OOS evaluation period has been discussed [1].

RLM: out-of-sync

- Extend the out-of-sync evaluation period

  - Option 1: Out-of-sync evaluation period is scaled by a fixed factor of N (N > 1)

  - Option 2: Same as for other measurements (out-of-sync evaluation is based on Lout, where Lout ≤Lout,max is the number of SSBs not available at the UE during TEvaluate_out_SSB)

    - FFS if UE can distinguish whether signal is available for RLM out-of-sync

For Option 2, it would rely on UE detection for the presence of SSBs to extend the period according to the number of SSBs not available at the UE. Since DL LBT failure is unknown to UE, UE would just perform SINR estimation on the candidate SSB positions by assuming the SSB is transmitted, even if the DRS was not transmitted by BS. One conjecture in last meetings was that UE may be able to tell whether DRS was transmitted or not based on the RSRP estimation. To address this comment, we conducted simulation in section 3 to further check the reliability of presence detection on SSBs in a single cell scenario. The corresponding simulation assumption is attached in the Appendix for reference. And it can observed that

Observation 1: It is not practical to rely on UE detection for the presence of SSB to extend the measurement/evaluation period under low SNR condition. The mis-detection rate would be >10% when the SNR<-5dB.

Since UE may not be able to reliably detect the existence of SSB in low SNR region as discussed above, the  UE would not know that unavailable SSBs is caused by bad channel condition or LBT failure at BS. Due to this fact, it proposes Opt. 1, where the OOS evaluation period is specified based on the assumption that UE does not detect the presence of SSB.

It can also observe that the fixed scaling factor on OOS evaluation will not always delay the indication of OOS. For example, for an advanced UE, which is able to achieve sufficient accuracy with fewer samples and may be able to detect the presence of SSBs under low SNR condition, it should allow such UE to indicate OOS earlier than the entire OOS evaluation period.

Observation 2: Advanced UEs could send out OOS indications earlier than the entire OOS evaluation period.

Proposal 1: As minimum requirement, the OOS evaluation period is scaled by a fixed factor of N (N > 1), and it should allow UE to indicate OOS indicate earlier than the entire of OOS evaluation period.

For INS evaluation, one discussion is whether for longer DRX cycles > 640, an additional criterion applies, to limit the acceptable period of consecutive LBT failures to [X] seconds. However, it would increase the probability of RLF, since it will reduce the chance of sending INS indications and reduce UE opportunities to keep link when the link is revered from bad channel condition.

Proposal 2: Not to specify limit the acceptable period of consecutive LBT failures for INS evaluation.

Periodic CSI-RS RLM

- RLM requirements based on periodic CSI-RS (TCSI-RS) are to be defined in Rel-16

  - Background: RAN1 agreed that CSI-RS-based RLM-RS both within and outside the SSB-based RLM measurement window (i.e., DRS transmission window) can be used for in-sync and out-of-sync evaluations (resolves FFS from previous agreement)

Another remaining issue is the requirement for the periodic CSI-RS based RLM.

The presence of CSI-RS is more difficult to be checked by UE detection, since CSI-RS was not designed for detection purpose. Therefore, the extension on CSI-RS measurement/evaluation period should not depend on the number of not available samples, since UE would not know whether the CSI-RS is available or not.

Observation 3: UE would not know whether the CSI-RS is available or not.

Nevertheless, it could leverage the decoding on CG-PDCCH. When CG-PDCCH is decoded successfully, UE can assume the LBT was successfully, and the CSI-RS will be available. In other words, for all CSI-RS measurements, including CSI-RS based RLM, CSI-RS based CBD, CSI-RS based L1-RSRP, UE is only required to measure those CSI-RS in COT where UE already decode the CG-PDCCH successfully.

Proposal 3: For CSI-RS measurements, UE is only required to measure those CSI-RS in COT where UE already decode the CG-PDCCH successfully.

Proposal 4: For CSI-RS based INS evaluation, UE is only required to measure those CSI-RS in COT where UE already decode the CG-PDCCH successfully. The evaluation period is extended if the corresponding CG-PDCCH is not decoded successfully.

However, at low SNR regime, the decoding on CG-PDCCH could be not reliable. Thus, it also proposes to extend the OOS evaluation period by a fixed factor.

Proposal 5: For CSI-RS based OOS evaluation, the evaluation period is scaled by a fixed factor of N (N > 1).

# Simulation results for SSB’ presence detection

The CDF of RSRP estimated at UE side under different SNR side conditions are shown in Figure 1 and Figure 2, for SCS of 15kHz and 30kHz respectively. The detailed simulation parameters are listed in the Appendix.   In the figures, the leftmost curve shows the RSRP would be estimated when the DRS samples is not transmitted. The RSRP overlapping region of the leftmost curve and the other curves indicate how likely UE would consider transmitted DRS samples as “not transmitted”.

Figure 1. CDF of 1-sample RSRSP estimation (TDL-C, SCS=15kHz).

Figure 2. CDF of 1-sample RSRSP estimation (TDL-C, SCS=30kHz).

Table 1 summarizes the mis-detection rate under different SNR side conditions, wherein the mis-detection rate is defined as the probability of the detected RSRP of transmitted SSB below the 95%-tile CDF of the detected RSRP when SSB is not transmitted.

Table 1: Mis-detection rate on the presence of a SSB, under fading channel used in R15 RLM test case.

| SNR side condition | Mis-detction rate |  |
| --- | --- | --- |
|  | SCS = 15 kHz | SCS = 30 kHz |
| 0dB | 0.2% | 0.2% |
| -5dB | 7.3% | 7.4% |
| -10dB | 53.6% | 53.6% |
| -12dB | 69.9% | 72.5% |
| -15dB | 85.7% | 86.0% |

Based on above analysis, the reliability of this kind of detection is questionable.

- When SNR is below -10dB, it will lead to high probability (>50%) that UE would consider a SSB as “not transmitted”. Therefore UE is very likely to keep extending the corresponding measurement/evaluation period.

- When the SNR above a certain level (e.g. [-5]dB), UE may be able to detect the present of SSB.

Note that above evaluation is based on one single cell. In other words, if BS does not transmit SSB, the estimated RSRP is purely contributed by noise. If the multi-cell scenario is considered, there could be still a neighboring cell which is transmitting SSB when UE’s serving cell fails LBT. In this case, the estimated RSRP for the case of SSB not transmitted would go higher, since the interference from other cells would be received and introduce additional estimation errors.

Observation 4: For single cell scenario, the mis-detection rate for the presence of SSB will be >50% at SNR<-10 dB and will be <10% at SNR > -5dB.

# Summary

In this paper, RLM switch requirements for NR-U have been discussed. We have the following observations and proposals:

Observation 1: It is not practical to rely on UE detection for the presence of SSB to extend the measurement/evaluation period under low SNR condition. The mis-detection rate would be >10% when the SNR<-5dB.

Observation 2: Advanced UEs could send out OOS indications earlier than the entire OOS evaluation period.

Proposal 1: As minimum requirement, the OOS evaluation period is scaled by a fixed factor of N (N > 1), and it should allow UE to indicate OOS indicate earlier than the entire of OOS evaluation period.

Proposal 2: Not to specify limit the acceptable period of consecutive LBT failures for INS evaluation.

Observation 3: UE would not know whether the CSI-RS is available or not.

Proposal 3: For CSI-RS measurements, UE is only required to measure those CSI-RS in COT where UE already decode the CG-PDCCH successfully.

Proposal 4: For CSI-RS based INS evaluation, UE is only required to measure those CSI-RS in COT where UE already decode the CG-PDCCH successfully. The evaluation period is extended if the corresponding CG-PDCCH is not decoded successfully.

Proposal 5: For CSI-RS based OOS evaluation, the evaluation period is scaled by a fixed factor of N (N > 1).

Observation 4: For single cell scenario, the mis-detection rate for the presence of SSB will be >50% at SNR<-10 dB and will be <10% at SNR > -5dB.

# References

[1] R4-1915777, WF on NR-U RRM Requirements, Ericsson.

# Appendix: Simulation Assumptions

Table A.1: Simulation Parameters

| Parameters | Value |
| --- | --- |
| Deployment | Single cell |
| SCS | 30 kHz |
| SNR range | -15dB ~ 0dB |
| Noc level | -100dBm/15kHz; - 97dBm/30kHz |
| Number of samples (M) | 1 (single sample measurement) |
| Propagation condition | Fading channel (TDL-C 300ns 100Hz, as used in RLM test case) |