3GPP TSG-RAN WG4 Meeting #94-e                                                R4-2000932

# E-meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.1.4.12

Source:	MediaTek Inc.

Title:	Discussion on measurement reporting criteria in NR-U

Document for:		Discussion

- Introduction

In this paper, the view on the reporting for the RSSI/CO measurement per subband is provided.

- Discussion

In RAN4-93 meeting, one remaining issue is the reporting for the measurement per subband as WF [1].

FFS: for UE capable of and configured (if such capability exists) with wideband operation with CCA, 1 report for RSSI and channel occupancy measurements is capable of minimum 1 RSSI measurement and 1 channel occupancy measurement per subband

According to the endorsed parameter list for Rel-16 NR, provided by RAN1 [2], it can be observed that the RSSI measurement is confined within a single LBT bandwidth, for a given ARFCN. Therefore, for one reporting, it is cable to report RSSI/CO measurement of one subband. In other words, to report RSSI/CO measurement of multiple subbands, multiple MOs should be configured.

| Parameter | New? | Description | Value range |
| --- | --- | --- | --- |
| rmtc-measARFCN-r16 | New | Add in RMTC-Config-r16. Measurement center frequency (in ARFCN) for intra- and inter-frequency RSSI measurement. RSSI measurement is confined within a single LBT bandwidth with the center frequency of configured ARFCN | ARFCN |

Observation 1: For one ARFCN, the RSSI measurement is confined within a single LBT bandwidth. Thus, to report RSSI/CO measurement of multiple subbands, multiple MOs should be configured.

Proposal 1: The report for RSSI/CO measurement is per subband, for UE capable and configured with wideband operation with CCA.

- Summary

In this paper, the reporting for RSSI/CO measurement has been discussed. We have the following observations and proposals:

Observation 1: For one ARFCN, the RSSI measurement is confined within a single LBT bandwidth.

Proposal 1: The report for RSSI/CO measurement is per subband, for UE capable and configured with wideband operation with CCA.

References

[1] R4-1915777, WF on NR-U RRM Requirements, Ericsson.

[2] R1-1913674, Updated consolidated parameter list for Rel-16 NR, Qualcomm.