3GPP TSG-RAN WG4 Meeting #94                                                 R4-2000934

# E-meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.1.4.13

Source:	MediaTek Inc.

Title:	Discussion on timing requirements in NR-U

Document for:		Discussion

- Introduction

For the timing requirements in NR-U, one remaining issue [1] is whether UE applies the one shot timing adjustment when UE takes an activated SCell as the new timing reference cell in the case of the Pcell/PSCell is unavailable. In this paper, the corresponding view is provided.

- Discussion

The one shot timing adjustment was motivated by the UE RX beam switching to compensate the BS receive timing jump due to the timing difference between 2 propagation paths received by different UE RX beam. The UE transmits timing immediately after applying the one shot timing adjustment [2] shall be: T2-(NTA+NTA offset)+2T, where T is the timing difference between the reception time at the UE just before the one shot timing adjustment (T1) and the reception time to be used at the UE just after the one shot timing adjustment (T2).

However, there could be a timing difference up to 33 us between PCell and SCell. For example, assuming the reception time from PCell is T1 and the reception time from SCell is T2, T could be up to 33 us. By applying the one shot timing adjustment, it will compensate 2T (e.g. 66 us) in total as round trip time, including DL path and UL path. As a result, it will over-compensate at PCell UL reception, since taking SCell as the timing reference cell will only introduce timing difference T on the DL path and will not further introduce timing difference on the UL path to PCell. Therefore, if UE changed its DL timing reference from PCell to SCell, UE takes gradual UL timing adjustment to compensate the DL timing difference between PCell and SCel.

Observation 1: By applying one shot timing adjustment when SCell is taken as timing reference, it will over-compensate at PCell UL reception.

Proposal 1: The one shot timing adjustment is not applied for the case that UE takes an activated SCell as the new timing reference cell. Gradual UL timing adjustment is assumed.

- Summary

In this paper, it provides the view on the applicability of one shot adjustment in NR-U. We have the following observations and proposals:

Observation 1: By applying one shot timing adjustment when SCell is taken as timing reference, it will over-compensate at PCell UL reception.

Proposal 1: The one shot timing adjustment is not applied for the case that UE takes an activated SCell as the new timing reference cell. Gradual UL timing adjustment is assumed.

References

[1] R4-1915777, WF on NR-U RRM Requirements, Ericsson.

[2] TS38.133, NR; Requirements for support of radio resource management, V15.8.0.