3GPP TSG-RAN WG4 Meeting #94-e	R4-2000946

e-meeting, 24th Feb - 6th Mar, 2020

Agenda item:	8.16.1.2

Source: 		NTT DOCOMO, INC.

Title: 	Definition of intra-frequency measurement for CSI-RS based L3 measurement

Document for:	Discussion

1. Introduction

At the RAN4 #93 meeting, the WF on RRM requirements for CSI-RS based L3 measurement was agreed [1]. Followings are the agreements on definition of the MO configurations [1].

MO configuration

- Case 1: all CSI-RS resources in the same MO have the same center frequency, BW, SCS, CP type configured.

- Case 2: all CSI-RS resources in the same MO have the same center frequency, SCS, CP type configured. BW can be different.

RAN4 will study the RRM requirements at least for Case 1 MO configuration. FFS how to address Case 2.

Note: this does not exclude defining requirements for Case 2.

Also the WF on the definition of CSI-RS based intra-/inter-frequency measurement was agreed as below [1].

- Case 1: CSI-RS resource of serving cell is available

  - CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of CSI-RS on the serving cell and neighbor cell is the same

    - the CP type of CSI-RS on serving cell and target cell is the same

      - It is applied for SCS = 60KHz

    - the center frequency, bandwidth and active BWP

      - Option 1: the center frequency of CSI-RS on the serving cell and neighbor cell is the same, and

      - Option 2: the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of UE, and

      - Option 3: the bandwidth of CSI-RS on the target cell is the same as bandwidth of CSI-RS resources on the serving cell

      - Any combination of options above

  - Note 1: the RAN4 design shall not contradict RAN2 agreements on MO signalling

  - Note 2: the definition of intra-frequency and inter-frequency measurement shall not impact RAN1’s agreement on the definition of the same MO

- Case 2: CSI-RS resource of serving cell is not available

  - Option 1: All MO are inter-frequency

  - Option 2: No requirement is applied

In this contribution, we present our views on which case and option about MO configuration and intra-/inter-frequency measurement of CSI-RS should be adopted from the point of RAN4 RRM and specifications of other WGs.

2. Discussion

2.1	MO configuration

In the RAN1 specification [2], the definition on calculation of CSI-SINR is described as below.

CSI signal-to-noise and interference ratio (CSI-SINR), is defined as the linear average over the power contribution (in [W]) of the resource elements carrying CSI reference signals divided by the linear average of the noise and interference power contribution (in [W]) over the resource elements carrying CSI reference signals reference signals within the same frequency bandwidth.

Described as above, denominator of the CSI-SINR is derived using REs carrying CSI-RS resources. Therefore, if there is difference between measurement BW of the serving cell and that of the target cell, it is difficult to compare these CSI-SINRs, since the level of the interference for each CSI-RS resource also can be different, e.g., frequency-domain scheduling at neighboring cells. According to the contributions from the previous RAN4 meeting, some companies have the concern on case 2 in the WF due to this reason.

Observation 1: The denominator of the CSI-SINR is derived using REs carrying CSI-RS resources. It might be difficult to compare two SINR with different BWs, since measured interference can be different depending on measured frequency.

However, the problem is completely related to implementation. More specifically, if the network configures same center frequency and BW between the two CSI-RSs, it is possible to fairly compare two measured results. Then, we think it is better to avoid unnecessary restriction in the RAN4 specifications. Based on the discussion, we think case 2 should be adopted.

Observation 2: The problem in observation 1 can be avoided by proper NW implementation.

Proposal 1: Following case should be applied for MO configuration.

- Case 2: all CSI-RS resources in the same MO have the same center frequency, SCS, CP type configured. BW can be different.

2.2	Definition of intra-frequency and inter-frequency measurement

Case 1 (CSI-RS resource of serving cell is available)

The below discussion assumes that the SCS and the CP is the same between the serving cell and the neighbor cell as described in [1].  If an active BWP is configured to a UE, the UE is expected to receive physical channels and signals included in the BW. Therefore, if CSI-RS resources which are to be measured are fully included in the active BWP, the UE can measure them without measurement gap because RF retuning is not necessary regardless of the frequency location and BW of them. In this sense, option 2 in the WF should be applied for the condition of intra-frequency measurement.

Observation 3: If CSI-RS resources of the target cells are fully included in active BWP with the same SCS, the UE can receive them without RF retuning regardless of their frequency location.

In [1], it is noted that the RAN4 design shall not contradict RAN2 agreements on MO signaling.  Followings are the CSI-RS requirements about this issue in the RAN2 specification [3].

For CSI-RS resources, the network applies the procedure as follows:

- to ensure that all CSI-RS resources configured in each measurement object have the same center frequency,

(startPRB+floor(nrofPRBs/2))

With option 2 above, there is a case when two CSI-RS resources for L3 measurement with different center frequency. However it doesn’t conflict RAN2 specification, since the CSI-RS resources can be configured using two separate MOs.

Proposal 2: Following option should be applied for condition of intra-frequency measurement.

- Option 2: the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of UE

Observation 4: Option 2 doesn’t contradict RAN2 specification, since CSI-RS resources with different frequency location can be configured with different MOs.

Case 2 (CSI-RS resource of serving cell is not available)

Case 2 assumes that there is no available CSI-RS resource in the active BWP of the serving cell. Under the assumption, it is thought that CSI-RS resources are located in the inter-frequency cell which is configured to measure by MeasObject or that CSI-RS resources are located in the outside of the active BWP. In these case, the UE is required to perform inter-frequency measurement and would need the measurement gap. According to RAN2 specification, since only one measurement gap is configured by the higher layer parameter MeasConfig in the case of per UE gap, such measurement gap would be shared between SSB and CSI-RS as long as new gaps for CSI-RS are not introduced. Even if existing measurement gap patterns are reused, all of CSI-RS resources can be covered since MGRP of existing gap pattern is 20/40/80/160 ms and the repetition period of CSI-RS is 4/5/10/20/40 ms. Therefore, it is relatively easy to specify the requirements of CSI-RS inter-frequency measurement. From the above observations, option 1 can be adopted.

Observation 5: It is relatively easy to specify the requirements of CSI-RS inter-frequency measurement all of CSI-RS resources can be covered by existing gap pattern.

Proposal 3: When CSI-RS is not available in the serving cell, the requirements of inter-frequency measurement should be applied.

3. Conclusion

In this contribution, we present our views on which case and option about MO configuration and intra-/inter-frequency measurement of CSI-RS should be adopted from the point of RAN4 RRM and some specifications of other WGs.

Observation 1: The denominator of the CSI-SINR is derived using REs carrying CSI-RS resources. It might be difficult to compare two SINR with different BWs, since measured interference can be different depending on measured frequency.

Observation 2: The problem in observation 1 can be avoided by proper NW implementation.

Proposal 1: Following case should be applied for MO configuration.

- Case 2: all CSI-RS resources in the same MO have the same center frequency, SCS, CP type configured. BW can be different.

Observation 3: If CSI-RS resources of the target cells are fully included in active BWP with the same SCS, the UE can receive them without RF retuning regardless of their frequency location.

Proposal 2: Following option should be applied for condition of intra-frequency measurement.

- Option 2: the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of UE

Observation 4: Option 2 doesn’t contradict RAN2 specification, since CSI-RS resources with different frequency location can be configured with different MOs.

Observation 5: It is relatively easy to specify the requirements of CSI-RS inter-frequency measurement all of CSI-RS resources can be covered by existing gap pattern.

Proposal 3: When CSI-RS is not available in the serving cell, the requirements of inter-frequency measurement should be applied.

References

[1] R4-1913749, CATT, “WF on CSI-RS based RRM measurements requirements,” Nov. 2019.

[2] TS 38.215.

[3] TS 38.331.

[4] TS 38.133.