3GPP TSG-RAN WG4 Meeting #94-e	R4-2000988

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.6.3.2

Source: 	OPPO

Title: 	On MR-DC Early Measurement reporting

Document for:	Approval

- Introduction

In RAN4#93, a way forward was agreed [1] on MR-DC Early Measurement Reporting. In this documents, we further provide our view on the remaining issues.

- Definition of overlapping carrier

- Number of carriers to measure

- Measurements and reporting for EMR

- Discussion

Based on working assumption on EMR Carriers, two scenarios are considered:

- Case 1: RAN4 EMR requirements for NR carriers while camped on NR only apply if EMR carriers are in CA combination supported by UE. (NR measurements for EMR while camped in NR only)

- Case 2: RAN4 EMR requirements for NR carriers while camped on LTE only apply if UE supports EN-DC and NR EMR carriers are in the EN-DC band combination supported by UE. (inter-RAT NR measurement for EMR while camped in LTE only)

#Issue 1: Definition of overlapping carrier and non-overlapping carrier

RAN4 agreed to adopt support of overlapping and non-overlapping carrier concept. Two options are for down selection:

- Option 1 (LTE definition):

  - An overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements

  - A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements

- Option 2:

  - An overlapping carrier is a carrier which the UE is actively measuring for mobility and EMR.

  - An non-overlapping carrier is a carrier which the UE is actively measuring for EMR only.

NR early measurement should apply to NR and/or inter-RAT LTE carriers. In our understanding, an overlapping carrier is a carrier configured for EMR and for all mobility measurement including intra-RAT and inter-RAT mobility measurements in case for cell reselection. We support Option 2 that an overlapping carrier is a carrier which the UE is actively measuring for mobility and EMR, and a non-overlapping carrier is a carrier which the UE is actively measuring for EMR only.

Proposal 1: Define that an overlapping carrier is a carrier which the UE is actively measuring for mobility and EMR, and a non-overlapping carrier is a carrier which the UE is actively measuring for EMR only.

#Issue 2: Number of carriers to measure for EMR

The maximum number of NR inter-frequency carriers that can be configured for early measurements depends on RAN4. Nevertheless, the maximum number of inter-frequency/inter-RAT carriers that can be included in early measurement reports could be less, which is configured by network and reuses the requirements from LTE euCA defined by RAN2.

RAN4 agreed the UE measurement capability will not change due to EMR. Basically, number of carriers configured for EMR should not exceed UE measurement capability. However, considering overlapping and non-overlapping carriers for EMR, since measurements on overlapping carriers would not increase the total number of actively measured carriers UE monitors, we propose to just limit the number of non-overlapping carriers, and not to restrict the total number of carriers for EMR in RAN4.

Observation 1: Number of carriers configured for EMR should not exceed UE measurement capability.

In LTE rel-15, the following multiplicity and type constraints are defined to limit the size of the different IEs used for configuring the early measurement configuration/results. The UE can be configured with 8 inter-frequency carriers to measure but it can report a maximum of 3 for EMR. There is a need to clarify the maximum number of inter-frequency/inter-RAT carriers that can be configured/reported for early measurements.

|  | Value | Definition | Used in |
| --- | --- | --- | --- |
| maxCellMeasIdle-r15 | 8 | Maximum number of neighboring inter-frequency cells per carrier measured in IDLE mode | CellList-r15 ::= SEQUENCE (SIZE (1.. maxCellMeasIdle-r15)) OF PhysCellIdRange |
| maxFreqIdle-r15 | 8 | Maximum number of carrier frequencies for IDLE mode measurements configured by eNB | EUTRA-CarrierList-r15 ::= SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF MeasIdleCarrierEUTRA-r15 |
| maxIdleMeasCarriers-r15 | 3 | Maximum number of neighboring inter-frequency carriers measured in IDLE mode | MeasResultListIdle-r15 ::= SEQUENCE (SIZE (1..maxIdleMeasCarriers-r15)) OF MeasResultIdle-r15 |

Based on this, for both Case 1 (NR measurements for EMR while camped in NR only) and Case 2 (inter-RAT NR measurement for EMR while camped in LTE only), RAN4 can only define the maximum number of non-overlapping carriers as 3 respectively, to satisfy reporting the results from the carriers most likely to be reported.

Besides, we do not preclude introducing a new capability signalling indicating the maximum number of configured non-overlapping carriers for EMR by RAN2, but such flexibility though useful for some scenarios may lead to specification complexity.

Proposal 2: RAN4 to define the maximum number of non-overlapping carriers only, with the value by 3 for NR inter-frequency and 3 for inter-RAT NR early measurement, respectively.

#Issue 3:  Measurements and reporting for EMR

According to RAN2’s agreements, E-UTRA and NR carrier lists are configured together in either in dedicated or broadcast signaling.

In our understanding, S-measure threshold is assumed not applicable to carriers for EMR, and beyond s-Measure threshold UE measures both EMR and non-EMR carriers. A new threhold seems meaningless, because EMR could be configured by network whether serving cell RxLev is good or good and UE in some scenarios (e.g., low-power or over-heat) is not suggested to do EMR even when Rxlev>threshold. If RAN4 cannot agree on this, we can wait for RAN2’s conclusion to further decide whether to apply threshold on serving cell condition for EMR.

Proposal 3: Do not introduce any threshold on serving cell condition for EMR, otherwise RAN4 can wait and follow RAN2’s conclusion.

The UE starts early measurement only when it was configured with T331 in RRC release message. Upon the expiry of T331 while in IDLE or INACTIVE mode, the UE deletes the dedicated idle mode measurement configuration, if any.

In our view, when T331 expires, it is up to UE implementation whether to continue the early measurements (e.g. based on configuration from SIB). For example, upon intra-RAT/inter-RAT cell reselection, only frequency list in the new SIB which implicitly presents on which frequencies the cell is interested to setup CA/DC, and UE can just keep the necessary measurement results or delete all so that the UE can save the storage and signaling load.

Regarding the measurement configuration and results may be released by UE, the simple way is that UE is not expected to report the results of EMR after the expiry of T331.

Proposal 4: UE is not expected to report the results of EMR after the expiry of T331

- Conclusion

In this contribution, we propose our view on MR-DC EMR as followings:

Proposal 1: Define that an overlapping carrier is a carrier which the UE is actively measuring for mobility and EMR, and a non-overlapping carrier is a carrier which the UE is actively measuring for EMR only.

Observation 1: Number of carriers configured for EMR should not exceed UE measurement capability.

Proposal 2: RAN4 to define the maximum number of non-overlapping carriers only, with the value by 3 for NR inter-frequency and 3 for inter-RAT NR early measurement, respectively.

Proposal 3: Do not introduce any threshold on serving cell condition for EMR, otherwise RAN4 can wait and follow RAN2’s conclusion.

Proposal 4: UE is not expected to report the results of EMR after the expiry of T331.

- References

- R4-1915842,Way forward on MR-DC Early Measurement Reporting, Nokia