3GPP TSG-RAN WG4 Meeting #94-e	R4-2000996

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Source:	OPPO

Title:	On measurement requirements for CSI-RS L3 measurement

Agenda Item:	8.16.1.4

Document for:	Discussion and Decision

Introduction

In RAN #84 meeting, a WID on RRM requirements for CSI-RS based L3 measurement was approved [1].

- Assumptions on Synchronization - Single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements. - Measurement bandwidth of CSI-RS for the minimum requirements - CSI-RS based intra-frequency and inter-frequency definition - Intra-frequency and inter-frequency measurement requirements - Evaluate and specify CSI-RS based L3 measurement accuracy - Introduce new UE measurement capability, including number of frequency layer and number of cells etc., for CSI-RS based measurement

In this contribution, we provide our view for several issues on CSI-RS based measurement requirements.

- CSI-RS and associatedSSB

- The specification structure of CSI-RS L3 measurement related to Gap

- CCSF

- Scheduling restriction

- Scaling factor for RX beam sweeping

Discussion

# Issue 1: CSI-RS and associatedSSB

As we discussed for measurement capabilities of CSI-RS measurement, associatedSSB is optional to configure for CSI-RS resource, and has impact on measurement time requirement, which should be clarified case by case.

For Case 1: associatedSSB is not configured for CSI-RS

If associatedSSB is not configured, the UE may base the timing of the CSI-RS resource on the timing of the serving cell. Network has to guarantee that the serving cell and the neighbour cell. If that is the case, the network should be synchronized, and UE is not required to perform cell identification and can directly measure the signal quality on the CSI-RS resource.

However, on the other hand, if associatedSSB is not configured, UE should measure all configured CSI-RS resources. According to RAN1’ agreements, the max number of CSI-RS resources that can be configured in one MO is 96.  It is not feasible for UE to measure all CSI-RS resources per MO, which would increase the complexity and power consumption of UE. Therefore, RAN4 also can consider not to specify requirements for the case that associatedSSB is not configured.

Proposal 1: If associatedSSB is not configured, two options are to be down selected for CSI-RS L3 measurement.

Option 1: No requirements specified for CSI-RS L3 measurement

Option 2: CSI-RS identification time is the CSI-RS measurement periodicity

For Case 2: associatedSSB is configured for CSI-RS

If a CSI-RS resource is configured with associated SSB, CSI-RS identification time are supposed to include cell research, PBCH decoding, and CSI-RS measurement periodicity.

For cell research, UE should first try to detect the associated SSB before measuring the CSI-RS. The requirement can follow Tpss/sss for SSB-based measurement. If the target cell is the first time to be measured, the AGC adjustment time be considered as well.

For PBCH decoding, UE needs to decode PBCH to acquire the SFN, frame boundary timing, and symbol level timing of the target cell. The requirement can follow the requirement for SIB acquisition in FR2. In FR1, we can skip the Rx beam sweeping factor. Note that this may be skipped if deriveSSB-IndexFromCell is indicated and UE already detects an SSB of the target cell.

For CSI-RS based measurement: consider whether ACG margin is needed duo to flexible measurement BW if any.

Proposal 2: If associated SSB is configured for CSI-RS resources, CSI-RS based cell identification consists: 1) Cell search via SSB, 2) PBCH decoding and 3) CSI-RS measurement.

Proposal 3: Study the impact of AGC control and if any define AGC margin additionally.

# Issue 2: The specification structure of CSI-RS L3 measurement related to Gap

The specification structure for measurement requirements is highly related the definition of intra-frequency and inter-frequency. If the definition of intra-frequency and inter-frequency is finished in this meeting, we can initially discuss the structure of CSI-RS L3 measurement from specification’s perspective. And the requirements for SSB based measurement can be used as a reference.

From our side, we would like to specify more concise requirements of measurement gap for intra-f and inter-f measurement. Based on this, for example, intra-frequency measurement can all be configured without gaps if the definition of intra-frequency includes the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE [2].  In the meantime, all others are inter-frequency measurement with gaps.

Observation 1: From specification’s perspective, the structure of CSI-RS L3 measurement can be more concise than those of SSB.

Proposal 4: Intra-frequency measurement can all be configured without gaps if the definition of intra-frequency includes the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE.  Otherwise, all others are inter-frequency measurement with gaps.

# Issue 3: CCSF

CSSF needs to be considered, either outside gap or within gap. Whether to use CSSF outside gap or within gap is highly related to the conclusion on the definition of intra and inter frequency measurement as well.

Regarding the current SSB-based requirements on deriving CSSF is already very complicated, RAN4 should avoid to create new definition of CSI-RS specific CSSF in this WI and try to re-use the existing requirement as much as possible.

Proposal 5: RAN4 try to re-use the existing requirement of CSSF as much as possible, and the framework of CSSF can be shared by SSB and CSI-RS based L3 measurement.

# Issue 4: Scheduling restriction

Regarding the scheduling restriction was introduced for SSB based measurement without gaps, the similar issues can be considered for CSI-RS L3 measurement [4].

- Collision with UL transmission and DL measurement for TDD band on FR1

- Mix-numerology between PDSCH/PDCCH and SSB symbols on FR1

- Rx beam sweeping and intra-band carrier aggregation on FR2

Since single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements and the SCS of CSI-RS should always be the same as PDCCH and PDSCH, RAN4 just need to consider 1st and 3rd issues for CSI-RS based L3 measurement without gaps.

Observation 2: RAN4 to consider the collision with UL transmission and DL measurement for TDD band on FR1, and Rx beam sweeping and intra-band carrier aggregation on FR2, for scheduling restriction of CSI-RS based L3 measurement without gaps.

Once RAN4 agreed on intra-frequency CSI-RS based L3 measurement without gaps, we can further decide how to define scheduling restriction. Here, two potential options of scheduling restriction can apply:

- Option 1: the scheduling restriction on the additional OFDM symbols before and after SSB is not needed for CSI-RS based L3 measurement.

- Option 2: define scheduling restriction on CSI-RS to be measurement and additional X OFDM symbol before and after consecutive CSI-RS symbols to be measured, FFS on X value.

Proposal 6: If CSI-RS based L3 measurement is configured without gaps, the requirements of scheduling restriction can be chosen from two options:

- Option 1: the scheduling restriction on the additional OFDM symbols before and after SSB is not needed for CSI-RS based L3 measurement.

- Option 2: define scheduling restriction on CSI-RS to be measurement and additional X OFDM symbol before and after consecutive CSI-RS symbols to be measured, FFS on X value.

# Issue 5: Scaling factor for RX beam sweeping

In addition, when CSI-RS resource is configured with associated SSB, and not QCL-ed to the associated SSB, Rx sweeping is needed, and the scaling factor can be reused with FFS on value N.

However, when CSI-RS resource is configured with associated SSB, and QCL-ed to the associated SSB, no Rx sweeping is needed. If this is the case, the scaling factor N due to Rx beam sweeping should not be considered especially for FR2. UE would use the same Rx beam selected for SSB measurement to perform CSI-RS based measurement in FR2.

Proposal 7: If CSI-RS is configured with associated SSB and QCL-ed to the associated SSB, no Rx sweeping is needed, and do not define any scaling factor due to Rx beam sweeping.

Proposal 8: If CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, Rx sweeping is needed and FFS on the scaling factor N.

Conclusion

In this contribution, we propose our view as followings issues.

# Issue 1: CSI-RS and associatedSSB

# Issue 2: The specification structure of CSI-RS L3 measurement related to Gap

# Issue 3: CCSF

# Issue 4: Scheduling restriction

# Issue 5: Scaling factor for RX beam sweeping

Proposal 1: If associatedSSB is not configured, two options are to be down selected for CSI-RS L3 measurement.

Option 1: No requirements specified for CSI-RS L3 measurement

Option 2: CSI-RS identification time is the CSI-RS measurement periodicity

Proposal 2: If associated SSB is configured for CSI-RS resources, CSI-RS based cell identification consists: 1) Cell search via SSB, 2) PBCH decoding and 3) CSI-RS measurement.

Proposal 3: Study the impact of AGC control and if any define AGC margin additionally.

Observation 1: From specification’s perspective, the structure of CSI-RS L3 measurement can be more concise than those of SSB.

Proposal 4: Intra-frequency measurement can all be configured without gaps if the definition of intra-frequency includes the bandwidth of the CSI-RS on the neighbor cell is within the active BWP of the UE.  Otherwise, all others are inter-frequency measurement with gaps.

Proposal 5: RAN4 try to re-use the existing requirement of CSSF as much as possible, and the framework of CSSF can be shared by SSB and CSI-RS based L3 measurement.

Observation 2: RAN4 to consider the collision with UL transmission and DL measurement for TDD band on FR1, and Rx beam sweeping and intra-band carrier aggregation on FR2, for scheduling restriction of CSI-RS based L3 measurement without gaps.

Proposal 6: If CSI-RS based L3 measurement is configured without gaps, the requirements of scheduling restriction can be chosen from two options:

- Option 1: the scheduling restriction on the additional OFDM symbols before and after SSB is not needed for CSI-RS based L3 measurement.

- Option 2: define scheduling restriction on CSI-RS to be measurement and additional X OFDM symbol before and after consecutive CSI-RS symbols to be measured, FFS on X value.

Proposal 7: If CSI-RS is configured with associated SSB and QCL-ed to the associated SSB, no Rx sweeping is needed, and do not define any scaling factor due to Rx beam sweeping.

Proposal 8: If CSI-RS configured with associated SSB but not QCL-ed to the associated SSB, Rx sweeping is needed and FFS on the scaling factor N.

References

- R4-2000995, On measurement capabilities of CSI-RS L3 measurement, OPPO

- R4-2000994, On definition of CSI-RS based intra-frequency and inter-frequency measurements, OPPO

- R4-1913749, WF on RRM requirements for CSI-RS based measurement, CATT

- R4-1913591, Cell identification requirements for CSI-RS RRM, MediaTek Inc.

- R4-1913748, Discussion on CSI-RS based measurement requirements, CATT

- R4-1914437, Discussion on CSI-RS based L3 measurement requirements, Huawei, HiSilicon