3GPP TSG-RAN WG4 Meeting #94-e						                  R4-2000999

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item: 8.8.2.1.3

Source: MediaTek

Title: Discussion on PRS-RSRP measurement

Document for:	Discussion

# Introduction

In RAN4#93 meeting, for PRS-RSRP measurement we have the following agreement which applies to all core requirements:

- Core requirements shall be applicable for:

  - All PRS BW.

  - All comb patterns.

  - All repetition factors.

In this contribution, we provide our views on these topics:

- Definition of intra- and inter-frequency PRS-RSRP measurement

- SINR side conditions for PRS-RSRP measurement

- PRS-RSRP measurement period

- PRS-RSRP report mapping table

# Definition of intra- and inter-frequency PRS-RSRP measurement

In our companion contribution [1], we propose the following definition on intra-frequency PRS-RSTD:

- RAN4 to define intra-frequency RSTD measurement as when the positioning frequency layer associated with a DL PRS resource set is

- the same as that of the DL PRS resource set of the reference cell

- within active BWP

A RSTD measurement is defined to be an inter-frequency RSTD measurement if it is not an intra-frequency RSTD

Also, in [1] we propose that

- Within UE’s active BWP, at most one positioning frequency layer can be configured to the UE

If PRS-RSRP measurement is configured along with RSTD measurement, then whether it is intra-frequency measurement or inter-frequency measurement may directly follow the definition of intra/inter-frequency RSTD measurement.

Proposal 1: For the scenario that PRS-RSRP measurement is configured independently, RAN4 to define intra-frequency PRS-RSRP measurement as when the positioning frequency layer of a DL PRS resource set is within UE’s active BWP.

A PRS-RSRP measurement is defined to be an inter-frequency PRS-RSRP measurement if it is not an intra-frequency PRS-RSRP measurement

If PRS-RSRP measurement is configured independently, we have the following proposal:

# SINR side conditions for PRS-RSRP measurement

In RAN4#92-bis meeting [2], there is note that:

- Side conditions for PRS-RSRP accuracy requirements should be the same as those for RSTD requirements for neighbor cells

However, we note that

- Main use case of PRS-RSRP measurement is for DL-AoD positioning

- In DL-AoD positioning, the focus should be on relative PRS-RSRP accuracy since the AoD estimation is done by making use of the difference between PRS-RSRPs of PRS beams from a TRP

- DL-AoD positioning is mostly likely to be used for indoor office scenario since it requires high LOS probability

- Side conditions for RSTD measurement is set mainly by considering UMa and UMi scenario

Furthermore, we note that in RAN2’s email discussion, the concept of differential PRS-RSRP reporting is supported by several companies (we will provide our view on differential PRS-RSRP in section 5).

Proposal 2:

- No need to define absolute PRS-RSRP accuracy requirements

- RAN4 to define relative accuracy requirements for PRS-RSRP

- RAN4 to discuss side conditions for PRS-RSRP relative accuracy requirement

Base on above observations, we have the following proposals:

If side condition of the PRS-RSRP relative accuracy requirement is set to be Es/Iot ≥-13 dB, then it is expected that accuracy range would be large and hence the requirement would be less meaningful (see our companion distribution [5]). From our SLS simulation results for indoor office as shown below, the side condition of the PRS-RSRP relative accuracy requirement can be set as Es/Iot ≥-6 dB.

Figure 1: Es/Iot for best 6 cells for indoor office scenario, comb-4 PRS, no muting

# PRS-RSRP measurement period

In RAN4#93 WF [3], we have the following agreement:

- If PRS-RSRP is configured to be measured along with RSTD using the same assistance data:

  - then the measurement periods of PRS-RSRP and RSTD are the same.

- Otherwise:

  - In non-DRX the PRS-RSRP measurement period is FFS.

  - When DRX is used then whether or not the PRS-RSRP measurement period depends on DRX cycle may depend on the positioning method; details are FFS.

Proposal 4: Non-DRX requirements apply for PRS-RSRP measurement period, regardless of whether and which DRX configuration is configured for the UE.

Proposal 3: For non-DRX case, UE measures and reports PRS-RSRP for at least n =16 cells within

TRSRP=TPRS⋅M⋅k ms

provided that there are 16 cells satisfying Es/Iot ≥-6dB, where k is the number of positioning frequency layers, M = 16, and 1≤NPRS≤8 is the number of  PRS slots in one PRS instance.

Furthermore, it is assumed that TPRS≥160ms is the same for all positioning frequency layers.

For the case that PRS-RSRP measurement is configured independently, the PRS-RSRP measurement period can be the same as that for RSTD measurement. Also, the positioning measurement has higher priority than other procedure (except handover), thus when DRX is used, the corresponding measurement period should remain the same as that for non-DRX case. Thus, we have the following proposals:

Proposal 5: The requirement on PRS-RSRP measurement period under cell is given by

TRSRP,  HO=TRSRP+K⋅TPRS ms

where K is the number of HO, TPRS≥160ms is the PRS periodicity, and TRSRP is given in proposal 3

The PRS-RSRP measurement period shall be extended based on the number of possibly missed PRS instances due to HOs. Since for one HO, one PRS instance may be dropped by the UE, we have the following proposal:

# PRS-RSRP report mapping table

In RAN4#93 WF [3], we have the following agreement:

- Maximum value of PRS-RSRP report mapping:

  - Reuse the maximum valid value of the existing SS-RSRP report mapping (-44 dBm)

- Minimum value of PRS-RSRP report mapping:

  - Investigate if the minimum value needs to be lower than the minimum value of the existing SS-RSRP report mapping

- Resolution of PRS-RSRP report mapping:

  - Reuse the resolution of the existing SS-RSRP report mapping  (1 dB)

Proposal 6: The minimum value of SS-RSRP reporting table, namely -140 dBm, can be reused for PRS-RSRP reporting

For the minimum value of PRS-RSRP reporting, we have the following proposal:

We’ve conducted SLS simulation for various scenarios to check the minimum value of PRS-RSRP for the best 6 cells (refer to [4] for simulation setting). It is found that the PRS-RSRPs for the 6 best cell is above -140 dbm for 100%UEs. The PRS-RSRP distribution for two selected scenarios are shown below (the PRS configuration is comb-4 PRS and no PRS muting):

Figure 2: The PRS-RSRP distribution for two selected scenarios

In addition, as our argument in section 3, the main use case for PRS-RSRP measurement is for DL-AoD positioning. In DL-AoD positioning, the AoD estimation is done by making use of the difference between PRS-RSRPs of PRS beams from a TRP. Therefore, it is reasonable to support differential PRS-RSRP reporting.

In RAN2 email discussion, differential PRS-RSRP has been taken into consideration in the running CR and it is expected that RAN4 provide analysis on the range and step of differential report.

In TS 38.133, differential SS-RSRP report mapping table is designed with 30 dB as maximum differential range and 2dB as step size. A natural question is whether the table can be also used for PRS-RSRP differential reporting.

In the following we show our simulation results:

Figure 3: DL-AoD positioning performance with PRS-RSRP differential reporting

From the simulation results, it can be observed that there is almost no performance drop by using step size 2dB and max differential range 30dB. Furthermore, if the reporting of (-140…-44)  in steps of 1 dB can be replaced by (0..30) in steps of 2dB, we can save 3bits (= 7 - 4) in each reported beam, which is an reduction of 42% reporting overhead.

Therefore, we have the following proposal:

Proposal 7: Support PRS-RSRP differential report. Reuse SS-RSRP differential reporting design for PRS-RSRP differential report, i.e., 30dB as maximum range and 2dB as step size

# Conclusions

In this contribution, we have the following proposals:

Proposal 1: For the scenario that PRS-RSRP measurement is configured independently, RAN4 to define intra-frequency PRS-RSRP measurement as when the positioning frequency layer of a DL PRS resource set is within UE’s active BWP.

A PRS-RSRP measurement is defined to be an inter-frequency PRS-RSRP measurement if it is not an intra-frequency PRS-RSRP measurement

Proposal 2:

- No need to define absolute PRS-RSRP accuracy requirements

- RAN4 to define relative accuracy requirements for PRS-RSRP

- RAN4 to discuss side conditions for PRS-RSRP relative accuracy requirement

Proposal 3: For non-DRX case, UE measures and reports PRS-RSRP for at least n =16 cells within

TRSRP=TPRS⋅M⋅k ms

provided that there are 16 cells satisfying Es/Iot ≥-6dB, where k is the number of positioning frequency layers, M = 16, and 1≤NPRS≤8 is the number of  PRS slots in one PRS instance.

Furthermore, it is assumed that TPRS≥160ms is the same for all positioning frequency layers.

Proposal 4: Non-DRX requirements apply for PRS-RSRP measurement period, regardless of whether and which DRX configuration is configured for the UE.

Proposal 5: The requirement on PRS-RSRP measurement period under cell is given by

TRSRP,  HO=TRSRP+K⋅TPRS ms

where K is the number of HO, TPRS≥160ms is the PRS periodicity, and TRSRP is given in proposal 3

Proposal 6: The minimum value of SS-RSRP reporting table, namely -140 dBm, can be reused for PRS-RSRP reporting

Proposal 7: Support PRS-RSRP differential report. Reuse SS-RSRP differential reporting design for PRS-RSRP differential report, i.e., 30dB as maximum range and 2dB as step size

# Reference

[1] R4-2000998, Discussion on PRS-RSTD measurement

[2] R4-1912793, NR Positioning AH-2

[3] R4-1915854 WF on NR positioning RRM

[4] R4-1910002

[5] R4-2001001, Link level evaluation on PRS-RSRP and PRS-RSTD