3GPP TSG-RAN WG4 Meeting #94-e              	R4-2001009

Online, 24 February 2020 – 06 March 2020

Title:	Discussion on one shot timing adjustment for UE UL timing adjustment

Source:	NEC

Agenda item:	6.10.7.1

Document for:	Approval

Introduction

In previous RAN4 meetings, UE UL timing adjustment upon UE UL timing change (due to Rx beam change or due to any other general timing shift) is discussed. It was agreed that UE could do gradual and one shot timing adjustment based on the magnitude of timing change. In this contribution, we provide our views on one shot timing adjustment.

Discussion

Timing adjustment behaviour that was agreed and captured in TS 38.133 [2] is shown in Figure.1.

Figure 1: UE UL timing adjustment behavior in terms of UL timing change

Before going to solution, let’s look at the problem origin closely by analysing Rx beam sweeping and problem scenario.

Figure 2: Rx beam sweeping

Rx beam sweeping:

When SMTC window periodicity/SMTC periodicity is 160ms, UE will be measuring each beam (beam 0, 1, etc. at every 160ms, as shown in fig. 1) once in 1280 ms (8*160ms). For example, let’s consider UE’s best Rx beam is the beam in beam 0 direction. UE does Rx beam sweeping every SMTC periodicity (160ms in this case) in each beam direction. Whenever UE finds better beam than serving beam, UE switches to that beam. The process of Rx beam sweeping and Rx beam switching is agnostic to gNB. For example, let’s say, when UE completes one full round of Rx beam sweeping, beam 0 is no longer the best beam and beam 1 is the next best beam. However, UE do not know the latest timing of beam 1, since it was measured 1120ms ago.

Figure 3: Beam path difference

When UE performs Rx beam switch to beam 1, with the timing of 1120ms before, there can be multiple factors contributing to total UE timing error, which is analysed in following sections. We use figure 3, which was agreed in the RAN4#92-Bis to analyse the scenario further.

In the figure 3, path1 corresponds to beam 0 of figure 2 and path 2 corresponds to beam 1 of figure 2.

Let’s represent DL and UL timing of the path “x” as TDLx and TULx and gNB transmit timing can be represented by TgNB.

Path 2 timing 1120ms back can be represented as TDL2’=TgNB+Tprop_path2;

After 1120 ms (at present), the timing of path 2 can be represented as TDL2= TgNB+Tprop_path2+TUE_timing_drift; Where TUE_timing_drift is the drift in UE timing due to clock error/inaccuracy, which can be positive or negative.

Timing of path 1 at present is the latest timing information UE has and it is TDL1= TgNB + Tprop_path1;

By subtracting timing advance (TTA_path1) from DL timing gives us the UE UL timing. Which can be represented asTUL1 = TgNB + Tprop_path1 –TTA_path1

Until gNB sends Timing advance, UE has to use the same timing advance information related to path 1. If there is any small deviation in timing, this old timing advance should be sufficient along with gradual timing adjustment. However, whenever there is a significant timing deviation, same timing advance may cause error at gNB. To avoid this, there needs a mechanism either UE based or network based to correct the UE UL timing as fast as possible, so that UE UL timing drift will not degrade the gNB demodulation performance. One shot timing advance proposed in RAN4 is UE based timing correction mechanism to improve UE UL timing when there is a significant timing shift due to Rx beam switch or any other mechanism.

One shot timing adjustment

If UE knows there is a timing change, and also if UE can figure out the amount of timing change by itself, it can apply that timing change to adjust its UL transmit timing. The main principle followed here is, estimate the new TA and apply that estimated TA to DL timing.

For example, let’s see how UE computes the one shot timing for the scenario shown figure 3.

By assuming TA is the 2×propagation delay, timing advance related to path 2 can be computed as TTA_path1 + 2* path difference.

TTA_path2= TTA_path1 +2 (TDL2-TDL1)

UE UL timing of path 2 is given by DL timing –TA, which can be represented as,

TUL2= TDL2 - TTA_path2

TUL2 = TDL2 - (TTA_path1 + 2 (TDL2-TDL1))

TUL2= TDL2 - TTA_path1 +2 (TDL1-TDL2) …. Equation (1)

Since it is the estimated timing, there can be errors in the timing estimation, which leads to errors at gNB. Let’s look at the corresponding timing errors seen at gNB due to new timing at UE. Timing error corresponding to each of the component of equation (1) is given below.

TDL2  UE transmit timing error seen at gNB is, Te + TUE_timing_drift

Further there may be error in timing advance command resolution received at UE and TA advance adjustment accuracy (according to TS 38.133, table 7.3.2.2-1, the UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement). Hence the error in TTA_path1 may include Timing command resolution error (TACE) and TA adjustment accuracy error (TAAE) as shown in table 1.

Table 1: Timing command resolution and adjustment errors

| UL SCS [kHz] | TACE [Tc] | TAAE [Tc] |
| --- | --- | --- |
| 15 | ± 512 | ± 256 |
| 30 | ± 256 | ± 256 |
| 60 | ± 128 | ± 128 |
| 120 | ± 64 | ± 32 |

And the component “2 × (TDL1-TDL2)” is the time difference due to path difference of beams.

Total error seen at gNB can be written as TBSE = Te +TUE_timing_drift + TACE + TAAE +2× (TDL1-TDL2) … Equation (2)

Te is given in the table of Table 7.1.2-1 of TS 38.133.

TUE_timing_drift can be 252Tc (calculated by assuming UE clock accuracy of 0.1ppm)

TACE and TAAE are given in the above table 1.

We have to decide on 2× (TDL1-TDL2) by deciding on what is the reasonable TBSE so that enough margin is available at BS to deal with multipath propagation errors. As a ballpark number let’s assume TBSE can be upto half CP, so that other half CP can serve as margin for multipath propagation conditions of wireless channel.

TBSE analysis in terms of CP

Table 2: TBSE analysis for different length of path difference

| SSB SCS (kHz) | UL SCS (kHz) | Te +TUE_timing_drift [Tc] | BS error (TBSE) [Tc] | UL CP in [TC] | BS error (TBSE) without path difference as percentage of UL CP length (%) | BS error (TBSE) with adding 2(TDL1-TDL2) as different fraction of CP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 2(TDL1-TDL2)= CP/2 | 2(TDL1-TDL2)= CP/3 | 2(TDL1-TDL2)= CP/5 | 2(TDL1-TDL2)= CP/10 |
| 15 | 15 | 1020 | 1788 | 9216 | 19 | 70 | 53 | 40 | 29 |
|  | 30 | 892 | 1404 | 4608 | 30 | 80 | 64 | 50 | 40 |
|  | 60 | 892 | 1148 | 2304 | 50 | 100 | 83 | 70 | 60 |
| 30 | 15 | 764 | 1532 | 9216 | 17 | 67 | 50 | 37 | 27 |
|  | 30 | 764 | 1276 | 4608 | 28 | 78 | 61 | 47 | 38 |
|  | 60 | 700 | 956 | 2304 | 42 | 92 | 75 | 62 | 51 |
| 120 | 60 | 476 | 732 | 2304 | 32 | 82 | 65 | 52 | 42 |
|  | 120 | 476 | 572 | 1152 | 50 | 100 | 83 | 70 | 60 |
| 240 | 60 | 444 | 700 | 2304 | 30 | 80 | 64 | 50 | 40 |
|  | 120 | 444 | 540 | 1152 | 47 | 97 | 80 | 67 | 57 |

From table 2 we draw few observations as below.

Observation 1: When SMTC window periodicity is 160 ms, for some SCS without any path shift of path difference also, there was around half CP of error seen at gNB. Any error introduced by change of path may cause error at gNB.

From the table 2, it can be observed that till the path difference of CP/5, TBSE is less than 50% for most of the SCS. Only when path difference crosses CP/3, TBSE is above 50% for most of the SCS. Hence, threshold above which one shot timing adjustment may be applied is CP/3.

After UE applying one shot timing adjustment there should not be a case where gNB may have to send TA. If any more margin of error in UE transmit timing is allowed, TBSE will cross half the CP, resulting in necessity to send TA. Hence UE transmit timing error limit should be same as previous Te. That means Te1=Te.

Based on the above analysis we make following proposals.

Proposal 1: UE transmit timing error after one shot timing adjustment shall be within ±Te.

Proposal 2: Threshold for one shot timing adjustment is CP/3.

Proposal 3: If proposal 1 and 2 are not agreeable, then RAN4 should remove one shot timing adjustment requirements from Rel-15.

- Conclusion

In this contribution, we have discussed threshold value for one shot timing adjustment and we make following proposals:

Proposal 1: UE transmit timing error after one shot timing adjustment shall be within ±Te.

Proposal 2: Threshold for one shot timing adjustment is CP/3

Proposal 3: If proposal 1 and 2 are not agreeable, then RAN4 should remove one shot timing adjustment requirements from Rel-15.

References

- RAN4#93 meeting report

- TS 38.133 Requirements for support of radio resource management

- R4-1915947