TSG-RAN Working Group 4 (Radio) meeting #94-e	R4-2001019

Electronic Meeting, February 24th – March 06th 2020

Source:	Ericsson

Title:	Technical background for IAB-Node reference sensitivity

Agenda item:	8.5.4

Document for:	Discussion

# Introduction

At the last RAN4 meeting (RAN4#93 in Reno) technical background information was included in the IAB technical report [1]. Following the WI time schedule, it is time to start to capture technical background information about all requirements allowing specification drafting to start. At last meeting, initial examples of draft specification text were presented in [2]. From the discussion it can be concluded that background information to be captured in the technical report is essential for efficient specification writing.

In this contribution the technical background information relevant for IAB-Node reference sensitivity requirement have been collected for FR1 and FR2.

In companion contributions [3, 4] a text proposal with technical background information relevant for an IAB-Node reference sensitivity requirement have been created for the technical report and specification text for the IAB-Node RF core specification (TS 38.174) have been created.

# Discussion

From a network perspective, the IAB-Node can be seen as a physical identity supporting two main logical identities or sub-functions; IAB-MT and IAB-DU.

The sub-functions can be described as;

- The access link, IAB-DU sub-function is responsible for transmit/receive in the child backhaul link or the access link. Where the downlink/uplink functionally can be exemplified as:

  - Transmit downlink channels (e.g. PDSCH)

  - Receive uplink channels (e.g. PUSCH)

- The backhaul, IAB-MT sub-function is responsible for transmit/receive only in the parent backhaul link. Where the downlink/uplink functionally can be exemplified as:

  - Transmit uplink channels (e.g. PUSCH)

  - Receives downlink channels (e.g. PDSCH)

The receiver sub-function identities can be implemented in one common physical radio design or in two individual radio designs, both complying to a common implementation neutral IAB-Node RF core specification (TS 38.174).

Traditionally, the RF core specifications for UE and BS have been divided into transmitter requirements and receiver requirements. For the IAB-Node, the specification structure needs to be enhanced to account for multiple receiver requirements due to the fact that the IAB-Node receiver operates with two different types of signals. Unlike the BS receiver, the IAB-Node receiver has the capability to receive two types of signals, as visualised in red in Figure 2-1.

# Figure 2-1: The IAB-Node receiver

From the previous discussions regarding reference sensitivity it was concluded to use the NR OTA BS approach as a base line for developing reference sensitivity requirements for the IAB-Node.

The driving factor behind the decision was that the NR OTA BS receiver requirements are designed to include the OTA sensitivity with a declared spatial region for which the BS is intended to operate and provide coverage within. This concept includes the array antenna performance essential for the IAB-Node function. For an IAB-Node the spatial characteristics is vital since the array excitation and consequently the radiation pattern will be different between access link transmissions and backhaul link transmission to optimize coverage.

In the current NR UE specification, the reference sensitivity for FR2 does not capture the array antenna aspect expected for an array antenna providing dynamic coverage for a specific area. The UE reference sensitivity requirement is defined in one direction only. Therefore, it is more suitable to use the NR BS requirement concept where the reference sensitivity is defined within a RoAoA as input for the IAB-Node specification work.

At last meeting, initial examples of draft specification text were presented in [2]. From the discussion it can be concluded that background information needs to be captured in the technical report before the specification can be created.

Regarding the structure of the IAB-Node receiver requirement the following aspects needs some considerations;

- The IAB-Node specification needs to support both FR1 and FR2, hence multiple requirement sets for Conducted, Hybrid and OTA for FR1 and OTA for FR2 needs to be considered.

- The IAB-Node specification needs to support different deployment scenarios (Wide Area, Medium Range and Local Area) in planned networks and unplanned networks. A concept of IAB-Node classes needs to be developed. More details about the IAB-Node class definitions is captured in a companion contribution [5].

- The IAB-Node specification needs to have requirements defined for the access link sensitivity as well as the backhaul link sensitivity, hence an extended set of Fixed Reference Channels (FRC) needs to be supported.

In Table 2-1, the NR specification requirement type overview is visualised.

Table 2-1: Overall NR UE and BS requirement coverage

| Specification | Requirement types |  |  |
| --- | --- | --- | --- |
|  | Conducted | Hybrid | OTA |
| UE | FR1 | Not defined | FR2 |
| BS | FR1 | FR1 | FR1/FR2 |

From table 2-1, it can be concluded that we can create IAB-Node type -H, 1-O and 2-O requirement sets.

For the NR BS RF core specification, the concept of supporting multiple BS types was introduced, as described in Table 2.2.

Table 2-2: NR BS types

| BS type | Description | Additional information |
| --- | --- | --- |
| 1-C | A base station operating within FR1 with all requirements defined at individual antenna connectors | Suitable for deployments with passive base station antennas. This requirement set is not really applicable for an IAB-Node. |
| 1-H | A base station operating within FR1 with a requirement set including requirements defined at the TAB and RIB | Suitable for AAS base stations with physical access to TAB |
| 1-O | A base station operating within FR1 with all requirements defined at the RIB | Supported by base stations where the number of transceiver units are equal to or larger than 8 |
| 2-O | A base station operating within FR2 with all requirements defined at the RIB | Supported by base stations where the number of transceiver units are equal to or larger than 8 |

In general, an IAB-node, type 1-H, type 1-O and 2-O are most relevant since they are designed to capture beamforming aspects.

Based on technical background information in TR 38.817-01 and TR 38.817-02 relevant for BS receiver sensitivity, technical background for the IAB-Node have been created for conducted reference sensitivity requirement in subclause 2.1 and OTA reference sensitivity requirements in subclause 2.2.

  - Conducted reference sensitivity

The conducted IAB-Node reference sensitivity requirement described in this subsection is influenced by the requirement defined for BS type 1-C and BS type 1-H.

The IAB-Node reference sensitivity requirement level is derived in dBm by the following expression:

PREFSENS=10log10kTB+F+IM+SNR		(Eq. 2.1-1)

, where k is Boltzmanns constant equal to 1.38.10-23 J/K, T is the temperature in K, B is the noise bandwidth of the FRC, F is the noise figure, IM is the implementation margin and SNR is the required signal-to-noise ratio.

The temperature was set to 290 K, implementation margin, IM was to 2 dB and the other parameters was set according to table 2.1-1 and table 2.1-2 for IAB-DU.

Table 2.1-1: Noise figure

| IAB-Node Class | Noise figure, F (dB) |
| --- | --- |
| Wide Area | 5 |
| Medium Range | 10 |
| Local Area | 13 |

Table 2.1-2: Required signal-to-noise ratio for 95% throughput

| FRC | Bandwidth, B (MHz) | SNR (dB) |
| --- | --- | --- |
| G-FR1-A1-1 | 5 | -1.2 |
| G-FR1-A1-2 | 5 | -0.8 |
| G-FR1-A1-3 | 10 | -0.9 |
| G-FR1-A1-4 | 20 | -1.1 |
| G-FR1-A1-5 | 20 | -1.2 |
| G-FR1-A1-6 | 20 | -1.1 |

For the IAB-MT, FRC with support for the following parameters ranges as required;

  - SCS 15, 30 and 60 kHzBW 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 MHz depending on supported operating band.

From the above parameters the reference sensitivity requirement can be defined as:

For the IAB-DU the throughput shall be ≥ 95% of the maximum throughput of the FRC as specified in table 2.1-3.

Table 2.1-3:  IAB-DU reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | WA Reference sensitivity power level, PREFSENS (dBm) | MR Reference sensitivity power level, PREFSENS (dBm) | LA Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -101.7 | -96.7 | -93.7 |
| 10, 15 | 30 | G-FR1-A1-2 | -101.8 | -96.8 | -93.8 |
| 10, 15 | 60 | G-FR1-A1-3 | -98.9 | -93.9 | -90.9 |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 | -95.3 | -90.3 | -87.3 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -95.6 | -90.6 | -87.6 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -95.7 | -90.7 | -87.7 |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full channel bandwidth. |  |  |  |  |  |

For the UE the throughput shall be ≥ 95% of the maximum throughput of the FRC as specified in table 2.1-4.

Table 2.1-4: UE reference sensitivity levels

| Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n41 | 15 |  | -94.8 | -93.0 | -91.8 |  | -89.9 | -88.6 | -87.6 |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  | -90.0 | -88.7 | -87.7 | -86.9 |  | -85.6 | -85.1 | -84.7 |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  | -90.1 | -88.9 | -87.8 | -87.1 |  | -85.6 | -85.1 | -84.7 |
| n77 | 15 |  | -95.3 | -93.5 | -92.2 | -91.2 | -90.4 | -89.1 | -88.1 |  |  |  |  |  |
|  | 30 |  | -95.6 | -93.6 | -92.4 | -91.3 | -90.5 | -89.2 | -88.2 | -87.4 | -86.7 | -86.1 | -85.6 | -85.1 |
|  | 60 |  | -96.0 | -93.9 | -92.6 | -91.5 | -90.6 | -89.4 | -88.3 | -87.5 | -86.8 | -86.2 | -85.7 | -85.2 |
| n78 | 15 |  | -95.8 | -94.0 | -92.7 | -91.7 | -90.9 | -89.6 | -88.6 |  |  |  |  |  |
|  | 30 |  | -96.1 | -94.1 | -92.9 | -91.8 | -91 | -89.7 | -88.7 | -87.9 | -87.2 | -86.6 | -86.1 | -85.6 |
|  | 60 |  | -96.5 | -94.4 | -93.1 | -92 | -91.1 | -89.9 | -88.8 | -88.0 | -87.3 | -86.7 | -86.2 | -85.7 |
| n79 | 15 |  |  |  |  |  |  | -89.6 | -88.6 |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  | -89.7 | -88.7 | -87.9 |  | -86.6 |  | -85.6 |
|  | 60 |  |  |  |  |  |  | -89.9 | -88.8 | -88.0 |  | -86.7 |  | -85.7 |

For the IAB-MT subfunction it is suggested to define the reference sensitivity requirement iindependently of frequency band, using the principle used for IAB-DU.

  - OTA reference sensitivity

The OTA reference sensitivity is defined within a spatial angular range referred to as OTA REFSENS RoAoA, where the Range of Angel of Arrival (RoAoA) is determined by the contour defined by the points at which the achieved EIS sensitivity level is 3dB higher than the achieved EIS in the reference direction. In practice the contour will be related to the average element/sub-array radiation pattern 3dB beam-width(s).

    - IAB type 1-O requirement

The conducted IAB-Node reference sensitivity requirement described in this subsection is influenced by the requirement defined for BS type 1-O.

For FR1 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

∆OTAREFSENS=44.1-10log10BeWθ,REFSENS∙BeWφ,REFSENS dB for the reference direction

and

∆OTAREFSENS=41.1-10log10BeWθ,REFSENS∙BeWφ,REFSENS dB for all other directions

The IAB-Node OTA reference sensitivity requirement level is derived in dBm by the following expression:

EISREFSENS=PREFSENS-∆REFSENS		(Eq. 2.2.1-1)

, where PREFSENS is derived for the conducted requirement in subclause 2.1.

From the above parameters the OTA reference sensitivity requirement can be defined as:

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding tables when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 2.2.1: Macro IAB-Node reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA Reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 | PREFSENS - ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | PREFSENS - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | PREFSENS - ΔOTAREFSENS |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 | PREFSENS - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | PREFSENS - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | PREFSENS - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

2.2.2	IAB type 2-O requirement

The OTA sensitivity requirement is defined in an angular region called OTA REFSENS RoAoA. The OTA REFSENS RoAoA defines the angular region over which the EISREFSENS value is fulfilled.

For FR2 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

∆FR2_REFSENS=-3 dB for the reference direction

and

∆FR2_REFSENS=0 dB for all other directions

In order to allow for base station implementations suitable for different deployment scenarios a range of antenna gain is identified for each BS class. This range of antenna gain can then be used along with the implementation margin and SINR requirements to calculate a range of potential minimum EIS values. The vendor declares the OTA reference sensitivity within this range, depending on their view on the needed array size considering the propagation scenarios under which they expect their BS to operate.

The sensitivity level can be expressed in dBm as:

EISREFSENS=10log10kTB+F+IM+SNR-G		(Eq. 2.2.2-1)

, where k is Boltzmanns constant equal to 1.38.10-23 J/K, T is the temperature in K, B is the noise bandwidth of the FRC, F is the noise figure, IM is the implementation margin, SNR is the required signal-to-noise ratio and G is the antenna gain.

The implementation margin, IM was to 2 dB and the other parameters was set according to table 2.2.2-1, table 2.2.2-2 and table 2.2.2-3.

Table 2.2.2-1: Noise figure

| Frequency (GHz) | Noise figure, F (dB) |
| --- | --- |
| 24.25 to 33.40 | 10 |
| 37.00 to 52.60 | 12 |

Table 2.2.2-2: Required signal-to-noise ratio for 95% throughput

| FRC | Bandwidth, B (MHz) | SNR (dB) |
| --- | --- | --- |
| G-FR2-A1-1 | 50 | -1.1 |
| G-FR2-A1-2 | 50 | -1.1 |
| G-FR2-A1-3 | 50 | -1.2 |

Table 2.2.2-3: Antenna gain

| BS Class | Antenna gain, G |  |
| --- | --- | --- |
|  | 30 GHz (dBi) | 45 GHz (dBi) |
| WA | 10 to 33 | 12 to 35 |
| MR | 5 to 28 | 7 to 30 |
| LA | 0 to 23 | 2 to 25 |

From the above parameters the OTA reference sensitivity requirement can be defined as:

The declared reference sensitivity value is per polarization under the assumption of polarization matching. The declared sensitivity applies when all receiver units in the BS per polarization are considered no matter the arrangement of the antenna elements and or antenna panels. As such it is not necessary to defined or discuss antenna panels in the specification. The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHZ BS channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz BS channel bandwidth it does not imply that BS has to support 50MHz BS channel bandwidth.

For Wide Area IAB-Node, EISREFSENS_50M is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For Medium Range IAB-Node, EISREFSENS_50M is an integer value in the range -91 to -114 dBm. The specific value is declared by the vendor.

For Local Area IAB-Node, EISREFSENS_50M is an integer value in the range -86 to -109 dBm. The specific value is declared by the vendor.

Table 2.2.2-4: IAB-DU reference sensitivity requirement

| Channel bandwidth (MHz) | Sub-carrier spacing (kHz) | FRC | EISREFSENS level (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full IAB-Node channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

Table 2.2.2-4: IAB-MT reference sensitivity requirement

| Channel bandwidth (MHz) | Sub-carrier spacing (kHz) | FRC | EISREFSENS level (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200, 400 | 60 | DFT-s-OFDM QPSK | EISREFSENS_50M + ΔFR2_REFSENS |
| 50, 100, 200, 400 | 120 | DFT-s-OFDM QPSK | EISREFSENS_50M + ΔFR2_REFSENS |
| NOTE 1: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

From the above tables it can be noticed that for the UE, the concept of defining FRC is different compared to the BS. For the IAB-Node it is suggested to use the BS concept and extend it to support FRC signals for both IAB-DU and IAB-MT.

# Conclusion

In this contribution the technical background information for BS is captured as reference for the IAB-Node requirement specification work. The reference sensitivity requirement for BS is defined for 1-C, 1-H, 1-O and 2-O. The concepts for 1-C and 1-H is exactly the same, while the concept for 1-O and 2-O are different.

For FR1 IAB-Node it is reasonable to use the requirement concept from 1-H and 1-O, depending on IAB-Node type.

For IAB-Node type 2-O, the BS concept of using a declared EIS met within a declared RoAoA can be adopted for both the DU and MT. Such a requirement would capture the array antenna capability and allow for various types of implementations.

In a companion contribution we have prepared a text proposal to the technical report with background information and in another companion contribution we have prepared a text proposal for TS 38.174 with specification text for the IAB-Node FR2 reference sensitivity requirement.

# References

[1]	R4-1913332, “Updated IAB TR”, Samsung, RAN4#93

[2] 	R4-1914525, “Draft specification text for IAB reference sensitivity in subclause 7.2 and subclause 10.3”, Ericsson, RAN4#93

[3]	R4-2001020, “TP to TR 38.xxx: Addition of background information for FR2 reference sensitivity in subclause 10.3”, Ericsson

[4]	R4-2001021, “TP to TS 38.174: Addition of FR2 IAB reference sensitivity requirement text in clause 3 and clause 10”, Ericsson

[5]	R4-200yzzz, “IAB class definition”, Ericsson