3GPP TSG-RAN WG4 Meeting #94-e                       	 R4- 2001030

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.4.5.5

Source:	MediaTek Inc.

Title:	Discussion on NR V2X unicast, groupcast related requirement

Document for:		Discussion

- Introduction

In RAN4 #92 meeting, a WF about SL RLM and HARQ requirement was approved. RAN4 shall study the UE’s procedure for SL RLM and HARQ.

- SL RLM - Study SL V2X RLM requirements in NR V2X for unicast based on RAN1/2 agreements on RLM metrics. - HARQ - Study potential RRM impact based RAN1 decision

In RAN4 #93 meeting, a WF about NR V2X unicast, group-cast requirement was approved. RAN4 shall discuss whether to introduce distance-based HARQ feedback for groupcast.

- Tx-Rx distance-based HARQ feedback for groupcast - Study to introduce the requirement for distance-based HARQ feedback option 1 - Next meeting, RAN4 should decide whether to introduce the requirement

In this paper, we will discuss these new procedures introduced by group-cast and unicast and whether to introduce related requirements in NR V2X RRM.

- HARQ

Reporting HARQ on sidelink

In legacy LTE sidelink, the retransmission is based on a best effort design. The number of transmissions K for a packet is (pre)configured, since it is often unclear about the channel condition of individual potential receivers, and therefore the worst case is always assumed.

RAN1 #96bis Agreements: - Confirm the following working assumption: - Working assumption: - When HARQ feedback is enabled for groupcast, support (options as identified in RAN1#95): - Option 1: Receiver UE transmits only HARQ NACK - Option 2: Receiver UE transmits HARQ ACK/NACK RAN1 #97 Agreements: - For at least option 1 based TX-RX distance-based HARQ feedback for groupcast, - A UE transmits HARQ feedback for the PSSCH if TX-RX distance is smaller or equal to the communication range requirement. Otherwise, the UE does not transmit HARQ feedback for the PSSCH - TX UE’s location is indicated by SCI associated with the PSSCH. - The TX-RX distance is estimated by RX UE based on its own location and TX UE location. - The used communication range requirement for a PSSCH is known after decoding SCI associated with the PSSCH RAN1 #98 Agreements: - For TX-RX distance-based HARQ feedback for groupcast Option 1, - The location information of TX UE is indicated by the 2nd stage SCI payload RAN1 #98bis Working assumption: - For TX-RX distance-based HARQ feedback for groupcast Option 1, - Zone is (pre-)configured with respect to geographical area, and Zone ID associated with TE UE’s location is indicated by SCI. Agreements: - For the communication range requirement for TX-RX distance-based HARQ feedback, explicit indication in the 2nd stage SCI is used. Working assumption: - For HARQ feedback in groupcast and unicast, when PSFCH resource is (pre-)configured in the resource pool, - SCI explicitly indicates whether HARQ feedback is used or not for the corresponding PSSCH transmission.

In NR sidelink unicast, if the channel conditions between a pair of UEs are good enough, it’s possible that the transmission is successful with less than K (re)transmissions. Hence HARQ feedback mechanism is introduced in sidelink unicast to avoid unnecessary re-transmissions. In NR sidelink groupcast, similar to unicast, the redundant retransmission can be reduced if HARQ-ACK feedback is supported. The difference to unicast is that a group of UEs need to feed back the HARQ-ACKs to the same transmitter. RAN1 had agreed to introduce new physical channel-PSFCH to feedback the ACK/NACK. The SCI information will explicitly indicate whether HARQ feedback is used or not for the corresponding PSSCH transmission.

RAN1 had also agreed two different types of HARQ feedback procedure for groupcast.

- Option 1: Receiver UE transmits only HARQ NACK

- Option 2: Receiver UE transmits HARQ ACK/NACK

For at least option 1 HARQ feedback for groupcast, UE transmits HARQ for the PSSCH if TX-RX distance is smaller or equal to the communication range requirement. Otherwise, the UE does not transmit HARQ feedback for the PSSCH.

Observation 1: RAN1 had introduced the new HARQ procedure on sidelink for groupcast, unicast sidelink communication.

Reporting HARQ on Uu uplink

RAN1 had also agreed the new HARQ feedback on Uu uplink from Tx UE after receiving the HARQ feedback on sidelink. The Tx UE shall use its PUCCH to report the sidelink HARQ based on the configured timing by RRC or DCI to gNB. Different HARQ feedback contents on uplink were defined for unicast, groupcast option1 and groupcast option 2.

RAN1 #97 Agreements: - Sidelink HARQ ACK/NACK report from transmitter UE to gNB is supported with details FFS. RAN1 #98bis Agreements: For reporting SL HARQ-ACK to the gNB: - For dynamic grant and configured grant type-2 in SL, the Rel-15 procedure and signalling for DL HARQ-ACK are reused for the purpose of selecting PUCCH offset/resource and format in UL. - The configuration for SL is separate from Uu link for a UE - FFS how to indicatae timing of transmission in PUCCH, including whether physical or logical slots are used - For configured grant type-1 in SL, RRC is used to configure PUCCH offset/resource and format in UL (if supported) Email approval in [98b-NR-12] Agreements: - For unicast: - TX UE reports contents received in PSFCH (i.e., ACK/NACK) to gNB. - TX UE reports NACK if PSFCH is not detected - When generating the HARQ-ACK report for the transmissions corresponding to a grant, the TX UE uses the most recent PSFCH occasion (as working assumption) associated with the transmissions. - For groupcast option 1: - TX UE reports ACK to the gNB if no PSFCH is detected. - TX UE reports NACK to the gNB if at least one PSFCH (i.e., NACK) is detected. - When generating the HARQ-ACK report for the transmissions corresponding to a grant, the TX UE uses the most recent PSFCH occasion (as a working assumption) associated with the transmissions. - FFS the cases when TX UE does not transmit/receive due to prioritization. - For groupcast option 2: - TX UE reports ACK if all expected PSFCH resources are received and carry ACK. - TX UE reports NACK if at least one received PSFCH resource carries NACK or if no PSFCH is detected. RAN1 #99 Working assumption: - The timing of the PUCCH used for conveying SL HARQ is indicated in DCI or RRC (only for transmissions without a DCI) in terms of PSFCH-to-PUCCH physical slots, where the slot duration is defined based on the PUCCH SCS.

Observation 2: RAN1 had introduced the new HARQ procedure on Uu uplink for groupcast, unicast sidelink communication in NR sidelink mode 1 UE.

Based on the above RAN1 agreement, RAN1 had already defined the detail scheduling timing, and feedback procedure in RAN1’s spec. RAN4 doesn’t need to redefine the same procedure in RAN4 core requirement again, but RAN4 should discuss whether and how to define the Demod test cases to verify these procedures.

Proposal 1: RAN4 doesn’t define core requirement for HARQ feedback on sidelink and HARQ feedback on uplink, but RAN4 can discuss whether and how to define the Demod test cases to verify these procedures.

- RLM

RAN2 had agreed not introduce SL RLM/RLF from Rx UE, but introduce the RLM procedure for unicast sidelink Tx UEs. RAN1 informed that no IS/OOS indication to upper layer from physical layer for sidelink RLM at Tx UE. Currently, RAN1 and RAN2 are discussing how to use HARQ feedback status (i.e., ACK, NACK) to indicate the RLM/RLF for Tx UEs.

RAN1 #99 Agreements: For sidelink RLM/RLF at Tx UE, the usage of HARQ feedback status is feasible from RAN1 perspective when sidelink HARQ is enabled - There is no IS/OOS indication to upper layer from physical layer for sidelink RLM at Tx UE - It is RAN1 understanding that HARQ feedback status (i.e., ACK, NACK) is available in upper layer without additional RLM indication from physical layer - DTX is reported to upper layer for sidelink RLM if RAN2 agree to use it - This doesn’t require RAN1 specification impact RAN1 #108 Agreements: - RAN2 will not introduce SL RLM/RLF from RX UE. - [Email#802]: To discuss if HARQ feedback (HARQ A/N and/or HARQ DTX) based TX-side RLF/RLF is required and if so, how to support in RAN2 specification

Basically, RLM procedure is a key RRM requirement from RAN4’s perspective. RAN4 shall define the related requirement based on agreed new RLM procedure from RAN1 and RAN2. However, considering the limit time schedule in RAN4 R16 sidelink, if RAN1 and RAN2 cannot finalize the procedure for sidelink RLM on Feb.’s meeting, there is no enough time left to RAN4 to discuss the requirement. Thus, our proposal is RAN4 won’t define sidelink RLM requirement on R16 if RAN1/RAN2 cannot finalize the sidelink RLM procedure before April’s meeting.

Proposal 2: RAN4 won’t define sidelink RLM requirement on R16 if RAN1/RAN2 cannot finalize the sidelink RLM procedure before April’s meeting.

- Summary

In this paper, we give our views on group-cast, unicast related requirement discussion.

Observation 1: RAN1 had introduced the new HARQ procedure on sidelink for groupcast, unicast sidelink communication.

Observation 2: RAN1 had introduced the new HARQ procedure on Uu uplink for groupcast, unicast sidelink communication in NR sidelink mode 1 UE.

Proposal 1: RAN4 doesn’t define core requirement for HARQ feedback on sidelink and HARQ feedback on uplink, but RAN4 can discuss whether and how to define the Demod test cases to verify these procedures.

Proposal 2: RAN4 won’t define sidelink RLM requirement on R16 if RAN1/RAN2 cannot finalize the sidelink RLM procedure before April’s meeting.