3GPP TSG-WG RAN4 Meeting #94-e	R4-2001045

# Online, 24th February – 6th March, 2020

Source: 		Nokia, Nokia, Shanghai Bell, BT plc

Title: 			TP to TR 37.716-21-11: CA_3-20_n7

Agenda Item: 		9.4.2

Document for:		Approval

# Introduction

This TP introduces the following EN-DC configurations for 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) .

- DL_3A-20A_n7A_UL_3A_n7A

- DL_3A-20A_n7A_UL_20A_n7A

- DL_3C-20A_n7A_UL_3A_n7A

- DL_3C-20A_n7A_UL_20A_n7A

- DL_3C-20A_n7A_UL_3C_n7A

## TP to TR 37.716-21-11

## 5.1.X	DC_3-20_n7

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-20_n7 | CA_3-20 | n7 | no |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A | CA_3A-20A CA_3C-20A | n7 |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_3-20_n7, the TIB,c and RIB,c values are reused from the LTE combination CA_3-7-20 given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n7 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n7 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n7 | 3 | 0 |
|  | 20 | 0 |
|  | n7 | 0 |

### 5.1.X.4	REFSENS requirements

There are IMD2 impacts from DC_3_n7 UL to Band 20 DL. The MSD value for band 20 DL is reused from LTE CA_3-7-20 in TS 36.101.

Table 5.1.X.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 | 5 | 25 | 1832 | N/A | FDD | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | FDD | IMD2 |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | FDD | N/A |