3GPP TSG-RAN WG4 Meeting #94-e	R4-2001127

Online, 24 Feb - 6 Mar 2020

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.823: Specific aspects for TDD

Agenda Item:	7.10.2

Document for:	Approval

# 1	Introduction

In the RAN4#92 meeting, a WF for Rel-16 LTE-MTC coexistence with NR is agreed [1], where it specifies that the TDD specific issues should be captured in the TR. This paper proposes the TP for this topic.

# 2	References

[1] R4-1907849, WF for Rel-16 LTE-MTC coexistence with NR

# 3	Text Proposal

## <Start of Text Proposal>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-190770, “Revised WID for Additional MTC enhancements for LTE”

[3]		     3GPP TS 36.211, Physical channels and modulation, v13.2.0

[4]                        R1-1903885, Coexistence of LTE-MTC with NR, Ericsson, RAN1#96bis

## <Skip Unchanged Parts>

## 8.4 Specific aspects for TDD

LTE-MTC supports all E-UTRA TDD configurations as specified in 3GPP TS 36.211. In the meantime, NR TDD configurations are very flexible. Not only slots but also individual symbols within a slot can be configured as either DL, UL or flexible (i.e. GP). This makes it possible to match the NR TDD configuration to that of LTE-MTC.

More explicitly, two different patterns may be configured via the IE TDD-UL-DL-ConfigCommon in SIB1. Furthermore, the common settings may be overridden by UE specific configuration: TDD-UL-DL-ConfigDedicated. Table 8.4-1 shows the details of the RRC signalling.

Table 8.4-1: RRC Signaling for NR TDD Configuration

| Field Name | Note |
| --- | --- |
| TDD-UL-DL-ConfigCommon: | Signalled by SIB1 |
| referenceSubcarrierSpacing | Subcarrier spacing in kHz: 15, 30, 60, etc |
| dl-UL-TransmissionPeriodicity | Pattern periodicity in ms |
| nrofDownlinkSlots | Can be used to configure DL transmissions that matches the duration of DL subframes or forms part of the DwPTS in LTE-MTC |
| nrofDownlinkSymbols | Specifies a partial DL slot, together with other DL slots (if needed), can match the duration of DwPTS in LTE-MTC |
| nrofUplinkSymbols | Specifies a partial UL slot, together with other UL slots (if needed), can match the duration of UpPTS in LTE-MTC |
| nrofUplinkSlots | Can be used to configure UL transmissions that matches the duration of UL subframes or forms part of the UpPTS in LTE-MTC |
| TDD-UL-DL-ConfigDedicated: | UE specific RRC signalling |
| slotIndex | Depending on the SCS, the duration of a NR slot may be less (i.e. ½, ¼, etc) or equal to a subframe in LTE-MTC |
| allDownlink | The whole slot is for DL, more than one DL slot may be needed to match the duration of a DL subframe in LTE-MTC |
| allUplink | The whole slot is for UL, more than one UL slot may be needed to match the duration of a UL subframe in LTE-MTC |
| nrofDownlinkSymbols | Specifies a partial DL slot, together with other DL slots (if needed), can match the duration of DwPTS in LTE-MTC |
| nrofUplinkSymbols | Specifies a partial UL slot, together with other UL slots (if needed), can match the duration of UpPTS in LTE-MTC |

## The main limitation of the NR TDD configuration is that the pattern has to be defined in the order of DL, GP and UL. A combination use of TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated shall enable the NR TDD configuration to match that of any LTE UL-DL configuration. For example, Table 8.4-2 shows the NR TDD configuration that matches the LTE-MTC uplink/downlink 1 with the special subframe configuration 7 (i.e. uplink/downlink subframe ratio is 1:2, DwPTS = 10 symbols, GP = 2 symbols, UpPTS = 2 symbols).

Table 8.4-2: Example of NR TDD configuration which matches with LTE TDD UL/DL configuration 1 and special subframe configuration 7

| NRTDD |  | LTE TDD |
| --- | --- | --- |
| Tdd-UL-DL-Configuration |  |  |
| referenceSubcarrierSpacing | 15 |  |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 |  |
| nrofDownlinkSlots | 1 | Subframe 0 is DL |
| nrofDownlinkSymbols | 0 |  |
| nrofUplinkSlots | 0 |  |
| nrofUplinkSymbols | 0 |  |
| Tdd-UL-DL-ConfigDedicated |  |  |
| slotIndex | 1 | Subframe 1 is special subframe |
| nrofDownlinkSymbols | 10 | DwPTS=10 |
| nrofUplinkSymbols | 2 | UpPTS=2 |
| slotIndex | 2,3 | Subframe 2,3 |
| symbols | allUplink | UL |
| slotIndex | 4 | Subframe 4 |
| symbols | allDownlink | DL |

## <End of Text Proposal>