3GPP TSG RAN WG4 Meeting #94-e	R4-2001188

Online, 24th Feb. – 06th March 2020

Agenda item:	9.15.2

Source:	LG Electronics

Title:	MSD test results for Power Class 2 UE for EN-DC (1 LTE FDD band +1 NR TDD band)

Document for:	Approval

1	Introduction

In the latest TR37.815, RAN4 decribed as follow [1].

Table 8.1-1: Required changes in TS 38.101-3

| Clause | Requirement | Required changes in TS 38.101-3 compared to the same band combination for power class 3 |
| --- | --- | --- |
| 6.2B.1 | UE maximum output power for EN-DC | Add power class 2 for inter-band EN-DC. The additional details of power class for LTE and NR may be also needed |
| 6.2B.4.1.3 | Configured output power for inter-band EN-DC within FR1 | It shall be verified whether the existing Pcmax for inter-band EN-DC can be applied to HP UE. Some changes shall be needed to address SAR issue. |
| 7.3B.2.3.1 | Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1 | No changes for case 1 No changes for case 2 considering the harmonic product of TDD band (high band) would not be possible to affect the lower FDD band. |
| 7.3B.2.3.2 | MSD due to receiver harmonic mixing for EN-DC in NR FR1 | No changes for case 1 MSD value may need to be re-evaluated for case 2 |
| 7.3B.2.3.3 | Reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1 | No changes for case 1 MSD value may need to be re-evaluated for case 2 |
| 7.3B.2.3.4 | Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1 | No changes for case 1 MSD value may need to be re-evaluated for case 2 |
| 7.3B.2.3.5 | Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 | MSD value may need to be re-evaluated for case 1 and case 2 |

Based on the specification impact, we analyse the required MSD levels for PC2 UE for EN-DC_3A_n78A.

2	MSD analysis

Table 1 show the RF component isolation parameters to derive MSD level at sub-6GHz.

Table 1: UE RF Front-end component parameters

| UE ref. architecture Component | Triplexer-Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- |
|  | PC2 UE to support DC_3A_n78A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |
| Quadplexer |  |  |  |  |
| Diplexer | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 |

In here, we assumed equal power backoff for both LTE CC and NR CC. So the Case1 and Case2 is not meaningful for the MSD analysis. Case1 and Case2 as shown in below [1]. Baseline RF architecture is Case1.

- Case1: LTE FDD PC3 (23dBm) + NR TDD PC3 (23dBm) and EN-DC PC2 (26dBm)

- Case2: LTE FDD PC3 (23dBm) + NR TDD PC2 (26dBm) and EN-DC PC2 (26dBm)

Table 2 show the isolation levels according to the RF component.

Table 2: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

The existing MSD levels in TS38.101-3 show in Table 3.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A-SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
|  |  |  |  |  |  | 28.74 |  |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A-SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |

Based on above assumptions and test configuration, we proposed the MSD levels as below.

Table 4: Proposed MSD test configuration and results by IMD problems

| DC bands | UL DC | IMD | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n78A | 3 | IMD2 ¦fB3 -fn78¦ | 1740 | 5 | 25 | 1835 | 5 | 32.0 |
|  | n78 |  | 3575 | 10 | 50 | 3575 | 10 | N/A |
|  | 3 | IMD4 ¦3*fB3 -fn78¦ | 1765 | 5 | 25 | 1860 | 5 | 17.5 |
|  | n78 |  | 3435 | 10 | 50 | 3435 | 10 | N/A |

3	Conclusion

Above simulation results, we recommend to merged the other interested companies’ simulation results as average manner to derive MSD levels and propose as follow

Proposal 1: For PC2 DC_3A_n78A UE, the proposed MSD levels in Table4 shall be considered in TS38.101-3.