3GPP TSG-RAN WG4 Meeting #94-e	R4-2001190

Electronic Meeting, February 24 – March 6, 2020

Agenda item:	8.12.3	UE RF core requirements (38.101-2)	[NR_DL256QAM_FR2]

Source: 		NTT DOCOMO, INC.

Title: 	NR UE maximum input level for FR2 DL 256QAM

Document for:	Approval

- Introduction

In RAN4#93, UE core requirements for FR2 DL 256QAM was discussed and proposed to be introduced in [1]. However, there was no agreement on whether to introduce the requirements or not. In this contribution, we provide our views on UE core requirements for FR2 DL 256QAM.

2.	Discussion

2.1	Previous agreement

In the last meeting, the following agreement was made in the chairman’s note [2]:

Agreement:

In 256QAM WI, BS Tx EVM will be defined as core requirements

Whether to introduce the UE maximum input level as core requirements will be further discussed.

In 256QAM WI, UE demod performance requirements and testability will be discussed in the performance part of this WI.

2.2	UE maximum input level

UE maximum input level requirement for FR2 DL 256QAM was proposed in [1], however no agreement was reached. This section clarifies the need for the requirement and proposes to introduce it.

We would like to clarify the motivation for introducing this core requirements from the following perspectives:

- WID perspective

- Testability perspective

- Performance perspective

Firstly, it has already been agreed to introduce UE core requirements in the WID on FR2 DL 256QAM [3]. The objective of core part WI is copied and pasted below.

4.1	Objective of SI or Core part WI or Testing part WI

The objectives of the core part are as follows:

Phase 1:  Continue and complete the feasibility and performance benefit study to identify applicable scenarios

- Both system and link simulations as well as RF and baseband implementation need to be considered in the Rel-16 evaluation to study the benefits of FR2 DL 256QAM.

- This phase is planned from RAN4#91 and should end by RAN4#93.

Phase 2:  Specify BS and UE requirements for NR DL 256QAM for FR2 if applicable scenarios and feasible requirements are identified

- The requirements for BS include Tx modulation quality, Tx power dynamic range etc.

- The requirements for UE include Rx maximum input power level, FRC etc.

As the above, it was clearly mentioned that UE core requirements including maximum input level will be specified if applicable scenarios and feasible requirements are identified in Phase 1. The feasibility has been identified and the conclusion was incorporated into TR38.883 [4] as below.

5.4	Conclusion

The feasibility of FR2 DL 256QAM has been evaluated, including link level simulation, system level simulation and implementation study. Based on the study output, it can be concluded that FR2 DL 256QAM is feasible and beneficial as concluded in section 5.2.1.10, 5.2.2.6 and 5.3, and corresponding requirements shall be discussed and specified afterwards.

From the above discussion, UE core requirements should be introduced to comply the objective described in the WID.

Secondary, the testability issues shall be decoupled from whether to introduce core requirements or not. Basically, the core requirements are used for product design targets and do not have to be testable. In fact, some non-testable requirements have been introduced in FR2 so far. For example, the testability of maximum input level requirement for QPSK has not yet been confirmed, however it has already been specified as a core requirement regardless of testability. Hence, the situation of the requirements for 256QAM is the same as for QPSK, and core requirements for 256QAM can be introduced as for QPSK.

Finally, from a performance perspective, the requirements are necessary to guarantee receiver dynamic range performance. In general, OFDM signals with a high modulation order, such as 256QAM, have a high peak-to-average power ratio (PAPR). In addition, the required SNR for a 256QAM modulation order is higher than for a lower modulation order. Taking into account such characteristics of 256QAM modulation, it is important to guarantee high PAPR signal performance at the maximum input level of the receiver dynamic range. It would be noted that the receiver dynamic range performance cannot be guaranteed by UE demodulation requirement, since the desired signal power specified for UE demodulation is lower than the maximum level of the receiver dynamic range. Thus, if there is no UE core requirement, it has a risk that the receiver performance will be degraded since the product design does not consider the receiver dynamic range for 256QAM.

From the above reasons, UE maximum input level requirements are very important and necessary to make sure the performance of DL 256QAM. Therefore, we propose to introduce the UE maximum input level for FR2 DL 256QAM.

Proposal: Introduce UE maximum input level core requirements for FR2 DL 256QAM.

3.	Conclusion

In this contribution, we discuss on the motivation of the UE core requirement for FR2 DL 256QAM. The following proposal is obtained.

Proposal: Introduce UE maximum input level core requirements for FR2 DL 256QAM.

References

- R4-1913904, “UE RF requirements for FR2 DL 256QAM”, China Telecom, RAN4 #93, November 2019

- RAN4 #93 Meeting report

- RP-191891, “Revised WID: Add support of NR DL 256QAM for FR2”, China Telecom, RAN #86, September 2019

- TR 38.883 v1.0.0: Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2) , MCC, November 2019