3GPP TSG-RAN WG4 Meeting #94-e	R4-2001192

Electronic Meeting, February 24 – March 6, 2020

Agenda item:	9.16.2	[NR_n259-Core]

Source: 		NTT DOCOMO, INC.

Title: 	Additional unwanted emission requirements for the EESS protection from Band n259

Document for:	Approval

- Introduction

In ITU WRC-19, the unwanted emission requirements for the Earth exploration-satellite service (EESS) (passive) in the frequency band 36-37GHz was discussed.　As described in page 394 – 397 in [1], it was concluded that IMT base stations operating in the frequency range of 37-40.5GHz shall meet -43dBW/MHz and -23dBW/GHz in order to protect the system.

In this contribution, we discuss on how to incorporate the above conclusion into the BS RF specifications.

2.	Discussion

2.1.	Summary of the EESS protection in ITU WRC-19

The unwanted emission limits for the EESS (passive) protection in the frequency band 36-37 GHz was concluded as follows:

According to TABLE 1 in [1], a limit of -43 dBW/MHz and -23 dBW/GHz apply for IMT base station with the “frequency band for IMT stations” (i.e., 37-40.5 GHz). Also a metric is defined by total radiated power (TRP) which is the same as the unwanted emission requirements defined in TS 38.104/141-2.

2.2.	Requirement for the EESS (passive) protection

Figure 1 shows the relationship among the EESS (passive) band and NR FR2 operating bands.

Figure 1: The relationship among the EESS (passive) band and NR operating bands

As mentioned in section 2.1, the unwanted emission requirement for the EESS (passive) protection may apply to BS operating within the “Frequency band for IMT station”. As shown in Figure 1, band n259 and n260 overlap with the “Frequency band for IMT station”. Hence band n259 and n260 can be considered for the new requirement.

In addition, for band-specific BS RF requirement, it is appropriate to introduce additional operating band unwanted emissions (OBUE) requirements. Thus, it is proposed to introduce new additional OBUE requirements for the EESS (passive) protection. It would be noted that the new requirements may apply in the frequency range 36 – 37 GHz even though part of the range falls in the spurious domain of band n259.

Proposal 1: Introduce new additional OBUE requirements for the EESS (passive) (36 – 37 GHz) protection.

For BSs supporting band n259, three typical overlapping cases can be considered as shown in Figure 2.

Case 1 (BS_A):	Completely-overlapped (BS supporting a frequency completely overlapped with “Frequency band for IMT stations”)

Case 2 (BS_B):	Partially-overlapped (BS supporting a frequency partially overlapped with “Frequency band for IMT stations”)

Case 3 (BS_C):	Non-overlapped (BS supporting a frequency not overlapped with “Frequency band for IMT stations”)

Figure 3: Relationship between“Frequency band for IMT station” and band n259

From an RF perspective, emission levels decrease with frequency offset from the edge of the operating band. Therefore, it is reasonable to interpret that the requirement may apply to BSs that support a frequency range that partially or completely overlaps with 37 - 40.5GHz. This means that the requirement for EESS (passive) protection applies to Case 1 and 2, but not to Case 3.

Proposal 2: The requirement for the EESS (passive) protection applies to BSs that support a frequency range that partially or completely overlaps with“Frequency band for IMT station” (i.e., 37 – 40.5 GHz).

Finally, the following limits can be considered in 3GPP specifications:

- -13 dBm/MHz  (= -43dBW/MHz)  and +7 dBm/GHz (= -23dBW/GHz)

Proposal 3: Define both limits of -13dBm/MHz and +7dBm/GHz in the frequency range 36 -37 GHz for the EESS (passive) protection.

Based on above discussion, the new additional OBUE requirements for the EESS (passive) protection are proposed as Table 1.

Table 1: Proposed additional operating band unwanted emissions basic limits for protection of EESS (passive)

| Frequency range | Basic limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 36 – 37 GHz | -13dBm | MHz | Applicable to NR BS operating in Band n259, with supported frequency range partially or completely overlaps with 37-40.5 GHz. The requirement applies even though part of the range falls in the spurious domain. |
|  | +7dBm | GHz |  |

3.	Conclusion

In this contribution, the approach to incorporate the EESS protection applicable to Band n259. The following proposals are obtained.

Proposal 1: Introduce new additional OBUE requirements for the EESS (passive) (36 – 37 GHz) protection.

Proposal 2: The requirement for the EESS (passive) protection applies to BSs that support a frequency range that partially or completely overlaps with“Frequency band for IMT station” (i.e., 37 – 40.5 GHz).

Proposal 3: Define both limits of -13dBm/MHz and +7dBm/GHz in the frequency range 36 -37 GHz for the EESS (passive) protection.

Table 1: Proposed additional operating band unwanted emissions basic limits for protection of EESS (passive)

| Frequency range | Basic limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 36 – 37 GHz | -13dBm | MHz | Applicable to NR BS operating in Band n259, with supported frequency range partially or completely overlaps with 37-40.5 GHz. The requirement applies even though part of the range falls in the spurious domain. |
|  | +7dBm | GHz |  |

References

- World Radiocommunication Conference 2019 (WRC-19) Provisional Final Acts (https://www.itu.int/en/ITU-R/conferences/wrc/2019/Documents/PFA-WRC19-E.pdf)