3GPP TSG-RAN WG4 Meeting #94-e	R4-2001259

Online, 24 February – 6 March, 2020

Source:	ZTE

Title:	Remaining issues on NR reporting criteria for EN-DC and NE-DC

Agenda item:	6.10.3

Document for:	Discussion

1. Introduction

The requirements for NE-DC reporting criteria has not been finalized. Moreover there is no reporting criteria in TS 36.133 for EN-DC and NE-DC when E-UTRA SCell(s) are configured. In the RAN4#93 meeting, there was discussions [1] and CR [2] to introduce reporting criteria requirements for EN-DC and NE-DC when E-UTRA SCell(s) are configured. However one company had concern on the structure of the reporting criteria requirements in TS 36.133, so the CR was not agreed.

In this contribution, we further provide our views on remaining issues on reporting criteria for EN-DC and NE-DC.

2. Discussion

2.1. EN-DC reporting criteria

There is no reporting criteria in TS 36.133 for EN-DC when E-UTRA SCell(s) are configured. It is not clear what the E-UTRA reporting criteria would be when SCell(s) is configured. It is necessary to add corresponding requirements for this scenario.

In the last meeting, during online discussion operators show high interests to introduce reporting criteria requirements when CA is configured at E-UTRA side. Companies also agreed to add corresponding requirements.

R4-1914769 Remaining issues on NR reporting criteria CR- rev Cat: (Rel-15) v Source: ZTE Abstract: Proposal 1. The reporting criteria requirements for EN-DC in TS36.133 should be revised by removing Proposal 1. Measurements on NR serving cells should be removed from TS36.133 and clarification that Ecat,EN-DC,NR can be configured by E-UTRA PCell and and PSCell should be added in TS38.133. Proposal 2. The E-UTRA reporting criteria for EN-DC when E-UTRA SCell(s) are configured is necessary to be specified. Proposal 3. Reporting criteria for EN-DC is 36+9*n when the UE is configured with E-UTRA SCell(s), and n is the number of E-UTRA SCell carrier frequencies. Proposal 4. For NE-DC, the total number of E-UTRA reporting criteria is Ecat,NE-DC,E-UTRA=10+9×n, and is the number of configured E-UTRA serving frequencies, including PSCell and SCells carrier frequencies. Discussion: E///: Can agree on proposal 2. Other proposals are not agreeable. Total number of reporting cells should cover all NR and LTE cells. Nokia: Smth needs to be corrected and need to have further discussion how to do it. DCM: Support proposal 2 and 3. CMCC: Support proposal 2 and 3 ZTE: We bring proposals for 3 meetings Decision: Noted

Proposal 1. The reporting criteria for EN-DC when E-UTRA SCell(s) are configured is to be specified.

Proposal 2. The reporting criteria for NE-DC when E-UTRA SCell(s) are configured is to be specified.

The concern was on how the requirements were specified. It was argued that similar to TS38.133 the total number of reporting criteria for EN-DC, including reporting criteria for all the measurements configured by both E-UTRA PCell and NR PSCell, should be specified in TS36.133 by adding the number of reporting criteria specified in TS38.133 onto the number of reporting criteria specified in current TS36.133. We don’t think this is the correct approach. Even in TS38.133 the reporting criteria are specified for NR and E-UTRA separately, rather than adding up together.

Moreover in current TS36.133, what specified in TS 38.133 is already included in the total number of reporting criteria specified in TS38.133 by taking the highlighted sentence into account. Maybe ‘excluding’ is not very accurate. In our view it may be better changing to ‘In addition to’.

The UE capable of supporting EN-DC operation with NR PSCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, inter-RAT per supported RAT, and NR cells on non-serving carrier frequencies (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the number of reporting criteria, excluding reporting criteria specified in TS 38.133 [50] that are applicable for the UE configured with EN-DC operation, as follows:

Proposal 3. The requirements structure for reporting criteria in TS36.133 is not changed by introducing requirements for CA at E-UTRA side.

When CA is configured at E-UTRA side, similar requirements as in TS 38.133 for serving carrier frequencies, i.e. total number of reporting criteria is scaled by the number of serving carrier frequencies, can be specified.

Proposal 4. Reporting criteria for EN-DC is 36+9*n when the UE is configured with E-UTRA SCell(s), and n is the number of E-UTRA SCell carrier frequencies.

2.1. NE-DC reporting criteria

The structure of reporting criteria requirements for NE-DC is quite similar to EN-DC. The reporting criteria is separated for measurement categories on NR carrier frequencies and E-UTRA carrier frequencies. Further reporting criteria for E-UTRA carrier frequencies is separated for serving carrier frequencies and non-serving carrier frequencies.

- For UE configured with NE-DC: Ecat,NE-DC,NR+Ecat,NE-DC,E-UTRA, where Ecat,NE-DC,NR=10+9×n is the total number of NR reporting criteria according to Table 9.1.4.2-1, and is the number of configured NR serving frequencies, including PCell and SCells carrier frequencies, Ecat,NE-DC,E-UTRA=Ecat,NE-DC,E-UTRA,inter-RAT+Ecat,NE-DC,E-UTRA,intra-RAT, where Ecat,NE-DC,E-UTRA,inter-RAT is the total number of inter-RAT E-UTRA reporting criteria configured by PCell except E-UTRA PSCell and E-UTRA SCell carrier frequencies, according to Table 9.1.4.2-1, Ecat,NE-DC,E-UTRA,intra-RAT is the total number of E-UTRA reporting criteria including E-UTRA PSCell and SCell carrier frequencies as specified in TS 36.133 [15] for UE configured with NE-DC.

Since measurements on NR carrier frequencies can only be configured by NR PCell, the current reporting criteria for measurements on NR carrier frequencies is fine when UE is in NE-DC operation. For reporting criteria for measurements on E-UTRA carrier frequencies, the number is not specified yet.

The UE capable of supporting and configured with NE-DC operation with PSCell and NR PCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells and E-UTRA inter-frequency cells, inter-RAT per supported RAT, and NR cells on serving and non-serving carrier frequencies (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the number of reporting criteria, excluding reporting criteria specified in TS 38.133 [50] that are applicable for the UE configured with NE-DC operation, as follows: - [TBD] reporting criteria if the UE is not configured with any SCell or NR SCell.

Regarding the number of reporting criteria for E-UTRA carriers specified in TS36.133 we propose to use similar approach as in TS 38.133 for NR carriers which makes the spec more consistently. The requirements should be specified for both single carrier and CA case.

For single carrier case, the number of reporting criteria should be 19 by taking inter frequency and intra frequency E-UTRA measurements into consideration. When CA is configured the number of reporting criteria is scaled by number of serving frequencies, similar to EN-DC case.

Proposal 5. For NE-DC, the total number of E-UTRA reporting criteria is Ecat,NE-DC,E-UTRA=10+9×n, and  is the number of configured E-UTRA serving frequencies, including PSCell and SCells carrier frequencies.

3. Conclusion

In this contribution, we provide our views on remaining issues on reporting criteria for NR. Based on the observations following proposals are present.

Proposal 1. The reporting criteria for EN-DC when E-UTRA SCell(s) are configured is to be specified.

Proposal 2. The reporting criteria for NE-DC when E-UTRA SCell(s) are configured is to be specified.

Proposal 3. The requirements structure for reporting criteria in TS36.133 is not changed by introducing requirements for CA at E-UTRA side.

Proposal 4. Reporting criteria for EN-DC is 36+9*n when the UE is configured with E-UTRA SCell(s), and n is the number of E-UTRA SCell carrier frequencies.

Proposal 5. For NE-DC, the total number of E-UTRA reporting criteria is Ecat,NE-DC,E-UTRA=10+9×n, and  is the number of configured E-UTRA serving frequencies, including PSCell and SCells carrier frequencies.

4. References

- R4-1909535	Remaining issues on NR reporting criteria, ZTE

- R4-1911556 Remaining issues on NR reporting criteria, ZTE

- R4-1914769	Remaining issues on NR reporting criteria, ZTE

- R4-1915786	CR to 36.133 on NR reporting criteria, ZTE