3GPP TSG-RAN WG4 Meeting #94-e	R4-2001271

Online, 24 February – 6 March, 2020

Source:	ZTE

Title:	Updated simulation assumption on SIB1 decoding for NR CGI reading

Agenda item:	8.15.1.3

Document for:	Approval

1. Introduction

In this contribution we provide updated simulation assumptions for SIB1 decoding for NR CGI reading.

2. Discussion

The simulation assumptions in [1] need some modifications. The reason for the modification is as follows.

The information payload size and channel bits in the simulation assumption are based on PDSCH RMC in 38.133. However it is calculated based on the number of PDSCH DM-RS CDM groups without data is 1. However, according to 38.214 the number of PDSCH DMRS CDM groups without data should be 2 when scheduling SIB1 with 12 symbols.

When receiving PDSCH scheduled by DCI format 1_0, the UE shall assume the number of DM-RS CDM groups 
without data is 1 which corresponds to CDM group 0 for the case of PDSCH with allocation duration of 2 symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases.

With the change of the number of PDSCH DMRS CDM groups without data from 1 to 2, the information payload size should be 1608 and channel bits per RMSI slot should be 5184 accordingly.

3. Simulation assumptions

The simulation assumptions are provided in Table 1.

Table 1: Parameters for SIB1 reading (RMSI acquisition)

| Parameters | Unit | Value |
| --- | --- | --- |
| Subcarrier spacing | KHz | 30 kHz for 4GHz; 120 kHz for 30GHz |
| Channel bandwidth | MHz | 10 for SCS=30kHz 100 for SCS=120kHz |
| Number of transmitter antennas |  | 1 |
| Number of Rx antennas |  | 2 |
| CP Length |  | Normal |
| Allocated resource blocks for PDSCH |  | 24 (allocated from the lowest edge of the carrier) |
| Number of PDSCH symbols |  | 12 |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| PDSCH mapping type |  | Type A |
| dmrs-TypeA-Position |  | 2 |
| dmrs-Type |  | 1 |
| dmrs-AdditonalPositions |  | 2 |
| Dmrs-maxLength |  | 1 |
| Number of PDSCH DMRS CDM group(s) without data |  | 2 |
| PRB bundling size |  | 2 |
| SIB1 transmission period | ms | 20 |
| Information Bit Payload |  |  |
| For slots with RMSI | Bits | 1608 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI | Bits | 5184 |
| SNR |  | -8, -6, -4, -2, 0 |
| Propagation Condition |  | AWGN, TDL-C with RMS delay spread = 300ns for FR1 TDL-C with RMS delay spread = 60ns for FR2 |
| UE speed |  | 3km/h |
| Detection Method |  | Option 1: Soft combining with 2, 4, 8 repetitions Option 2: One shot |
| Metrics |  | The number of repetitions required to decode SIB1 with 90%-ile success rate. |

4. References

- R4-1915851 Simulation assumption on SIB1 decoding for NR CGI reading, ZTE