3GPP TSG-RAN WG4 Meeting #94-e	R4-2001278

Online, 24 February – 6 March, 2020

Source:	ZTE

Title:	Remaining issues on NR reporting criteria for EN-DC and NE-DC

Agenda item:	12

Document for:	Discussion

1. Introduction

There was a RAN2 LS [1] on measurement reporting criteria for EN-DC. Following question was raised in the LS.

RAN4 agreed a CR in R4-1907862 which may have an impact on the current MN and SN coordination defined by RAN2 for sharing the UE measurement reporting capability for measurements reporting criteria. With this change, RAN2 understands that the component 9×n in Ecat,EN-DC,NR needs to be coordinated between the MN and the SN. Question 1: RAN2 asks RAN4 to confirm whether the changes to UE capabilities for measurements reporting criteria in R4-1907862 imply that the component 9×n in Ecat,EN-DC,NR needs to be coordinated between the MN and the SN.

In this contribution, we further provide our views on the questions in RAN2 LS on measurement reporting criteria for EN-DC operation.

2. Discussion

In the RAN4#92bis meeting, RAN4 reached common understanding on reporting criteria for measurements on NR serving cells configured by E-UTRA PCell in EN-DC, which is copied as follows.

Agreement: The common understanding in RAN4 group is the reporting criterion for measurement on NR serving cells configured by E-UTRA PCell is calculated in the .

The reporting criteria for measurement on NR serving cell carrier frequencies configured by E-UTRA PCell is specified in  Ecat,EN-DC,NR in TS38.133 according to the above agreements. Specifically in

In the RAN4#93 meeting, CR [2] was agreed to capture the above agreements in TS38.133. The new text for reporting criteria for EN-DC in TS38.133 v15.8.0 is as follows.

For UE configured with EN-DC: Ecat,EN-DC,NR+Ecat,EN-DC,E-UTRA, where Ecat,EN-DC,NR=10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies, Ecat,EN-DC,E-UTRA is the total number of E-UTRA reporting criteria configured by E-UTRA PCell except PSCell and SCells carrier frequencies, as specified in TS 36.133 [15] for UE configured with EN-DC.

Therefore the reporting criteria on NR serving cell frequencies are shared between PCell and PSCell. So RAN2’s understanding is correct that that component 9×n in Ecat,EN-DC,NR needs to be coordinated between the MN and the SN in EN-DC operation.

Proposal 1. Reporting criteria for NR serving cell frequencies, i.e. component 9×n in Ecat,EN-DC,NR, needs to be coordinated between the MN and the SN in EN-DC operation.

3. Conclusion

In this contribution, we provide our views on the questions in the RAN2 LS on measurement reporting criteria for EN-DC operation. Based on the observations following proposals are present.

Proposal 1. Reporting criteria for NR serving cell frequencies, i.e. component 9×n in Ecat,EN-DC,NR, needs to be coordinated between the MN and the SN in EN-DC operation.

A companion reply LS [3] is provided.

4. References

- R2-1916595	LS on measurement reporting criteria for EN-DC, RAN2, Nokia

- R4-1914770	CR to 38.133 on NR reporting criteria, ZTE

- R4-2001270	Reply LS on measurement reporting criteria for EN-DC, ZTE