3GPP TSG-RAN WG4 Meeting #94-e				  											R4-2001296

Online, 24th Feb. 2020 – 6th Mar. 2020

Source: 	Nokia, Verizon

Title: 		TP for 37.716-31-11 to introduce DC_2-46-48_n5

Agenda Item: 			9.5.2 [DC_R16_3BLTE_1BNR_4DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-31-11 (v0.9.0) to introduce DC_2-46-48_n5.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.1.x	DC_2-46-48_n5

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-46-48_n5 | CA_2-46-48 | n5 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-46E-48A_n5A | DC_2A_n5A DC_48A_n5A | CA_2A-46E-48A | n5A |
| DC_2A-46D-48A_n5A | DC_2A_n5A DC_48A_n5A | CA_2A-46D-48A | n5A |
| DC_2A-46C-48A_n5A | DC_2A_n5A DC_48A_n5A | CA_2A-46C-48A | n5A |
| DC_2A-46A-48A_n5A | DC_2A_n5A DC_48A_n5A | CA_2A-46A-48A | n5A |

### 5.1.x.3	∆TIB and ∆RIB values

For DC_2-46-48_n5, the TIB,c and RIB,c values are derived from CA_2-5, CA_5-46, CA_5-48, CA_2-46-48 in TS 36.101.

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-48_n5 | 2 | 0.6 |
|  | 46 | 0 |
|  | 48 | 0.8 |
|  | n5 | 0.3 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-46-48_n5 | 2 | 0.2 |
|  | 46 | 0 |
|  | 48 | 0.5 |
|  | n5 | 0 |

*********************** End of the TP ***************************************************

# 3	References

RP-192446 - Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)