3GPP TSG-RAN WG4 Meeting #94-e				  											R4-2001301

Online, 24th Feb. 2020 – 6th Mar. 2020

Source: 	Nokia, T-Mobile

Title: 		TP for 37.716-21-11 to introduce DC_46-66_n25

Agenda Item: 			9.4.2 [DC_R16_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-21-11 to introduce DC_46-66_n25.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.1.x	DC_46-66_n25

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-66_n25 | CA_46-66 | n25 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |

### 5.1.x.3	Coexistence studies

Co-existence analysis for DC_66_n25 in TR 37.716-11-11 shows that 3rd order harmonics and 3rd and 5th IMD might fall in Band 46.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_46-66_n25, the TIB,c and RIB,c values are derived from CA_46-66 in TS 36.101 and DC_66A_n25A in TR 37.716-11-11.

Table 5.1.x.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n25 | 46 | 0 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

Table 5.1.x.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n25 | 46 | 0 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |

### 5.1.x.5	REFSENS requirements

MSD is derived from DC_66A_n25A captured in TR 37.716-11-11.

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_46A-66A_n25A3 DC_46C-66A_n25A3 DC_46D-66A_n25A3 | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

*********************** End of the TP ***************************************************

# 3	References