3GPP TSG-RAN WG4 Meeting#94-e                	R4-2001336

e-Meeting, February 24th – March 6th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Conditional handover for LTE

Agenda item:	7.12.1.1

Document for:	Approval

Introduction

In RAN4 meeting in Reno RAN4 agreed on the CHO requirements for LTE Rel-16 in [1]. One thing was left open regarding preparation time, which would need to be solved. Additionally, there is a need to correct a reference error. This paper addresses both aspects.

Discussion

Preparation time

In the agreed CR RAN4 left the preparation time as [FFS]:

Preparation time is the time from when after measurement time and the time during which the UE prepares the target cell for conditional handover.

TCHO_execution is the time needed for preparing the conditional handover to the target cell.

TCHO_execution can be up [FFS].

The need for this time was discussed and was supposed to cover some additional time needed on the UE side to prepare the triggering target cell one cell change is to be executed.

As the current delay requirements are already quite relaxed leaving UE enough time to measure, perform the cell change and access the new cell, we believe any preparation would be well covered within the current delay requirements within any additional delays.

Additionally, the current CHO delay includes a significant amount of time uncertainty related to the different part of the procedure (especially the measurement time, we do not see how any additional preparation time delay would impact the overall delay in a meaningful way.

Hence, we propose to remove the TCHO_execution from the CHO delay.

Remove TCHO_execution from Dhandover for conditional handover.

Alternatively, if removing the delay is not agreeable to all parties, we propose that the delay is very short.

Reference problem

The current base requirement for conditional handover is captured in 5.1.2.6 which is covering FDD-FDD conditional handover.

In the related CHO requirements for FDD-TDD and TDD-TDD are captured in sections 5.1.2.7 and 5.1.2.9. However, these sections just refer to section 5.1.2.6 which is for FDD-FDD CHO, using references to FDD requiements for the target cell.

For the case when the target cell is a TDD cell the references should refer to TDD requirements (not FDD as now). This needs to be corrected.

Additionally, we wold like to correct one reference error in section 5.1.2.6.2

Agree to one of the text proposals in section 2.3.

Next are two alternative text proposals how to capture the correction.

Text Proposals

Text proposal 1

#### 5.1.2.6	E-UTRAN FDD – FDD conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers.

##### 5.1.2.6.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2] plus the measurements time stated in clause 5.1.2.6.2 plus the conditional execution preparation time in clause 5.1.2.6.3 plus the interruption time stated in clause 5.1.2.6.4.

5.1.2.6.2	Measurement time

The measurement time delay is defined as the time between the RRC procedure delay and the point when the UE executes a handover to a target cell and interruption time starts.

The measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than T identify intra defined in clause 8.1.2.2.1.1(FDD) and 8.1.2.2.2 (TDD). When TTT or L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.1.2.2.1.1(FDD) and 8.1.2.2.2 (TDD) becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TMeasurement_Period_Intra provided the timing to that cell has not changed more than ± 50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

5.1.2.6.3	Preparation time

Preparation time is the time from when after measurement time and the time during which the UE prepares the target cell for conditional handover.

TCHO_execution is the time needed for preparing the conditional handover to the target cell.

TCHO_execution can be up [FFS].

5.1.2.6.4	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = TIU + 20 ms

Where:

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.1(FDD) and 8.1.2.2.2 (TDD) for intra-frequency handover and Clause 8.1.2.3.1(FDD) and 8.1.2.3.2 (TDD) for inter-frequency handover.

#### 5.1.2.7	E-UTRAN FDD – TDD conditional handover

The requirements in this clause are applicable to conditional handover from FDD to TDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.1.2.61 apply for this section.

#### 5.1.2.8	E-UTRAN TDD – FDD conditional handover

The requirements in this clause are applicable to conditional handover from TDD to FDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.1.2.6 apply for this section.

#### 5.1.2.9	E-UTRAN TDD – TDD conditional handover

The requirements in this clause are applicable to handover from TDD to TDD. The requirements in this clause shall apply to UE supporting TDD.

The requirements in clause 5.1.2.6 apply for this section.

Text proposal 2

#### 5.1.2.7	E-UTRAN FDD – TDD conditional handover

The requirements in this clause are applicable to conditional handover from FDD to TDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers.

##### 5.1.2.6.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2] plus the measurements time stated in clause 5.1.2.6.2 plus the conditional execution preparation time in clause 5.1.2.6.3 plus the interruption time stated in clause 5.1.2.6.4.

5.1.2.6.2	Measurement time

The measurement time delay is defined as the time between the RRC procedure delay and the point when the UE executes a handover to a target cell and interruption time starts.

The measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than T identify intra defined in clause 8.1.2.2.2. When TTT or L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.1.2.2.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TMeasurement_Period_Intra provided the timing to that cell has not changed more than ± 50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

5.1.2.6.3	Preparation time

Preparation time is the time from when after measurement time and the time during which the UE prepares the target cell for conditional handover.

TCHO_execution is the time needed for preparing the conditional handover to the target cell.

TCHO_execution can be up [FFS].

5.1.2.6.4	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = TIU + 20 ms

Where:

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.2 for intra-frequency handover and Clause 8.1.2.3.2 for inter-frequency handover.

#### 5.1.2.8	E-UTRAN TDD – FDD conditional handover

The requirements in this clause are applicable to conditional handover from TDD to FDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.1.2.6 apply for this section.

#### 5.1.2.9	E-UTRAN TDD – TDD conditional handover

The requirements in this clause are applicable to handover from TDD to TDD. The requirements in this clause shall apply to UE supporting TDD.

The requirements in clause 5.1.2.7 apply for this section.

Conclusion

In RAN4 meeting in Reno RAN4 agreed on the CHO requirements for LTE Rel-16 in [1]. One thing was left open regarding preparation time, which would need to be solved. Additionally, there is a need to correct a reference error. This paper addressed both aspects

- Remove TCHO_execution from Dhandover for conditional handover.

- Agree to one of the text proposals in section 2.3.

References

- R4-1915943, CR Introduction of handover delay requirements for conditional handover (section 5.1), Nokia, Nokia Shanghai Bell