3GPP TSG-RAN WG4 Meeting #94-e	R4-2001351

Electronic meeting, 24 February – 6 March 2020

Agenda Item:	7.10.4

Source:	Ericsson

Title:	Open issues on UE/BS demodulation requirements for Rel-16 eMTC

Document for:	Discussion

# 1	Introduction

RAN4#93 agreed with the way forward on UE/BS demodulation performance for additional MTC enhancements, where several FFS(s) are listed [1]. This contribution discusses the open issues on UE/BS demodulation requirements.

BS demodulation requirements - FFS to introduce new BS demodulation requirements for multi-TB scheduling for PUSCH UE demodulation requirements - FFS to introduce new UE demodulation requirements for - Multi-TB scheduling for PDSCH - MPDCCH/PDSCH demodulation requirements due to the use of LTE control region

# 2	Discussion

## 2.1	Multi-TB scheduling for PDSCH/PUSCH

One of the open issues is whether to introduce PDSCU/PUSCH demodulation requirements with the multi-TB scheduling. According to the RAN1 agreement, PDSCH/PUSCH in Rel-16 supports up to 8 simultaneous transport block transmissions for CE Mode A and 4 simultaneous transport block transmissions for CE Mode B.

Figure 1 and Figure 2 illustrates the examples of multi-TB scheduling for DL and UL, respectively. In these examples, 4 simultaneous transport block transmissions are assumed. Although one DCI in MPDCCH can schedule multiple TBs, each TB is transmitted with the same resource allocation, MCS and repetition level. Moreover the same redundancy version sequence is used for each TB.

When the network configures the multi-TB scheduling with repetition, there are two configurations as shown in Figure 1 and Figure 2; one is the interleaved transmission where the transmitted transport block changes during the middle of repetition; another one is the non-interleaved transmission where the transmission is based on transport block.

Figure 1	Example of the downlink multi-TB transmission.

Figure 2	Example of the uplink multi-TB transmission.

From PDSCH/PUSCH demodulation point of view, we don’t see any difference from the existing single-TB scheduling. We therefore propose not to specify new PDSCH/PUSCH demodulation requirements with multi-TB scheduling.

Proposal 1: RAN4 does not define new PDSCH demodulation requirements with multi-TB scheduling.

Proposal 2: RAN4 does not define new PUSCH demodulation requirements with multi-TB scheduling.

## 2.2	MPDCCH/PDSCH demodulation requirements due to the use of LTE control region

According to RAN1 agreement, the LTE control region is also used to transmit MPDCCH/PDSCH, it is 1 to 4 additional OFDM symbols (= lPDCCH) depending on the system bandwidth if the network configures the higher layer parameter ce-dl-lte-control-region-config. For MPDCCH, the first lPDCCH OFDM symbols in the second slot is copied to the LTE control region of the first slot. For PDSCH, on the other hand, the transport block is rate-matched to the available PDSCH symbols. Effectively this feature reduce the coding rate and it is expected the receiver performance improvement.

We studied the potential performance gain by using the LTE control region with different MCS (QPSK 1/3 with 6RPB, 16QAM 1/2 with 3PRB, and 64QAM with 1PRB), as shown in Figure 1. Table 1 summarizes the SNRs to achieve 70% of the maximum throughput and shows the performance gain with this feature. It is observed the performance improvement especially for the higher modulation scenario such as 64QAM. However, we don’t have strong motivation to define new PDSCH demodulation requirements with this feature.

Proposal 3: RAN4 does not define new MPDCCH/PDSCH demodulation requirements with the use of LTE control region.

Figure 3	PDSCH throughput with/without using LTE control region (Full duplex FDD).

Table 1	Performance gain with regard to SNR at 70% of the maximum throughput.

| MCS | Rel-13 (dB) | Use control region (dB) | Gain (dB) |
| --- | --- | --- | --- |
| QPSK 1/3 | -0.2 | -0.7 | 0.5 |
| 16QAM 1/2 | 7.0 | 6.2 | 0.8 |
| 64QAM 1/2 | 12.5 | 11.2 | 1.3 |

# 3	Summary

Proposal 1: RAN4 does not define new PDSCH demodulation requirements with multi-TB scheduling.

Proposal 2: RAN4 does not define new PUSCH demodulation requirements with multi-TB scheduling.

Proposal 3: RAN4 does not define new MPDCCH/PDSCH demodulation requirements with the use of LTE control region.

# 4	References

- R4-1915876, “Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE”.